3GPP RAN WG3 Meeting #130	R3-258449

Dallas, USA, 17th – 21st November 2025

Source:	CATT

Title:	Consideration on AI/ML use cases

Agenda Item:	10.5.1

Document for:	Discussion and decision

- Introduction

This document shows our understanding on AI/ML use cases, on AI for network.

- Discussion

  - High-level Principles

The 5G-era high-level principles were captured in TR 37.817 as following:

- The detailed AI/ML algorithms and models for use cases are implementation specific and out of RAN3 scope. - The study focuses on AI/ML functionality and corresponding types of inputs/outputs. - The input/output and the location of the Model Training and Model Inference function should be studied case by case. - The study focuses on the analysis of data needed at the Model Training function from Data Collection, while the aspects of how the Model Training function uses inputs to train a model are out of RAN3 scope. - The study focuses on the analysis of data needed at the Model Inference function from Data Collection, while the aspects of how the Model Inference function uses inputs to derive outputs are out of RAN3 scope. - Where AI/ML functionality resides within the current RAN architecture, depends on deployment and on the specific use cases. - The Model Training and Model Inference functions should be able to request, if needed, specific information to be used to train or execute the AI/ML algorithm and to avoid reception of unnecessary information. The nature of such information depends on the use case and on the AI/ML algorithm. - The Model Inference function should signal the outputs of the model only to nodes that have explicitly requested them (e.g., via subscription), or nodes that take actions based on the output from Model Inference. - An AI/ML model used in a Model Inference function has to be initially trained, validated and tested by the Model Training function before deployment. - NG-RAN SA is prioritized; EN-DC and MR-DC are down-prioritized, but not precluded from Rel.18. - Functional framework and high-level procedures defined in this TR should not prevent from “thinking beyond” them during normative phase if a use case requires so. - User data privacy and anonymisation should be respected during AI/ML operation.

At the last meeting, only the first bullet was captured, while the last bullet was considered within SA3’s scope. For the rest, we believe that the situation does not change much from 5G to 6G, and all of these bullets except the “NG-RAN SA is prioritized…” one are also applicable for 6G.

Proposal 1: Consider all of the high-level principles captured in TR 37.817 valid for 6G, except the bullet “NG-RAN SA is prioritized; EN-DC and MR-DC are down-prioritized, but not precluded from Rel.18” and the bullet on User data privacy.

  - Scope of study item

Besides use cases which are already captured in 5G specifications, some additional use cases were captured as potential ones for 6G:

- AI/ML assisted QoE - Cross-domain collaboration for AI/ML - Agentic AI - AI computing resource management - AI/ML-enabled RAN Digital Twin for leveraging environmental data - AI/ML-assisted resilience - Network for AI

However, we found two issues over these use cases from the perspective of SI scope.

One issue is about the use cases relying on the features which are unlikely to specify in the first release of 6G specifications (i.e. Rel-21). One example is AI/ML-assisted QoE optimisation—QoE itself will unlikely be defined in the first 6G release, and if QoE is not defined, AI-assisted QoE optimisation is not possible to define.

Nevertheless, a Rel-20 SI may serve not only the Rel-21 WI but also WIs in later releases. So it may still be reasonable to discuss these use cases now. We thus hope some clarification.

Proposal 2: Discuss whether the use cases relying on features unlikely to support in Rel-21 are within the Rel-20 AI SI scope, and/or whether they should be assigned with lower priority.

The other issue is about inter-WG coordination. The use/benefit/requirement of some use cases seem to be beyond the radio network, and thus may not be suitable to evaluate in RAN WG3. Some input from other WGs on their use/benefit/requirement may be useful for discussion in RAN WG3. We may need to determine how to collaborate with other WGs for this type of use cases.

Proposal 3: Discuss the form of inter-WG collaboration for the use cases of which the use/benefit/requirement is beyond the radio network.

  - Content of each use case

We believe that assuming the inference output is suitable to be the first step in discussing each use case: the inference output is usually much more convergent—in the WI phase—compared with the various inference inputs and performance-monitoring parameters, and it is literally clearer than the usually vague and lengthy description of use case itself. In addition, assuming the inference output facilitates the discussion of listing potential inference inputs, as the mainly use of inference inputs is to improve the precision/accuracy of inference output.

For the use case of mobility, we believe the inference output should be assumed as the (future) UE trajectory prediction (granularity FFS) or potentially measurement results. Prediction of (future) data volume/rate is also useful in our view, but we think it may not be suitable to include it into the namingly “mobility” use case, although we did so in Rel-17 discussion. We prefer discussing data volume/rate prediction in a separate use case.

Proposal 4: The inference output of mobility use case at least includes (future) UE trajectory

Proposal 5: Prediction of per-UE (e.g. one per UE/session/flow) data volume/rate is not considered as a part of mobility use case—it should be considered in a separate use case.

For the use case of NES, we believe the inference output should be assumed as the prediction of change of overall Energy Cost (EC) or potentially node-level performance result(s), each upon change of ES state. NES action (e.g. recommended cell activation/deactivation) itself may not be a suitable candidate of inference output, as it relying on reinforcement training which seems to be very unpopular.

In addition, the use case of NES and CCO may be generalised into one single use case, as their inference output can both be the node-level performance result(s) upon change of coverage configuration—that is to say, some operator may aim at optimising a general “node-and-neighbour performance” value which takes many performance indices into account, e.g. energy cost, number of RLFs, average cross-link inference level, rather than optimising any separate performance index. Other operator may still aim at optimising separate performance index, which can be achieved as defining the general “node-and-neighbour performance” value identical to the separate performance index.

Proposal 6: The inference output of NES use case is assumed to be the prediction of change of overall EC or node-level performance result(s), each upon change of ES state.

Proposal 7: Considering generalising NES and CCO into one use case, which inference output is assumed to be the prediction of change of a general “node-and-neighbour performance” value upon change of node-level configuration, e.g. coverage configuration, synchronising signal configuration.

  - AS performance metrics, especially per-UE ones

In 5G, UE performance feedback plays a critical role in numerous AI/ML use cases within RAN3, and a clear definition is essential to support inter-vendor deployment. Most UE performance metrics are specified in either SA5 or RAN2 standards, or in both. In TS 28.552, regarding its relationship with Layer 2 measurements defined in TS 38.314, the following statements are provided.

When it comes to Layer 2 measurement definitions, some of the L2 measurement definitions used in the present document are referring to TS 38.314 [29]. The L2 measurement definitions in TS 38.314 [29] and in the present document have some differences:

-	The measurement definitions in TS 38.314 [29] are often defined to be reported 'per UE or per DRB', to support MDT and Trace use cases.

-	The measurements defined in the present document define L2 measurements that is aggregated and often reported per a Managed Object class (e.g. NRCellDU).

Thus, for those L2 measurements, the definition in TS 38.314 [29] is re-used in the present document, but without requirement of 'per UE or per DRB' reporting to be performed.

Based on above statement, for L2 measurements defined in both RAN2 and SA5 specifications, the underlying rationale follows a clear progression. RAN2 first defines L2 measurements at the “per UE per DRB” level. SA5 then adopts this definition in TS 28.552 to monitor aggregated measurements at the per-node level.

Subsequently, SA5 further introduced “per UE” level measurements in TS 28.558, with the intent to support AI/ML for NG-RAN and NWDAF analytics, as detailed below.

The UE level measurements, i.e., the measurements per UE, in addition to the traditional performance measurements which are aggregated for the UEs to a measured object (e.g., a cell), are required to support AI/ML for NG-RAN, and NWDAF analytics.

The UE level measurement is produced by 5GC Network Function or NG-RAN node for a particular UE, and provided by the MnS producer to the authorized consumer(s).

Observation 1: Many of the AS performance measurement metrics in RAN2 and SA5 originate from the granular “per DRB per UE” level in TS 38.314. This definition is then used in TS 28.552 for node-level aggregation. Finally, TS 28.558 defines the “per UE” level, following the model set by TS 28.552.

RAN3 was not involved in the discussions on defining Layer 2 measurements. However, it is crucial for RAN3 to have a clear definition of these measurements, as well as a standardized method for deriving measurement results to support AI/ML in NG-RAN. Although several specifications have addressed the definition of L2 measurements, certain ambiguities remain, making it difficult to implement AI/ML functions in multi-vendor deployments. For example, in the Release 19 AI/ML Work Item, it is still unclear how to derive UE-level packet delay from DRB-level packet delay.

We may need to do something to prevent this from happening again.

The use of AS performance metrics includes at least three aspects:

- Performance monitoring at the OAM (seemingly SA5);

- (Non-AI) SON purpose in RAN (RAN2);

- AI model training / performance monitoring at the OAM or RAN (RAN3).

Proposal 8: Discuss the principle of inter-WG collaboration in defining AS performance metrics, especially per-UE ones.

In 5G, the work on Layer 2 measurements and the corresponding specification (TS 38.314 MDT) in RAN2 was initiated by the SON/MDT Work Item in Release 16, not as part of the 5G Day 1 features. This timeline posed no issue for 5G, as AI/ML work in RAN3 began in Release 17, by which time the necessary measurements were already defined—even if some ambiguities remained. However, the situation is different as AI will be supported in day 1, whereas it is not clear if SON/MDT will. So for the first 6G release, the triggering work of defining AS performance metrics seems fall on RAN3 and SA5. For most metrics, RAN3 knows no less background information than SA5, so we think that it should be permitted for RAN3 to trigger the discussion of defining AS performance metrics and to identify which WG(s) should be involved for each metric. SA WGs, such as WG5, surely has their own right to trigger defining metrics for their purpose, but should not define AS metrics all on their own, i.e. consulting with at least one RAN WG  for each AS performance metric.

Proposal 9: It should be permitted for RAN3 to trigger the discussion of defining AS performance metrics for AI/ML and to identify which WG(s) should be involved for each metric in 6G day1.

For detail work split if the requirement is from RAN3 but other WG (e.g. RAN2) needs to be involved, we prefer RAN3 to decide the granularity, including both the aspect of what an integer “one” means and the aspect of whether it should be one value per UE, per session or per flow. That is to say, in the LS out, RAN3 should state clearly the granularity of a metric which RAN3 requests other WG to define and specify.

Proposal 10: If an AS performance metric should be defined for RAN3 AI purpose but the definition should be provided by other WG, its required granularity should be stated clearly in the LS out.

The next question is what specification to capture the definitions. We think capturing all metrics in SA5 specifications is the most suitable, as all metrics introduced for RAN3 purpose require support of delivering measurement results to the OAM (for training / monitoring in the OAM), but all metrics introduced for SA5 purpose does not require anything in RAN3 scope. Capturing all metrics in specifications maintained by one WG is clearer than capturing some metrics in specifications maintained by one WG but capturing other metrics in specifications maintained by another WG, so capturing every metric in SA5 specifications is the cleanest solution.

Proposal 11: All AS performance metrics should be captured in SA5 specification(s).

- Conclusion

Proposal 1: Consider all of the high-level principles captured in TR 37.817 valid for 6G, except the bullet “NG-RAN SA is prioritized; EN-DC and MR-DC are down-prioritized, but not precluded from Rel.18” and the bullet on User data privacy.

Proposal 2: Discuss whether the use cases relying on features unlikely to support in Rel-21 are within the Rel-20 AI SI scope, and/or whether they should be assigned with lower priority.

Proposal 3: Discuss the form of inter-WG collaboration for the use cases of which the use/benefit/requirement is beyond the radio network.

Proposal 4: The inference output of mobility use case at least includes (future) UE trajectory.

Proposal 5: Prediction of per-UE (e.g. one per UE/session/flow) data volume/rate is not considered as a part of mobility use case—it should be considered in a separate use case.

Proposal 6: The inference output of NES use case is assumed to be the prediction of change of overall EC or node-level performance result(s), each upon change of ES state.

Proposal 7: Considering generalising NES and CCO into one use case, which inference output is assumed to be the prediction of change of a general “node-and-neighbour performance” value upon change of node-level configuration, e.g. coverage configuration, synchronising signal configuration.

Observation 1: Many of the AS performance measurement metrics in RAN2 and SA5 originate from the granular “per DRB per UE” level in TS 38.314. This definition is then used in TS 28.552 for node-level aggregation. Finally, TS 28.558 defines the “per UE” level, following the model set by TS 28.552.

Proposal 8: Discuss the principle of inter-WG collaboration in defining AS performance metrics, especially per-UE ones.

Proposal 9: It should be permitted for RAN3 to trigger the discussion of defining AS performance metrics and to identify which WG(s) should be involved for each metric.

Proposal 10: If an AS performance metric should be defined for RAN3 AI purpose but the definition should be provided by other WG, its required granularity should be stated clearly in the LS out.

Proposal 11: All AS performance metrics should be captured in SA5 specification(s).

- pCR for TR 38.760-3

7	AI/ML for RAN

7.1	High-level Principles

-	The detailed AI/ML algorithms and models for use cases are implementation specific and out of RAN3 scope.

-	The study focuses on AI/ML functionality and corresponding types of inputs/outputs.

-	The input/output and the location of the Model Training and Model Inference function should be studied case by case.

-	The study focuses on the analysis of data needed at the Model Training function from Data Collection, while the aspects of how the Model Training function uses inputs to train a model are out of RAN3 scope.

-	The study focuses on the analysis of data needed at the Model Inference function from Data Collection, while the aspects of how the Model Inference function uses inputs to derive outputs are out of RAN3 scope.

-	Where AI/ML functionality resides within the current RAN architecture, depends on deployment and on the specific use cases.

-	The Model Training and Model Inference functions should be able to request, if needed, specific information to be used to train or execute the AI/ML algorithm and to avoid reception of unnecessary information. The nature of such information depends on the use case and on the AI/ML algorithm.

-	The Model Inference function should signal the outputs of the model only to nodes that have explicitly requested them (e.g., via subscription), or nodes that take actions based on the output from Model Inference.

-	An AI/ML model used in a Model Inference function has to be initially trained, validated and tested by the Model Training function before deployment.

-	Functional framework and high-level procedures defined in this TR should not prevent from “thinking beyond” them during normative phase if a use case requires so.

-	User data privacy and anonymisation should be respected during AI/ML operation.

7.2	AI/ML use cases

Editor’s note: Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc…

7.3	AI/ML framework