3GPP TSG-RAN WG3 Meeting #130	R3-258465

# Dallas, USA, 17-21 November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR |  | rev |  | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE Context Retention for NG – Stage 2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Huawei, Jio Platforms, Thales |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2025-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | When the TNL is re-established after a TNL failure, releasing UE contexts which remain consistent after TNL re-establishment is not necessary. This functionality has been imported from E-UTRAN; it is captured in NGAP since Rel-15 but it is currently not mentioned in NG-RAN stage 2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | If NG-RAN node and AMF had agreed at NG Setup to keep UE-related contexts and signaling connections, respective handling when the signaling connection is interrupted and when it is operational again is as specified in NGAP. The NG-RAN node and AMF can agree during NG Setup on not performing the NG Reset procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing stage 2 description for a functionality captured in NGAP since Rel-15. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.2, 15.3.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

#### 4.3.1.2	NG Control Plane

The NG control plane interface (NG-C) is defined between the NG-RAN node and the AMF. The control plane protocol stack of the NG interface is shown on Figure 4.3.1.2-1. The transport network layer is built on IP transport. For the reliable transport of signalling messages, SCTP is added on top of IP. The application layer signalling protocol is referred to as NGAP (NG Application Protocol). The SCTP layer provides guaranteed delivery of application layer messages. In the transport, IP layer point-to-point transmission is used to deliver the signalling PDUs.

Figure 4.3.1.2-1: NG-C Protocol Stack

NG-C provides the following functions:

-	NG interface management;

-	UE context management;

-	UE mobility management;

-	Transport of NAS messages;

-	Paging;

-	PDU Session Management;

-	Configuration Transfer;

-	Warning Message Transmission.

Further details of NG-C can be found in TS 38.410 [16].

If the NG signaling transport layer notifies the NGAP layer that the signaling connection broke and the AMF and the NG-RAN node had agreed at NG Setup to keep UE-related contexts and related signaling connections, handling of UE-related contexts and signaling connections is according to TS 38.413 [26].

If the NG signaling transport layer notifies the NGAP layer that the signaling connection is operational again and the NG-RAN node and AMF had agreed at NG Setup to keep UE-related contexts and related signaling connections, handling of UE-related contexts and signaling connections is according to TS 38.413 [26].

NEXT CHANGE

#### 15.3.1.3	Application layer initialization

Once SCTP connectivity has been established, the NG-RAN node and the AMF shall exchange application level configuration data over NGAP with the NG Setup procedure, which is needed for these two nodes to interwork correctly on the NG interface:

-	The NG-RAN node provides the relevant configuration information to the AMF, which includes list of supported TA(s), etc.;

-	The AMF provides the relevant configuration information to the NG-RAN node, which includes PLMN ID, etc.;

-	When the application layer initialization is successfully concluded, the dynamic configuration procedure is completed and the NG-C interface is operational.

After the application layer initialization is successfully completed, the AMF may add or update or remove SCTP endpoints to be used for NG-C signalling between the AMF and the NG-RAN node pair as specified in TS 23.501 [3].

The AMF and the gNB may agree at the NG Setup procedure that UE-related contexts and related signalling connection that have been existing before the NG Setup shall not be affected. The AMF or gNB or both may trigger an NGAP Reset procedure for any UE-related context and related signalling connection for UEs which could not be kept in RRC_CONNECTED mode, or for UEs for which the AMF or the gNB decided to remove the UE context and related signalling connection. If either the AMF or the gNB do not agree to keep the UE contexts (if any), then they are removed and all related signalling connections are erased.

END OF CHANGES