3GPP TSG-RAN WG3 Meeting #130	R3-258466

Dallas, US, 17-21 Nov, 2025

Title: 	Discussion on removal of support of PWS over satellite NG-RAN in Rel-17 and Rel-18

Source: 	Huawei

Agenda item:	8.1

Document Type:	Discussion

# 1. Introduction

With regards to the LS on removal of support of PWS over satellite NG-RAN in Rel-17 and 18 in [1], after double checking, the agreed CRs in these LSs, we found that the support on early release is a partial support which must be clarified somewhere … We also notice some discrepancy among different groups, hence send a reply LS to other groups to address the issue might be polite.

# 2. Discussion

## 2.1. History of PWS discussion

Let us recap PWS in RAN3 according to the timeline.

In RAN3-128 meeting, the LS from CT#153 meeting was received on the support for PWS over satellite NGRAN in Rel-17, and copied to RAN3 as follows:

1. Overall Description: SA1 has agreed to introduce a new requirement for the support for PWS over satellite NG-RAN (CR#0081 to TS 22.268 in S1-244777) from Rel-17. CT1 has discussed the stage 2 aspects of this requirement and determined that this requirement would have UE impacts that would create backward compatibility issues (e.g. country-specific duplicate detection timers for PLMNs using non-country specific PLMN ID with MCC 9xx over satellite access). Consequently, CT1 has decided to introduce the support for PWS over satellite NG-RAN in Rel-19. This work will be done under the new CT-wide Rel-19 work item on the support for PWS over satellite E-UTRAN and satellite NG-RAN. CT1 kindly asks SA1 to align their specifications accordingly. 2. Actions: To SA1 group. ACTION: CT1 kindly asks SA1 to align the stage 1 specifications to reflect the support for PWS over satellite NG-RAN from Rel-19.

Based on above, RAN3 discussed the PWS support for NR NTN, and the IoT NTN, then all the stage 2 CRs were endorsed to remove the PWS for both NR NTN and IoT NTN for R17 and R18.

| PWS for NTN |  |  |
| --- | --- | --- |
| R3-253449 | Remove the PWS Support for NR NTN (Ericsson, Huawei, Jio Platforms Limited, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Thales) | DraftCR CATT: The first change is not needed - Only keep the second change - Update the coversheet: Clauses affected: - Update the WI code Rev in R3-253817 Endorsed unseen |
| R3-253538 | Remove the PWS support for IoT NTN (Qualcomm Incorporated, Huawei, Ericsson, Jio Platforms, Nokia, Nokia Shanghai Bell, Thales) | draftCR - Only keep the second change - Update the coversheet: Clauses affected: - Remove “Security Indication” Rev in R3-253819 Endorsed unseen |
| R3-253708 | Discussion on the support for PWS over satellite NGRAN in Rel-17 and beyond (Huawei, Ericsson, Jio Platforms, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Thales, Deutsche Telekom) | discussion |
| R3-253709 | Remove the PWS support for NR NTN (Huawei, Ericsson, Jio Platforms , Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Thales, Deutsche Telekom) | draftCR - Only keep the second change - Update the coversheet: Clauses affected: - Update the WI code Rev in R3-253820 Endorsed unseen |
| R3-253717 | Remove the PWS support for IoT NTN (Nokia, Nokia Shanghai Bell, Huawei, Ericsson, Jio Platforms, Qualcomm Incorporated, Thales, Deutsche Telekom) | draftCR - Only keep the second change - Update the coversheet: Clauses affected: Rev in R3-253818 Endorsed unseen |
| R3-253785 | Reply LS to LS on stage 1 requirements for the support for PWS over satellite NGRAN in Rel-17 (SA1(Ericsson)) | LS in # PWSNTNLSout - Provide the reply LS to SA1 and cc other WGs (SA2, CT1, CT4, RAN2, RAN) in R3-253821 - Summary the decision of RAN3 to align with SA1 on PWS for NTN, and inform RAN (moderator - E///) LS in R3-253821 - cc SA Rev in R3-253867 Agreed unseen |

At the same meeting, SA1 sent the LS on the stage1 requirement for the support for PWS over satellite NGRAN in Rel-17 in [4]. As a reply LS, the RAN3 reply LS was agreed in [5].

In the RAN-108 meeting, these RAN3 endorsed CRs were noted. And the LS was sent to other CT groups, with the following contents in [6]. It can be observed that question on PWS support for satellite NG-RAN was raised, though both NR NTN and IoT NTN work item codes are indicated.

Title: Reply LS from RAN on removal of support of PWS over satellite NG-RAN in Rel-17 and 18 Response to: Reply LS (R3-253867 = RP-250850) on stage 1 requirements to support PWS over satellite NG-RAN in Rel-17 from RAN3 Release: Rel-17/18 Work Item: NR_NTN_solutions-core, LTE_NBIOT_eMTC_NTN-Core <skip the irreverence> 1. Overall Description: RAN thanks RAN3 for raising awareness on the Reply to the SA1 LS on Stage 1 requirements for PWS support over satellite NG-RAN. RAN discussed the issue from a RAN perspective, also considering the original conclusion by CT1 captured in the CT1 LS to SA1 (C1-250715) and highlighted by RP-251718, which identified possible “backward compatibility issues (e.g. country-specific duplicate detection timers for PLMNs using non-country specific PLMN ID with MCC 9xx over satellite access)”, and concluded that the issue identified by CT1 might not have impact on the NG-RAN, and thus the removal of PWS support for satellite NG-RAN in Rel-17 and Rel-18 need further assessment. Several companies highlighted that the removal of PWS support for satellite NG-RAN in Rel-17 and Rel-18 may have been driven by a misinterpretation of the relationship between non-country specific (e.g. 9xx) PLMNs and satellite access, highlighting that satellite access may be deployed without the use of non-country specific (e.g. 9xx) PLMNs, and non-country-specific PLMNs may be deployed without satellite access. 2. Actions: To CT1, CT, SA1, SA: ACTION: RAN asks CT1, CT, SA1, SA to take the above into account and either revert or confirm the removal of the support for PWS over satellite NG-RAN in Rel-17 and Rel-18. To RAN3, RAN2: ACTION: RAN asks RAN3 and RAN2 to take appropriate action based on the conclusions in the other working groups.

Afterwards, the CT meeting agreed CRs to support the PWS for satellite NG-RAN in Rel-17 and R18 in the LS [2].

1. Overall Description: CT thanks RAN for their LS on removal of support of PWS over satellite NG-RAN in Rel-17 and 18. CT discussed the LS and agreed CRs (attached) to introduce support for PWS over satellite NG-RAN in Rel-17 and 18. 2. Actions: To RAN group. ACTION: CT asks RAN to take the information above into account.

Afterwards, as discussed above, the SA2-171 meeting sent the following LS to align with CT agreement in [1].

# 1 Overall description SA2 thanks CT for their LS in CP-252246/ S2-2508164. SA2 has agreed attached CRs to align with CT and RAN agreements. # 2 Actions To CT, RAN ACTION: - SA2 kindly asks to take above information into account.

## 2.2. Discrepancy among different groups

We believe there is some discrepancy among different groups after reading the details:

- It is said the agreements at SA2 is to align with RAN agreements. However, it should be understood that there are no RAN agreements on the PWS.

- The agreed CRs for TS 23.401 were attached targeting for IoT NTN. However, it can be observed that all the CT agreements are targeting satellite NG-RAN (i.e., NR NTN), but not for IoT NTN.

Hence there are mis-alignments between the SA2 agreements and CT agreements.

Observation 1: for the incoming SA2 LS, there are following issues:

  - It is said the agreements at SA2 is to align with RAN agreements. However, it should be understood that there are no RAN agreements on the PWS while sending LS to CT to make final decision.

  - The agreed CRs for TS 23.401 were attached targeting for IoT NTN. However, it can be observed that all the CT agreements are targeting for satellite NG-RAN (i.e., NR NTN), but not for IoT NTN.

## 2.3. Partial support of PWS in Rel-17 and Rel-18

For NR NTN, clearly according to CT LS [2], the PWS should be supported for Rel-17 and Rel-18. Hence the stage2 texts in TS 38.300 on the PWS are correct, and no changes are needed.

The Cell Identity, as defined in TS 38.413 [26] and TS 38.423 [50], used in following cases corresponds to a Mapped Cell ID, irrespective of the orbit of the NTN payload or the types of service links supported: - The Cell Identity indicated by the gNB to the Core Network as part of the User Location Information; - The Cell Identity used for Paging Optimization in NG interface; - The Cell Identity used for Area of Interest; - The Cell Identity used for PWS.

.

Observation 2: For NR-NTN, the Rel-17 and Rel-18 specifications already support the PWS and no changes are needed.

However for IoT NTN, the RAN question to CT in [2] is enquiring the PWS support for NR NTN though both the NR NTN and IOT NTN WI codes are included. Hence for IoT NTN, there are no clear answers on the PWS support from CT. From RAN perspective, the public warning system for NB-IoT is supported till Rel-19 after a whole release discussion. The detailed changes for Rel-19 TS 36.300 were added in 4.10 and 23.3, where the following three SIBs are introduced.

  - SystemInformationBlockType10-NB contains an ETWS primary notification;

  - SystemInformationBlockType11-NB contains an ETWS secondary notification;

  - SystemInformationBlockType12-NB contains a CMAS warning notification;

Observation 3: For NB-IoT NTN, the PWS is supported till Rel-19, after introducing new three SIBs.

While for other IoT devices (including the BL UEs and UEs in enhanced coverage), the PWS is already supported for IoT NTN, without any new SIBs, but not for other NB-IoT UEs for Rel-17 and Rel-18.

Observation 4: For other IoT devices (including the BL UEs and UEs in enhanced coverage), the PWS is already supported, without any new SIBs, but not for NB-IoT for Rel-17 and Rel-18.

In section 23.21.6 of TS 36.300, it is specified that the Cell Identity used for PWS corresponds to a mapped cell ID, which hints that all IoT devices support the PWS. We think it should be clarified that the PWS is supported for BL UEs, UEs in enhanced coverage, but not for NB-IoT for Rel-17 and Rel-18.

### 23.21.6 Signalling The Cell Identity, as defined in TS 36.413 [25] and TS 36.423 [42], corresponds to a Mapped Cell ID, irrespective of the orbit of the NTN payload or the types of service links supported in the following cases: - The Cell Identity indicated by the eNB to the Core Network as part of the User Location Information, or as E-UTRAN CGI in the related S1AP messages; - The Cell Identity used for Paging Optimization in S1 interface; - The Cell Identity used for PWS.

- RAN3 to clarify that the Cell Identity used for PWS is provided only for BL UEs and UEs in enhanced coverage, but not for NB-IoT for Rel-17 and Rel-18.

If RAN3 made agreements, especially for IoT NTN, RAN3 should send reply LS to other groups. Also, as discussed in the above observation 1, RAN3 should point out the issues in the SA2 LS, so that SA2 can revisit it, and correct their specification if deemed necessary.

- RAN3 to send the reply to other groups with the agreements, and point out to SA2 the issues in above observation 1.

# 3. Conclusion

Based on the discussion in this paper, we make the following proposals.

Observation 1: for the incoming SA2 LS, there are following issues:

  - It is said the agreements at SA2 is to align with RAN agreements. However, it should be understood that there are no RAN agreements on the PWS while sending LS to CT to make final decision.

  - The agreed CRs for TS 23.401 were attached targeting for IoT NTN. However, it can be observed that all the CT agreements are targeting for satellite NG-RAN (i.e., NR NTN), but not for IoT NTN.

Observation 2: For NR-NTN, the R17/R18 specs already support the PWS and no changes are needed.

Observation 3: For NB-IoT NTN, the PWS is supported till Rel-19, after introducing new three SIBs.

Observation 4: For other IoT devices (including the BL UEs and UEs in enhanced coverage), the PWS is already supported, without any new SIBs, but not for NB-IoT for Rel-17 and Rel-18.

- RAN3 to clarify that the Cell Identity used for PWS is provided only for BL UEs and UEs in enhanced coverage, but not for NB-IoT for Rel-17 and Rel-18.

- RAN3 to send the reply to other groups with the agreements, and point out to SA2 the issues in above observation 1.

The reply LS and CRs can be found in [7-9].

# 4. Reference

- R3-258010/S2-2509321 Reply to Reply LS on removal of support of PWS over satellite NG-RAN in Rel-17 and 18, SA2

- CP-252246 LS on allocation of CN assigned subgroup ID for LP-WUS, RAN3

- S2-2509834 	Reply LS on allocation of CN assigned subgroup ID for LP-WUS, SA2

- R3-253785 Reply LS to LS on stage 1 requirements for the support for PWS over satellite NGRAN in Rel-17 SA1

- R3-253867 Reply LS on stage 1 requirements to support PWS over satellite NG-RAN in Rel-17, RAN3

- RP‑251859	Reply LS to R3-253867 = RP-250850 on removal of support of PWS over satellite NG-RAN in Rel-17 and 18 (to: CT1, SA1, CT, SA, RAN3, RAN2; cc: SA2, CT4; contact: Aalyria)	RAN

- R3-258469	[draft] Reply LS on removal of support of PWS for NR NTN and IoT NTN in Rel-17 and 18 , Huawei

- R3-258467 Correction of PWS support for IoT NTN, Huawei

- R3-258468 Correction of PWS support for IoT NTN, Huawei