3GPP TSG-RAN WG3 Meeting #130	R3-258482

Dallas, US, 17th – 21st November, 2025

Title: 	Further considerations on multi-hop UE trajectory in AI/ML-based mobility

Source: 	Huawei

Agenda item:	12.2.1

Document Type:	pCR

# 1. Introduction

A new SI “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN Phase 3” focusing on AI/ML-based mobility was approved for Rel-20 5G-Advanced (5G-A) at RAN #108 meeting [1], then revised in RAN#109 and eventually approved in [2], aiming at investigating mobility scenarios that could benefit from using AI/ML techniques.

This contribution focuses in particular on the “Multi-hop UE trajectory across gNBs” scenario, providing our considerations and proposals for discussion based on the outcome of the last RAN3 meeting.

# 2. Discussion

## 2.1 Background

In the last RAN3 meeting the following agreements were captured in the Chair’s meeting minutes pertaining the multi-hop UE trajectory in AI/ML-based mobility [3]:

Multi-hop predicted UE trajectory across gNBs consists of a list of cells belonging to one or more gNBs where the UE is expected to connect and these cells are listed in chronological order. Deployment of AI/ML model training/inference: The following solutions are considered for supporting multi-hop UE trajectory: - AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB. - AI/ML Model Training and AI/ML Model Inference are both located in the gNB. In case of CU-DU split architecture, the following solutions are possible: - AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU. - AI/ML Model Training and Model Inference are both located in the gNB-CU. Focus on cell-based granularity for measured and predicted multi-hop UE trajectory. Study multi-hop UE trajectory for L3 HO. The delivery of the measured&predicted cell-based multi-hop UE trajectory should not impact the core network. The request and reporting to support measured multi-hop UE trajectory between gNBs should utilize the Data Collection Reporting Initiation procedure and Data Collection Reporting procedure, at least for the first hop.

A TP for the TR 38.745 was agreed in [4], capturing the agreed definition of multi-hop predicted UE trajectory and the deployment scenarios above, while the following FFS aspects need further discussion:

Transfer the multi-hop UE trajectory prediction in UE associated or non-UE associated message? Configuration of measurement feedback: How to transfer measurement feedback: • Option1: Parallel transmission from each hop gNB to the initial source gNB (i.e., Node2 to Node1, Node3 to Node1) • Option2: Hop-by-hop transmission (i.e., Node3 to Node2, Node2 to Node1) • Option3: Final-hop to initial-hop transmission (i.e., Node2 to Node3, Node3 to Node1) • Option4: Measured UE trajectory transfer via AMF (i.e. Node3 to AMF, AMF to Node1).

We will discuss the above open aspects in the following subsections.

## 2.2 Transfer of the multi-hop UE trajectory prediction

The inference-based UE mobility enhancement specified in Rel-18 focused on optimizing UE handovers across cells within the (single-hop) target gNB. For example, the AI/ML model in the source gNB predicts the cell-based UE trajectory (i.e., the list of target cells where the UE will likely be handed over, with optional time of stay in each of these cells) and provides this prediction to the next-hop gNB within the HANDOVER REQUEST message of the Handover Preparation procedure, i.e., a UE-associated procedure.

Observation 1: In Rel-18 a UE-associated procedure, i.e., Handover Preparation, is used by the source gNB which predicted the cell-based UE trajectory to provide such prediction to the target gNB.

In order to enable the multi-hop UE trajectory prediction to span across multiple gNBs, we think that the multi-hop UE trajectory prediction could also be transferred in the HANDOVER REQUEST message from the source gNB to target gNB through multiple intermediate gNBs. The intermediate gNB may not need to transfer the list of cells previously predicted to be visited (or include the whole UE trajectory prediction from the source gNB) to the next-hop gNB, since this information is not relevant for the next-hop intermediate target gNB.

Proposal 1: RAN3 to agree to transfer the overall multi-hop UE trajectory prediction predicted by the source gNB(-CU) to the 1st hop target gNB by enhancing the single-hop UE trajectory prediction transmission mechanism specified in Rel-18 (i.e., in the HANDOVER REQUEST message).

Proposal 2: RAN3 to agree that, for subsequent hops, from the intermediate node to the next hop target gNB there is no need to transfer the overall UE trajectory prediction as received from the source gNB(-CU). Each intermediate gNB removes its own cells actually visited from the list or predicted cells.

## 2.3 Transfer of the measured multi-hop UE trajectory

In the single-hop case specified in Rel-18, the source gNB configures the collection and one-time reporting from the target gNB of the measured UE trajectory in the Data Collection Reporting Initiation procedure, within the DATA COLLECTION REQUEST message, i.e., a non-UE associated procedure. And the collection configuration comprises the collection time duration and number of visited cells.

Observation 2: In Rel-18 a non UE-associated procedure, i.e., Data Collection Reporting Initiation, is used by the source gNB to configure the collection and one-time reporting from the target gNB of the cell-based measured UE trajectory.

At successful handover, the target gNB starts collecting information related to the measured UE trajectory (i.e., list of visited cells and corresponding time UE stayed in each cell) and it shall terminate the collection of the UE trajectory information either at expiration of the collection time duration previously configured by the source gNB, or the UE moves to inactive or idle state, or the UE is handed over to a cell belonging to a different gNB. Then, the target gNB can send the measured UE trajectory back to the source gNB within the DATA COLLECTION UPDATE message of the Data Collection Reporting procedure, i.e., another non UE-associated procedure.

Observation 3: In Rel-18 a non UE-associated procedure, i.e., Data Collection Reporting, is used by the target gNB to report cell-based measured UE trajectory to the source gNB.

Since the multi-hop UE trajectory prediction could be transferred across multiple gNBs, we need to specify how to transfer the measured multi-hop UE trajectory. Assume for instance that the multi-hop UE trajectory prediction spans three gNBs: Node1 (initial source node), Node2 (intermediate node) and Node3 (final node).

Figure 1 - Multi-hop UE trajectory prediction spanning three gNBs.

Collecting the measured UE trajectory across multiple gNBs to eventually be provided to the source gNB is challenging from a complexity point of view due to a number of reasons:

- the need to keep the UE context in the source gNB after the UE has been handed over to subsequent hop target gNBs, so that the source gNB is able to then associate the received UE trajectory as feedback information;

- the potential lack of the Xn interface between the source gNB and “far away” target gNB(s). To be specific, if a trajectory prediction covers the N future cell hops, it is very likely that the gNB serving the nth cell will not be Xn-connected to the source gNB that produced the prediction. Hence, even if the source gNB kept the UE context stored, there would be likely no means for the nth target gNB to provide the actual UE trajectory feedback back;

- cell deployment changes (e.g. cells’ activation and deactivation) during the time interval between the UE trajectory prediction is provided by the source gNB and the measured UE trajectory is received at the source gNB.

Observation 4: There are technical challenges preventing the source gNB to be provided with the measured multi-hop UE trajectory feedback.

Proposal 3: RAN3 to agree to capture the technical challenges preventing the source gNB to be provided with the measured multi-hop UE trajectory feedback in the use case description of TR 38.745.

In the last meeting, four possible option were captured allowing for the collection and reporting of the measured multi-hop UE trajectory [3]:

How to transfer measurement feedback: • Option1: Parallel transmission from each hop gNB to the initial source gNB (i.e., Node2 to Node1, Node3 to Node1) • Option2: Hop-by-hop transmission (i.e., Node3 to Node2, Node2 to Node1) • Option3: Final-hop to initial-hop transmission (i.e., Node2 to Node3, Node3 to Node1) • Option4: Measured UE trajectory transfer via AMF (i.e. Node3 to AMF, AMF to Node1).

In addition, the following agreement from the last RAN3 meeting is also relevant in the view of down-selecting the above options [3]:

The delivery of the measured&predicted cell-based multi-hop UE trajectory should not impact the core network.

Based on the above, Option4: Measured UE trajectory transfer via AMF (i.e. Node3 to AMF, AMF to Node1) should not be considered for further discussions.

Observation 5: Option4: Measured UE trajectory transfer via AMF (i.e. Node3 to AMF, AMF to Node1) should not be considered in future discussions pertaining the multi-hop UE trajectory scenario.

From the offline discussion occurred in the last RAN3 meeting, and also based on contributions from other companies, it seems to us that, for the sake of progressing, the focus should be on the first two options, i.e.,

•	Option1: Parallel transmission from each hop gNB to the initial source gNB (i.e., Node2 to Node1, Node3 to Node1)

•	Option2: Hop-by-hop transmission (i.e., Node3 to Node2, Node2 to Node1)

From our perspective, one of the aspect that should be considered in the final selection between Option 1 and option 2 above is whether, in subsequent hops (i.e., after the 1st hop target gNB), the data collection parameters (i.e., collection time duration as well as number of visited cells) should be transferred via either non UE-associated or UE-associated procedures, i.e., via either the Handover Preparation or Data Collection Reporting Initiation procedure. This in addition to the agreement from the last meeting [3]:

The request and reporting to support measured multi-hop UE trajectory between gNBs should utilize the Data Collection Reporting Initiation procedure and Data Collection Reporting procedure, at least for the first hop.

Observation 6: Down-selection between Option1 (Parallel transmission from each hop gNB to the initial source gNB) and Option2 (Hop-by-hop transmission) should also be based on whether the data collection parameters (i.e., collection time duration and number of visited cells) should be transferred via Handover Preparation (UE-associated procedure) or Data Collection Reporting Initiation (non UE-associated procedure).

In our view, transferring of UE-specific information via a non UE-associated procedure is something that is already allowed in current XnAP specification. To give an example, the reporting of both single-hop measured UE trajectory and UE performance – which are UE-specific – are transferred via a non UE-associated procedure, that is, Data Collection Reporting procedure (within the DATA COLLECTION UPDATE message, by means on the UE Associated Info Result List IE).

Observation 7: Transferring of UE-specific information via a non UE-associated procedure is already allowed in current XnAP specification.

By assuming Proposal 1 and Proposal 2 for the propagation of the cell-based multi-hop UE trajectory prediction are agreed, let’s assume that the UE trajectory is predicted by Node 1 (source gNB) and spans cell A and cell B in Node 2 (intermediate node) as well as cell C, cell D and cell E in Node 3 (final target gNB), and for simplicity the UE is expected to stay for 1ms in each predicted visited cell:

Figure 2 - Predicted UE trajectory spanning cells of two gNBs (Node 2 and Node 3).

By leveraging the UE trajectory prediction, in order for the source to retrieve the measured UE trajectory from involved nodes it needs to configure such reporting by initiating Data Collection Reporting Initiation procedures.

Hence, for the 1st hop (Node 1 to Node 2), similar to the :

- Node 1 starts a Data Collection Reporting Initiation with Node 2, where it configures the data collection in terms of collection time duration and number of visited cells: in our understanding, for the example under discussion the collection time duration should at least be equal to the expected time of stay in each cell of Node 2, i.e., 2ms (1ms per each predicted cell) while the number of visited cells should be set to 2 (since in Node 2 the UE is expected to visit cells A and B);

- According to Proposal 1, Node 1 sends the full UE trajectory prediction consisting of cells A, B, C, D, E to Node 2 via the HANDOVER REQUEST message; providing the full UE trajectory prediction is needed for Node 2 to understand that it may handover the UE to cells in Node 3 (from cell B in Node 2 to cell C in Node 3);

- When at least one of the exit conditions to terminate the collection of the measured UE trajectory in Node 2 is fulfilled, if Option1 is considered, Node 2 sends the portion of the measured UE trajectory (i.e., for cells in Node 2) to Node 1 via the Data Collection Reporting procedure between Node 2 and Node 1. While, if Option2 is considered, this transfer of data may not be needed if it is Node 2 that sends the full measured UE trajectory to Node1 once it has collected the portion of the UE trajectory measured while the UE is in Node 3.

While, for the 2nd hop (Node 2 to Node 3):

- If Option1 is considered, it is Node 1 (that predicted the full UE trajectory) that starts a Data Collection Reporting Initiation with Node 3, where it configures the data collection in terms of collection time duration and number of visited cells: in our understanding, for the example under discussion the collection time duration should at least be equal to the expected time of stay in each cell of Node 3, i.e., 3ms (1ms per each predicted cell) while the number of visited cells should be set to 3 (since in Node 3 the UE is expected to visit cells C, D and E);

- If Option2 is considered, it is now Node 2 (that received the full UE trajectory from Node 1) that starts a Data Collection Reporting Initiation with Node 3, where it configures the data collection in terms of collection time duration (3ms, 1ms per each predicted cell) and number of visited cells (3, since in Node 3 the UE is expected to visit cells C, D and E);

- According to Proposal 2, Node 2 sends the UE trajectory prediction consisting of only cells C, D, E to Node 3 via the HANDOVER REQUEST – there is no need for Node 3 to know the cells which were predicted to be visited in Node 2;

- When at least one of the exit conditions to terminate the collection of the measured UE trajectory in Node 3 is fulfilled,

  - If Option1 is considered, Node 3 sends the portion of the measured UE trajectory (i.e., for cells in Node 3) to Node 1 directly via the Data Collection Reporting procedure between Node 3 and Node 1. Node 1 then merges this portion of the UE trajectory comprising cells in Node 3 with the portion of the UE trajectory comprising cells in Node 2;

  - If Option2 is considered, Node 3 sends the portion of the measured UE trajectory (i.e., for cells in Node 3) to Node 2 via the Data Collection Reporting procedure between Node 3 and Node 2. Node 2, in turn, sends such portion to Node 1 which eventually merges this portion of the UE trajectory comprising cells in Node 3 with the portion of the UE trajectory comprising cells in Node 2; alternatively, Node 2 can perform the merging of the portions of the measured UE trajectory (the one collected by Node 2 and the one collected by Node 3 and reported to Node 2) and send the full measured UE trajectory to Node 1.

In our view, the main difference between Option1 and Option2 is the slight increase in complexity of Option2 compared to Option1; in fact, Option2 to actually work, each portion of the measured UE trajectory collected by a certain node needs to go Node 1 via each visited intermediate node. In our example, the portion of the measured UE trajectory comprising cells in Node 3 needs to be sent to Node 2 before reaching Node 1. This may pose some additional requirements on the time Node 2 is requested to keep the UE context of the handed over UE for which the multi-hop measured UE trajectory needs to be provided to Node 1. Moreover, it is clear that it will be up to Node 1 only to merge each received portion of the measured UE trajectory collected in each visited node to derive the full measured UE trajectory.

Observation 8: Option2 (hop-by-hop transmission) shows higher complexity compared to Option1 (parallel transmission from each gNB to the initial source gNB) due to the need of each portion of the measured UE trajectory collected in each visited node to go through each intermediate node.

Observation 9: In Option1 (parallel transmission from each gNB to the initial source gNB) there is no uncertainty on the fact that it will be up to Node 1 only (i.e., the initial source gNB) to merge each received portion of the measured UE trajectory collected in each visited node to derive the full measured UE trajectory.

It could be worth to note that, with Option 1, the issue on the potential unavailability of the Xn interface between the initial source gNB and a “far away” final target gNB could be more severe compared to Option2, where such “far away” gNB could be reached via a set of intermediate nodes which are Xn-connected one to each other. However, we should consider that, in general, a UE trajectory predictions spanning many gNBs up to a “far away” gNB is a piece of information which “looks” very far also in the future in terms of timing, and hence it is very likely that such prediction is very inaccurate. Therefore, in that sense, Option1 seems a good trade-off between accuracy of the prediction and complexity in the retrieval of the measured UE trajectory.

Based on the above, our analysis suggests to select Option1 (parallel transmission from each gNB to the initial source gNB) for the retrieval of the measured UE trajectory across multiple gNBs.

Proposal 4: RAN3 to agree to select Option1 (parallel transmission from each gNB to the initial source gNB) for the retrieval of the measured UE trajectory across multiple gNBs.

## 2.4 Other aspects

As we stated at the beginning, in the last meeting the definition of multi-hop predicted UE trajectory was agreed and a TP for the TR 38.745 in was agreed as well. However, we notice that there is a mismatch between the definition as captured in the agreed TP for the TR in [4] and the definition included in the Chairman’s minutes in [3]:

--- definition captured in the Chairman’s minutes --- Multi-hop predicted UE trajectory across gNBs consists of a list of cells belonging to one or more gNBs where the UE is expected to connect and these cells are listed in chronological order. --- definition captured in agreed TP for TR 38.745 in R3-257329 --- Multi-hop predicted UE trajectory across gNBs consists of a list of cells belonging to gNBs where the UE is expected to connect and these cells are listed in chronological order.

We actually think that what has been captured in the TR is semantically correct compared to the agreement in the meeting minutes, hence we propose to update the definition of the multi-hop predicted UE trajectory and align it with the one as captured in the TR, i.e., remove “one or more” in the definition captured in the Chairman’s minutes from RAN3#129bis.

Proposal 5: RAN3 to agree to update the definition of the multi-hop predicted UE trajectory and align it with the one as captured in the TR 38.745.

A pCR to TR 38.745 reflecting the above proposals can be found in the Annex of this paper.

# 3. Conclusion

Based on the above discussion, we have the following observations and proposals:

Observation 1: In Rel-18 a UE-associated procedure, i.e., Handover Preparation, is used by the source gNB which predicted the cell-based UE trajectory to provide such prediction to the target gNB.

Proposal 1: RAN3 to agree to transfer the overall multi-hop UE trajectory prediction predicted by the source gNB(-CU) to the 1st hop target gNB by enhancing the single-hop UE trajectory prediction transmission mechanism specified in Rel-18 (i.e., in the HANDOVER REQUEST message).

Proposal 2: RAN3 to agree that, for subsequent hops, from the intermediate node to the next hop target gNB there is no need to transfer the overall UE trajectory prediction as received from the source gNB(-CU). Each intermediate gNB removes its own cells actually visited from the list or predicted cells.

Observation 2: In Rel-18 a non UE-associated procedure, i.e., Data Collection Reporting Initiation, is used by the source gNB to configure the collection and one-time reporting from the target gNB of the cell-based measured UE trajectory.

Observation 3: In Rel-18 a non UE-associated procedure, i.e., Data Collection Reporting, is used by the target gNB to report cell-based measured UE trajectory to the source gNB.

Observation 4: There are technical challenges preventing the source gNB to be provided with the measured multi-hop UE trajectory feedback.

Proposal 3: RAN3 to agree to capture the technical challenges preventing the source gNB to be provided with the measured multi-hop UE trajectory feedback in the use case description of TR 38.745.

Observation 5: Option4: Measured UE trajectory transfer via AMF (i.e. Node3 to AMF, AMF to Node1) should not be considered in future discussions pertaining the multi-hop UE trajectory scenario.

Observation 6: Down-selection between Option1 (Parallel transmission from each hop gNB to the initial source gNB) and Option2 (Hop-by-hop transmission) should also be based on whether the data collection parameters (i.e., collection time duration and number of visited cells) should be transferred via Handover Preparation (UE-associated procedure) or Data Collection Reporting Initiation (non UE-associated procedure).

Observation 7: Transferring of UE-specific information via a non UE-associated procedure is already allowed in current XnAP specification.

Observation 8: Option2 (hop-by-hop transmission) shows higher complexity compared to Option1 (parallel transmission from each gNB to the initial source gNB) due to the need of each portion of the measured UE trajectory collected in each visited node to go through each intermediate node.

Observation 9: In Option1 (parallel transmission from each gNB to the initial source gNB) there is no uncertainty on the fact that it will be up to Node 1 only (i.e., the initial source gNB) to merge each received portion of the measured UE trajectory collected in each visited node to derive the full measured UE trajectory.

Proposal 4: RAN3 to agree to select Option1 (parallel transmission from each gNB to the initial source gNB) for the retrieval of the measured UE trajectory across multiple gNBs.

Proposal 5: RAN3 to agree to update the definition of the multi-hop predicted UE trajectory and align it with the one as captured in the TR 38.745.

A pCR to TR 38.745 reflecting the above proposals can be found in the Annex of this paper.

# 4. Reference

- RP-251868, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN Phase 3” (ZTE, NEC), 3GPP RAN#108

- RP-252867, “Revised SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN Phase 3” (ZTE, NEC), 3GPP RAN#109

- 3GPP RAN3#129bis Chairman’s notes (RAN3_129bis_agenda_20251017_EOM2.doc)

- R3-257329, (TP to TR 38.745) Consideration on Multiple-hop UE trajectory, ZTE Corporation, LG Electronics Inc, Samsung, Qualcomm, Nokia, FiberCop, Huawei, Ericsson, CATT, NEC, CMCC

# Annex – pCR to TR 38.745 (based on TR 38.745 V0.1.1 (2025-11))

<<<<<<<<<< Start of Changes >>>>>>>>>>

# 4	Use cases and Solutions

## 4.1	Multiple-hop UE trajectory

### 4.1.1	Use case description

Editor’s Note: Capture the description of use case

In Rel-18, the cell-based UE trajectory prediction is limited to the first-hop target NG-RAN node.

Multi-hop predicted UE trajectory across gNBs consists of a list of cells belonging to gNBs where the UE is expected to connect and these cells are listed in chronological order.

Provisioning of the multi-hop predicted UE trajectory could be done by enhancing the existing mechanism introduced for transferring the cell-based UE trajectory prediction, i.e., via the Handover Preparation procedure. However, collecting the measured UE trajectory across multiple gNBs to eventually be provided to the source gNB is challenging from a complexity point of view due to a number of reasons:

- the need to keep the UE context in the source gNB after the UE has been handed over to the subsequent hop target gNBs, so that the source gNB is able to then associate the received measured UE trajectory as feedback information;

- the potential lack of the Xn interface between the source gNB and “far away” target gNB(s): if a trajectory prediction covers the N future cell hops, it is very likely that the gNB serving the nth cell will not be Xn-connected to the source gNB that produced the prediction;

- cell deployment changes (e.g. cells’ activation and deactivation) during the time interval between the UE trajectory prediction is provided by the source gNB and the measured UE trajectory is received at the source gNB.

<<<<<<<<<< Next Change >>>>>>>>>>

### 4.1.2	Solutions and standard impacts

Editor’s Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

#### 4.1.2.1	Locations for AI/ML Model Training and AI/ML Model Inference

The following solutions are considered for supporting multi-hop UE trajectory:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

-	AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

In case of CU-DU split architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

#### 4.1.2.2	Input data of multi-hop UE trajectory

The following data is required as input data.

From the local node:

-	Multi-hop measured/predicted UE trajectory

-	Measured/predicted radio resource status

-	Measured/predicted UE traffic

From the neighbouring gNBs:

-	UE’s history information from neighbour

-	QoS parameters and the performance information of historical HO-ed UE (e.g., loss rate, delay, etc.)

-	Measured/predicted radio resource status

-	UE handovers in the past that were successful and unsuccessful, including too-early, too-late, or handover to wrong (sub-optimal) cell, based on existing SON/RLF report mechanism.

From the UE:

-	UE location information (e.g., serving cell ID, moving velocity) interpreted by gNB implementation when available.

-	Radio measurements related to serving cell and neighbouring cells associated with UE location information, e.g., RSRP, RSRQ, SINR.

-	UE Mobility History Information.

#### 4.1.2.3	Output of multi-hop UE trajectory

The following information can be generated as output:

-	Multi-hop UE trajectory prediction (list of cells predicted to be visited, optionally including the predicted time of stay of the UE in each cell)

-	UE traffic prediction in each hop (output used internally by the RAN)

#### 4.1.2.4	Feedback of multi-hop UE trajectory

The following data is required as feedback data:

-	Measured multi-hop UE trajectory

-	per-UE UE performance (Average UE Throughput DL/UL, Average Packet Loss DL/UL, Average Packet Delay DL/UL)

-	Measured radio resource status from target gNBs

#### 4.1.2.5	Standard impact

The following standard impacts are listed for subsequent Rel-20 normative work compared to what was specified in Rel-18.

Xn interface:

-	Enhance the existing Handover Preparation and Data Collection Reporting (Initiation) procedures to transfer the multi-hop UE trajectory prediction and to collect the measured multi-hop UE trajectory, respectively.

<<<<<<<<<< End of Changes >>>>>>>>>>