3GPP TSG-RAN WG3 #130																	R3-258556

Dallas, USA, 17 - 21 November 2025

Title: 	Fast LTM recovery for Inter-CU LTM

Source: 	Ericsson, Jio Platforms, Lenovo, Ofinno

Agenda item:	9.2.5

Document Type:	Discussion and Decision

# Introduction

In this contribution, we present an issue for inter-CU LTM related to Fast recovery, and a potential solution to that.

# Discussion

For both Rel-18 and Rel-19 the source node can configure a UE for attempting Fast LTM recovery in case of failures. In 38.331 this corresponds to the “attemptLTM-Switch” parameter, included in the LTM-Config IE:

LTM-Config-r18 ::=   SEQUENCE {

ltm-ReferenceConfiguration-r18        SetupRelease {ReferenceConfiguration-r18}                             OPTIONAL,   -- Need M

ltm-CandidateToReleaseList-r18        SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-CandidateId-r18    OPTIONAL,   -- Need N

ltm-CandidateToAddModList-r18         SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-Candidate-r18      OPTIONAL,   -- Need N

ltm-ServingCellNoResetID-r18          INTEGER (1..maxNrofLTM-Configs-plus1-r18)                             OPTIONAL,   -- Need N

ltm-CSI-ResourceConfigToAddModList-r18  SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourceConfigurations-r18)) OF LTM-CSI-ResourceConfig-r18

OPTIONAL,   -- Need N

ltm-CSI-ResourceConfigToReleaseList-r18 SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourceConfigurations-r18)) OF LTM-CSI-ResourceConfigId-r18

OPTIONAL,   -- Need N

attemptLTM-Switch-r18                 ENUMERATED {true}                                                     OPTIONAL,   -- Cond LTM-MCG

ltm-ServingCellUE-MeasuredTA-ID-r18   INTEGER (1..maxNrofLTM-Configs-plus1-r18)                             OPTIONAL,   -- Need N

...,

[[

ltm-ServingCellNoSecurityChangeID-r19       LTM-NoSecurityChangeId-r19                                      OPTIONAL,   -- Need N

ltm-ServingCellExecutionCondition-r19       SetupRelease {LTM-ExecutionConditionList-r19}                   OPTIONAL    -- Need M

]]

The “Fast LTM recovery” parameter is unique per UE and given that it is not a lower layer parameter, it is our understanding that the CU (source CU) decides its value. In Rel-18, only one CU is involved in LTM, and the use of a single parameter fits very well. In Rel-19, for inter-CU LTM, RAN2 decided to keep the same signaling design, i.e., using a single parameter, irrespective of the number of CUs involved in the LTM preparation.

- There is only one parameter to configure a UE for Fast LTM Recovery, which applies to intra-CU LTM and inter-CU LTM.

This creates some problems at the network side, as we briefly discuss below.

First, from the source CU point of view, it does not know the preference of other CUs involved in LTM in terms of using/not using Fast LTM recovery. Since there only one parameter available to configure the UE, the source CU needs to take a decision “on behalf” of other candidate CUs. It would make sense, for example, that if all involved CUs (source CU and candidate CUs) prefer to use Fast LTM recovery, the UE is configured to use Fast LTM recovery. However, the source CU does not know the candidate CU’s “preference” on this.

- In case of inter-CU LTM, the source CU decides whether the UE is configured for Fast LTM Recovery, without knowing the candidate CU’s preference on this.

Another issue is that if some (or all) the candidate CUs (e.g., a CU2 and CU3) would like to use Fast LTM recovery, but the source CU (e.g., CU1) decides otherwise, the UE is configured for not using Fast LTM recovery, and the candidate CUs remains unaware of that. Given that RAN2 opted for keeping only one Fast LTM recovery configuration parameter for the UE (instead of, for example, having different parameters for different LTM candidate cells), we think that this aspect should be discussed in RAN3. In particular, it seems that some kind of coordination/sync-up is needed to provide the source CU good input for its decision, and also to let every CU involved to be in-sync in terms of knowing how the UE has been configured. During offline discussion, some comments assumed that candidate CUs always support fast LTM recovery based on their awareness of the UE capability. However, we think these aspects as independent and do not consider it is a realistic assumption.

- In case of inter-CU LTM, the candidate CUs are not aware of whether the source CU configured the UE for Fast LTM Recovery or not.

- For inter-CU LTM, the source CU receives from the candidate CUs their preference on Fast LTM Recovery for the UE.

- For inter-CU LTM, a candidate CU is informed about whether the UE can use Fast LTM Recovery.

From RAN3 perspective, we think that the coordination/sync-up can be achieved using the same logical steps used to achieve a sync-up among the CUs for other information, namely:

Step 1: The source CU requests to the candidate CUs its preference on Fast LTM recovery

Step 2: The source CU receives from the candidate CUs its preference on Fast LTM recovery

Step 3: The source CU decides how to configure the UE with respect to Fast LTM recovery

Step 4: The source CU propagates the setting of the UE to the candidate CUs

Step 1 and step 2 can be achieved with the initial Handover Preparation procedure. For example, for the given cell being prepared, the source CU requests to the candidate CU its preference for Fast LTM Recovery in the HANDOVER REQUEST message. Then, the candidate CU sends to the source CU its preference in the HANDOVER REQUEST ACKNOWLEDGE message.

- Add a new IE (e.g., “Fast LTM Recovery Request”) in the HANDOVER REQUEST ACKNOWLEDGE message, to inform the source CU about the candidate CU preference about Fast LTM Recovery.

- Add a new IE (e.g., “Fast LTM Recovery Response”) in the HANDOVER REQUEST ACKNOWLEDGE message, to inform the source CU about the candidate CU preference about Fast LTM Recovery.

Step 3 is internal to the source CU, so we don’t see any RAN3 specifications impact for that.

Step 4 i.e., the sync-up can be achieved with the LTM Configuration Update procedure, which is already used for this purpose. For example, the source CU, during LTM preparation sends its decision to each candidate CU in the LTM CONFIGURATION UPDATE message.

- Add a new IE (e.g., “Fast LTM Recovery”) in the LTM CONFIGURATION UPDATE message, to inform a candidate CU about the UE configuration about Fast LTM Recovery.

- Agree on the CR for 38.423 in R3-258557 that captures the above proposals.

# Conclusions and Proposals

In this contribution we made observations and proposals for different LTM related topics, as listed below.

Observation 1	There is only one parameter to configure a UE for Fast LTM Recovery, which applies to intra-CU LTM and inter-CU LTM.

Observation 2	In case of inter-CU LTM, the source CU decides whether the UE is configured for Fast LTM Recovery, without knowing the candidate CU’s preference on this.

Observation 3	In case of inter-CU LTM, the candidate CUs are not aware of whether the source CU configured the UE for Fast LTM Recovery or not.

Proposal 1	For inter-CU LTM, the source CU receives from the candidate CUs their preference on Fast LTM Recovery for the UE.

Proposal 2	For inter-CU LTM, a candidate CU is informed about whether the UE can use Fast LTM Recovery.

Proposal 3	Add a new IE (e.g., “Fast LTM Recovery Request”) in the HANDOVER REQUEST ACKNOWLEDGE message, to inform the source CU about the candidate CU preference about Fast LTM Recovery.

Proposal 4	Add a new IE (e.g., “Fast LTM Recovery Response”) in the HANDOVER REQUEST ACKNOWLEDGE message, to inform the source CU about the candidate CU preference about Fast LTM Recovery.

Proposal 5	Add a new IE (e.g., “Fast LTM Recovery”) in the LTM CONFIGURATION UPDATE message, to inform a candidate CU about the UE configuration about Fast LTM Recovery.

Proposal 6	Agree on the CR for 38.423 in R3-258557 that captures the above proposals.