3GPP TSG RAN3 meeting #130   	         	                                     R3-258580

Dallas, US, 17 – 21 November 2025

Agenda item: 	    11.2.2

Source: 	China Unicom

Title: 	Discussion on MRO enhancements for C-LTM

Document for:	Discussion

1	Introduction

At RAN#108, the new Work Item on Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR Phase 5 was agreed and the WID was revised at RAN#109 in[1] introducing the following objectives in the core part:

The objective of this work item is to specify data collection enhancement in NR standalone for SON/MDT purpose. The specific objectives of this work item are: - MRO enhancement for R19 mobility mechanisms, including inter-CU LTM and intra-CU conditional LTM [RAN3, RAN2]: - Specification of the inter-node information exchange, including possible enhancements on the existing interfaces [RAN3] - Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2] If needed, co-operate with RAN1, SA5.

This contribution provides our views on the scenarios of MRO enhancements which can be supported in Rel-20.

2	Discussion

2.1	MRO for conditional LTM

For intra-CU C-LTM, the definition of connection failure cases for CHO defined in 38.300 are taken as base line.

For intra-CU C-LTM, the detection mechanism of connection failure cases in 38.300 are taken as base line.

The agreements from the last meeting listed above.

From the existing definition in 38.300 below, the definition of connection failure cases doesnot differentiate between the LTM type, thus for CLTM, the definition of connection failure cases in 38.300 can be reused directly in R20.

One of the functions of Mobility Robustness Optimization is to detect connection failures that occur due to Too Early or Too Late Handovers, or Handover to Wrong Cell. These problems are defined as follows: - Intra-system Too Late Handover: an RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell. - Intra-system Too Early Handover: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell. - Intra-system Handover to Wrong Cell: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell. In the definition above, the "successful handover" refers to the UE state, namely the successful completion of the RA procedure. In case of LTM cell switch, the "successful handover" in the definitions above refers to a successful completion of the RA procedure in case of RACH-based LTM or the successful reception of RRCReconfigurationComplete in case of RACH-less LTM. In case of LTM Cell switch, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late LTM cell switch, Too Early LTM cell switch and LTM cell switch to Wrong Cell.

Proposal 1: The definition of connection failure cases in 38.300 can be reused directly in R20 for Conditional LTM.

The detailed detection mechanisms for too late handover, too early handover and handover to wrong cell are carried out through the following in the NG-RAN node that served the UE before the reported connection failure: - Intra-system Too Late Handover: there is no recent handover for the UE prior to the connection failure e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt), or if CHO/LTM/CLTM or CHO with candidate SCG(s) is configured but the CHO/LTM/CLTM execution is not initiated for the UE prior to the connection failure, e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt). - Intra-system Too Early Handover: there is a recent handover/LTM/CLTM cell switch for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/the successful re-connect cell/the cell UE attempts LTM recovery is the cell that served the UE at the last handover/LTM/CLTM initialisation or fall back to the source cell configuration in case of DAPS HO. - Intra-system Handover to Wrong Cell: there is a recent handover/LTM/CLTM cell switch for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/ the cell UE attempts to re-connect/the cell UE attempts CHO recovery/the cell UE attempts LTM recovery is neither the cell that served the UE at the last handover/LTM/CLTM initialisation nor the cell that served the UE where the RLF happened or the cell that the handover/LTM/CLTM was initialized toward. The "UE reported timer" above indicates the time elapsed since the last handover initialisation until connection failure or the time elapsed since the CHO/LTM/CLTM execution until connection failure.

The detection mechanism of connection failure cases also can be reused directly in R20 adding CLTM case..

Proposal 2: The detection mechanism of connection failure cases in 38.300 can be reused directly in R20 for inter-CU LTM.

The gNB-CU determines to initiate conditional LTM configuration with the candidate gNB-DU(s). If the candidate gNB-DU accepts the request of conditional LTM configuration, it provides a list of L1-based execution conditions generated for other candidate cells. The source gNB-DU is responsible for the L1-based execution condition. For failures due to inappropriate L1-based execution condition, CU initiates analysis and may forwards the RLF report to the source gNB-DU responsible for the failure.

Proposal 3:The source gNB-DU is responsible for the failure caused by inappropriate L1-based execution condition.

Proposal 4:The XnAP should be enhanced to transfer the RLF for failures due to inappropriate L1-based execution condition.

3	Conclusion

Proposal 1: The definition of connection failure cases in 38.300 can be reused directly in R20 for Conditional LTM.

Proposal 2: The detection mechanism of connection failure cases in 38.300 can be reused directly in R20 for inter-CU LTM.

Proposal 3:The source gNB-DU is responsible for the failure caused by inappropriate L1-based execution condition.

Proposal 4:The XnAP should be enhanced to transfer the RLF for failures due to inappropriate L1-based execution condition.

4   References

- RP-234038: Revised WID: Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR Phase 5; TSG RAN Meeting #109, Beijing, China, Sep 15-18, 2025