3GPP TSG-RAN WG3 Meeting #130	R3-258584

Dallas, US, 17 - 21 November 2025

Title:	Corrections on inter-CU MCG LTM without SN change

Agenda Item:	9.2.5

Source:	Huawei, Jio Platforms, Lenovo, Google

Document for:	Discussion

- Introduction

The Rel-19 Mobility Enhancement WI was completed at last meeting. Inter-CU MCG LTM with SN unchanged case was supported. In this case, both the MN key and the SN key need to be updated. However, according to the current BL CR, it is not clear when the SN gets the updated key and when to apply the new SN key.

This contribution provides further analysis on the SN key update procedure.

2	Discussion

2.1	SN key update

In the inter-CU MCG LTM with SN unchanged case, the MN key needs to be updated since the MN PDCP anchor is changed. In addition, the SN key needs to be updated as well since the SN key is derived based on the MN key. RAN2 discussed the UE side behaviour about how to derive the new SN key, and the agreement is shown below:

RAN2 #129bis 3. NW configures the corresponding sk-Counter in all LTM candidate configurations, and UE uses the configured value for generating the SN key when security key update is performed in MCG. RAN2 130 16. (RRC-7) The legacy sk-counter is used for inter-MN LTM (this means sk-counter value can be re-used).

According to the above RAN2 agreements, the UE can update the SN key by the new MN key and the configured sk-counter. However, it is not clear how does the SN get the updated SN key in the network side.

In the latest TS 37.340, for the initial LTM cell switch, the SN Addition Request message from the candidate/target MN to the SN in step 2 will include the updated SN key, which is a legacy behaviour for SN addition.

However, for subsequent LTM cell switch, it is not possible to use the SN Addition Request message to send the updated SN key to the SN, since at the preparation phase, the updated MN key is not determined yet for the subsequent LTM cell switch. So that step 28a is introduced to deliver the updated SN key, as shown in the figure below.

Figure 10.7.2-2: Inter-MN MCG LTM with Secondary Node […] 28a. The other potential candidate MN sends the SN Modification Request message to the other candidate SN to update the security information

Observation 1: For subsequent inter-CU MCG LTM with SN unchanged, the candidate MN uses the SN Modification Request message to deliver new SN key to the SN.

There are two points that may cause ambiguity:

- The LTM Information SN Modification IE includes LTM Reconfiguration IE which only has one enumerated value: “intra-MN-LTM”. However, in the subsequent inter-CG MCG LTM with SN kept, if we use the SN Modification Request to reconfigure the SN key, it should be “inter-MN-LTM”.

- The SN may receive multiple SN keys from different candidate MNs. When the SN receives the new SN keys in step 28a, it should not immediately apply the keys. Instead, it should wait until the UE successfully switches to a target MN, and the SN chooses the corresponding SN key associated to the target MN and apply it. The SN knows the correct target MN and when to apply the new SN key by receiving the SN Reconfiguration Complete message from the target MN. Procedural text is missing to capture the security key applying behaviour.

Proposal 1: For subsequent inter-CU MCG LTM with SN unchanged, to add a new enumerated value “inter-MN-LTM” in the LTM Reconfiguration IE in the SN Modification Request message to update the SN key.

Proposal 2: To capture the security key applying behaviour for the SN that the SN begins to apply the new SN key when receiving the SN Reconfiguration Complete message.

2.2	Stage 2 message flow refreshment

In the TS 37.340, the step 4a corresponds to the Xn-U Address Indication message sent from the source MN to the SN for early data forwarding of SN-terminated bearers. This step is currently specified as mandatory. However, the Xn-U Address Indication message should be optional as it is only applicable for the case of inter-CU MCG LTM with SN kept, and is not needed in case of inter-CU MCG LTM with SN addition.

Proposal 3: The step 4a should be defined as an optional procedure, applying only in the case of inter-CU MCG LTM with SN kept.

Furrthermore, step 4a may need to be repeated multiple times, as the message may come from multiple target MNs. A note similar to CHO with Secondary Node should be added after the step 4a to clarify this behavior.

Proposal 4: A note should be added after the step 4a stating that separate Xn-U Address Indication procedures may be initiated to provide different forwarding addresses from multiple target MNs.

3	Conclusion

This document proposes:

Proposal 1: For subsequent inter-CU MCG LTM with SN unchanged, to add a new enumerated value “inter-MN-LTM” in the LTM Reconfiguration IE in the SN Modification Request message to update the SN key.

Proposal 2: To capture the security key applying behaviour for the SN that the SN begins to apply the new SN key when receiving the SN Reconfiguration Complete message.

Proposal 3: The step 4a should be defined as an optional procedure, applying only in the case of inter-CU MCG LTM with SN kept.

Proposal 4: A note should be added after the step 4a stating that separate Xn-U Address Indication procedures may be initiated to provide different forwarding addresses from multiple target MNs.

The XnAP CR and TS37.340 draft CR are provided in [1] and [2].

Reference

[1] R3-258585	Stage 2 corrections to inter-CU MCG LTM

[2] R3-258586	Correction on security key update for inter-CU MCG LTM