3GPP TSG-RAN WG3 Meeting #130	R3-258686

# Dallas, USA, 17th – 21st Nov, 2025

Agenda item:	9.2.10

Source: 	Samsung, Qualcomm, Charter

Title: 	Discussion on the open issues regarding L1 UE-to-UE CLI configuration

Document for:	Discussion and decision

Introduction

During the meeting of RAN3#129bis, there were some open issues of SBFD regarding L1 UE-to-UE CLI:

Clarify how the gNB-CU can be aware about whether DU has configured L1 UE-to-UE CLI measurement configuration in CellGroupConfig.

CellGroupConfig is generated and compiled by which node, i.e., CU or DU?

This paper provides further discussion on the open issues above.

Discussion

In [1], It was proposed to add an explicit indication in the DU to CU RRC Information IE to information CU that DU has configured L1 UE-to-UE CLI measurements in the CellGroupConfig, so that CU can avoid configuring L1 and L3 CLI simultaneously. However, the opponents were holding the view that it is the CU to compile the CellGroupConfig and CU would undoubtedly know whether L1 CLI is configured by DU. However, there was no consensus amongst companies regarding this point, and the following open issue was captured in the chair notes:

CellGroupConfig is generated and compiled by which node, i.e., CU or DU?

The answer to the open issue above is obvious in our mind —— it should be the DU to generate and compile the CellGroupConfig, and both higher layer specs and RRC spec can provide strong evidence to this conclusion.

According to TS38.473, CellGroupConfig is generated by the DU and transmitted to CU via the DU to CU RRC Information IE in UE context management related messages. After receiving the DU to CU RRC Information IE, it is clearly stated in the procedure text of the UE context management related messages that “The CellGroupConfig IE shall transparently be signaled to the UE as specified in TS 38.331 [8].”

With reference to TS38.331, CellGroupConfig is also clearly specified as OCTET STRING in RRC messages, which aligns with the statement in F1AP, taking RRCReconfiguration message as an example below:

RRCReconfiguration message

-- ASN1START

-- TAG-RRCRECONFIGURATION-START

RRCReconfiguration ::=                  SEQUENCE {

rrc-TransactionIdentifier               RRC-TransactionIdentifier,

criticalExtensions                      CHOICE {

rrcReconfiguration                      RRCReconfiguration-IEs,

criticalExtensionsFuture                SEQUENCE {}

}

}

RRCReconfiguration-IEs ::=              SEQUENCE {

radioBearerConfig                       RadioBearerConfig                                                      OPTIONAL, -- Need M

secondaryCellGroup                      OCTET STRING (CONTAINING CellGroupConfig)                              OPTIONAL, -- Cond SCG

measConfig                              MeasConfig                                                             OPTIONAL, -- Need M

lateNonCriticalExtension                OCTET STRING (CONTAINING RRCReconfiguration-v15t0-IEs)                 OPTIONAL,

nonCriticalExtension                    RRCReconfiguration-v1530-IEs                                           OPTIONAL

}

-- Regular non-critical extensions:

RRCReconfiguration-v1530-IEs ::=            SEQUENCE {

masterCellGroup                         OCTET STRING (CONTAINING CellGroupConfig)                              OPTIONAL, -- Need M

fullConfig                              ENUMERATED {true}                                                      OPTIONAL, -- Cond FullConfig

dedicatedNAS-MessageList                SEQUENCE (SIZE(1..maxDRB)) OF DedicatedNAS-Message                     OPTIONAL, -- Cond nonHO

masterKeyUpdate                         MasterKeyUpdate                                                        OPTIONAL, -- Cond MasterKeyChange

dedicatedSIB1-Delivery                  OCTET STRING (CONTAINING SIB1)                                         OPTIONAL, -- Need N

dedicatedSystemInformationDelivery      OCTET STRING (CONTAINING SystemInformation)                            OPTIONAL, -- Need N

otherConfig                             OtherConfig                                                            OPTIONAL, -- Need M

nonCriticalExtension                    RRCReconfiguration-v1540-IEs                                           OPTIONAL

}

<unrelated text omitted>

N3C-ExtIndirectPathAddChange-r19 ::=            SEQUENCE (SIZE(1..maxNrofN3C-RelayUE-r19)) OF N3C-RelayUE-Info-r18

-- TAG-RRCRECONFIGURATION-STOP

-- ASN1STOP

With the support of standards, we believe there is no doubt that the CU signals the CellGroupConfig, which is generated by DU, transparently to UE as OCTET STRING.

Observation 1: From standard perspective, CU signals the CellGroupConfig transparently to UE as OCTET STRING.

Now it is also clear that the DU shall compile the CellGoupConfig before sending it to CU, because CU would only compile the final RRC message, in which the CellGroupConfig is included as an OCTET STRING. In other words, it should be the DU to generate the CellGroupConfig and compile it.

Proposal 1: It should be DU that generates and compiles the CellGroupConfig, and CU only compiles the RRC message with the CellGroupConfig included as OCTET STRING.

With the above understanding, the issue mentioned in [1] still stands, i.e., the CU is not aware of whether L1 CLI measurement is configured in CellGroupConfig. Because CU only compiles CellGroupConfig as an OCTET STRING, the detailed content, including the L1 CLI configuration if configured, is supposed to be transparent to the CU. If there is not any other explicit indication, the only way for the CU to understand the presence of L1 CLI configuration is to proactively decode the content of CellGroupConfig. However, such behaviour is not a standardized behaviour, and shall not be recommended as a perfect solution for the issue we discussed.

In the whole pack of specifications, there is not any rule anywhere to mandate CU to read the content of CellGroupConfig, which means the CU does not necessarily decode the IEs in CellGroupConfig. So, when we consider how the CU can be aware of the L1 CLI configuration in CellGroupConfig, the basic assumption should that CU would not read the content of CellGroupConfig. Some vendors may insist to let CU read the CellGroupConfig, in order to figure out whether L1 CLI configuration is there, but in our mind, this is not a harmonized solution in any sense, and different vendors may have different implementations.

Observation 2: Without extra indication, CU can only know whether DU has configured L1 CLI by proactively decoding the CellGroupConfig.

Observation 3: CU is not required to read the content of CellGroupConfig, which is signalled to the UE transparently.

With the above discussion, we think it is clear that an explicit indication from DU to CU is necessary, to inform CU that L1 CLI is configured in the CellGroupConfig. This solution is a standardized solution which can guarantee the awareness of L1 CLI configuration in CU, without relying on decoding CellGroupConfig. Without the indication, the solution to avoid simultaneously configuring L1 and L3 CLI configuration is incomplete, because there is no other promised way to make CU be aware of the presence of L1 CLI configuration.

Proposal 2: Include an indication for L1 UE-to-UE CLI configuration from DU to CU in the DU to CU RRC Information IE over F1.

A corresponding CR is provided in R3-258687 [2].

Proposal 3: Agree the CR in R3-258687.

Conclusion

In this paper, the following proposals are provided:

Observation 1: From standard perspective, CU signals the CellGroupConfig transparently to UE as OCTET STRING.

Proposal 1: It should be DU that generates and compiles the CellGroupConfig, and CU only compiles the RRC message with the CellGroupConfig included as OCTET STRING.

Observation 2: Without extra indication, CU can only know whether DU has configured L1 CLI by proactively decoding the CellGroupConfig.

Observation 3: CU is not required to read the content of CellGroupConfig, which is signalled to the UE transparently.

Proposal 2: Include an indication for L1 UE-to-UE CLI configuration from DU to CU in the DU to CU RRC Information IE over F1.

Proposal 3: Agree the CR in R3-258687.

References

[1] R3-256692 CR to 38.473 on SBFD

[2] R3-258687 CR to 38.473 on SBFD