3GPP TSG-RAN WG3 Meeting #130	R3-258799

# Dallas, US, 17th – 21st Nov,2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.320 | CR | - | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for Continuous management-based MDT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Ericsson,ZTE, CMCC, Samsung, NEC, Huawei, FiberCop |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_NGRAN_enh-Core |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduce Continuous Management-based MDT. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce Continuous Management-based MDT in the definitions. - Introduce immediate and logged MDT mode in MDT mode part. - Introduce the new section to describe the solution for continuous MDT measurement collection. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Continuous Management-based MDT is not described at Stage 2 level. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 4.1, 5.4.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.423 CR1599 TS 38.401 CR 0484 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] apply.

Continuous Management-based MDT: as specified in TS 32.422 [6].

Immediate MDT: MDT functionality involving measurements performed by the UE in CONNECTED state and reporting of the measurements to RAN available at the time of reporting condition as well as measurements by the network for MDT purposes.

*** skip unmodified text ***

<Next change>

4.1	General

The general principles and requirements guiding the definition of functions for Minimization of drive tests are the following:

MDT mode

- There are two modes for the MDT measurements: Logged MDT and Immediate MDT. There are also cases of measurement collection not specified as either immediate or logged MDT, such as Accessibility measurements. In some case of management based MDT, the immediate and logged MDT mode is also supported.

2.	UE measurement configuration
It is possible to configure MDT measurements for the UE logging purpose independently from the network configurations for normal RRM purposes. However, in most cases, the availability of measurement results is conditionally dependent on the UE RRM configuration.

--------skipped---------

5.4.3	Support of NPN

MDT is supported in PNI-NPN and SNPN.

5.4.x	Support of Continuous Management-based MDT

Continuous Management-based MDT is a management-based MDT functionality that enables continuous collection of MDT data within a certain area, including both Immediate MDT and Logged MDT as specified in TS 32.422 [6]. This continuous data collection is supported during UE’s RRC state transition from idle/inactive to connected, as well as during connected mode mobility.

Continuous Management-based MDT is activated by OAM towards each gNB within a certain area, as specified in TS 32.422 [6].

If the UE has not been configured with Continuous Management-based MDT, the gNB shall configure the UE selected for Continuous Management-based MDT with Immediate MDT and Logged MDT by using the configuration provided by the OAM; a unique Trace Recording Session Reference is assigned to the UE by the gNB which is valid for both Immediate MDT and Logged MDT, allocated based on the received OAM configuration.

Measurement continuity is ensured in such a way that:

- When a UE transits to RRC_Connected state from RRC_Idle, the gNB shall trigger reporting of MDT measurement logs if available and identify whether the UE needs to be selected for Continuous management-based MDT via the Trace Reference (TR) included in such logs by comparing it with the TR provided by OAM configuration. If the reported TR is the same, and if the area scope and user consent conditions are satisfied, the gNB shall reconfigure the UE with Immediate MDT and may reconfigure the UE with Logged MDT.

- When a UE transits to RRC_Connected state from RRC_Inactive or in case of handover, the gNB shall identify whether the UE needs to be selected for Continuous management-based MDT via the TR included in the RETRIEVE UE CONTEXT RESPONSE message or HANDOVER REQUEST message, respectively. If the gNB determines that the UE is configured with Continuous MDT, and if the area scope and user consent conditions are satisfied, the gNB shall re-configure the UE with Immediate MDT and may reconfigure the UE with Logged MDT..

Correlation of MDT measurement reports provided by the same UE is specified in TS 32.422 [6].

<End of change>