3GPP TSG-RAN WG3 Meeting #130	R3-258879

Dallas, US, 17 – 21 November 2025

Title:	(pCR to TR 38.765) Study on Integrated Sensing And Communication (ISAC) for NR

Agenda Item:	13.1

Source:	China Telecom, Xiaomi

Document for:	pCR

1	Introduction

This contribution provides the baseline pCR for TR 38.765 from RAN3's perspective, captures the agreed TPs listed in [2] ~ [3], building on the content of pCR [1].

2	Reference

- R3-258088, (pCR to TR 38.765) Study on Integrated Sensing and Communication (ISAC) for NR, China Telecom, Xiaomi.

- R3-258820	(TP to pCR 38.765) ISAC general aspects and protocol stacks	Xiaomi, Huawei, CATT, LG Electronics, CMCC, Ericsson, NEC, ZTE, Samsung, China Telecom

- R3-258867	(TP for pCR) RAN-CN procedures and signaling for ISAC	Huawei, CATT, Xiaomi, Nokia, Nokia Shanghai Bell, LG Electronics, CMCC, Ericsson, ZTE, NEC, Samsung, China Telecom

3		pCR to TR 38.765

<<<<<<<<<<<<<<<<<<<< Change Begins >>>>>>>>>>>>>>>>>>>>

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP RP-252819: "Revised SID: Study on Integrated Sensing And Communication (ISAC) for NR".

[x1]	3GPP 3GPP TR 23.700-14: “Study on Integrated Sensing and Communication; Stage 2”.

<<<<<<<<<<<<<<<<<<<< Next change>>>>>>>>>>>>>>>>>>>>

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

Sensing Function: as defined in TR 23.700-14 [x1].

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

<ABBREVIATION>	<Expansion>

SF	         Sensing Function

7	Network architecture

Editor’s note: This section is to capture the study outcome of network architecture. Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. No inter-gNB coordination will be studied.

This clause identifies and describes the logical architecture to support the sensing in the overall 5G system architecture.

Figure 7.1 depicts a logical architecture for ISAC, where the Nx interface is between the gNB and the SF.

Figure 7.1	Logical architecture for ISAC

This logical architecture is independent of the transport, e.g. direct or via the AMF, between the gNB and the SF.

Editor’s Note:  The details of "Nx" interface need further discussion and decision.

The SF selects gNB(s) for sensing based on at least the information about the gNB(s). The gNB information includes the supported sensing area of the gNB.

Editor’s note x1: FFS other information for SF to select gNB(s).

Editor’s note x2: FFS whether the signalling approach is needed for SF to obtain the gNB information.

7.x1	Protocol stack for sensing signalling

Figure 7.x1-1 shows the protocol stack for sensing signaling between the gNB and the SF:

Figure 7.x1-1. Protocol Stack for sensing signalling

Editor’s Note x3: FFS whether Nx-AP is NGAP or a new application protocol.

Editor’s Note x4: FFS on the protocol stack for sensing data report.

8	RAN-CN procedures and signalling

Editor’s note: This section is to capture the study outcome of procedures and signaling aspects between RAN and CN for gNB-based monostatic sensing.

Editor’s Note 2: the following may need further refinement.

The Nx-AP protocol supports the Sensing Information Transfer function.

The following signaling procedures are used for Sensing Information Transfer function:

-	Sensing Initiation: a Class 1 procedure (including Sensing Request, Sensing Response and Sensing Failure);

-	SF-initiated Sensing Abort;

- 	gNB-initiated Sensing Failure indication: a Class 2 procedure.

Editor’s Note 3: FFS on other functions and procedures.

Editor’s Note 4: FFS whether SF-initiated Sensing Abort procedure is class 1 or class 2 procedure.

8.x	Sensing Initiation

Editor’s Note: the following may need further refinement.

The SF initiation procedure is illustrated in Figure 8.x.1.

Figure 8.x-1: Message flow for Sensing Initiation

1.	The SF sends Sensing Request message to the gNB, including the sensing measurement ID, the target sensing area, and the reporting mode (e.g., one time, periodic).

Editor’s Note 2: FFS on other information which can be included in Sensing Request message.

Editor’s Note 3: FFS on the definition of sensing measurement ID and the target sensing area.

2.	The gNB sends Sensing Response message to the SF.

3.	If requested in Step 1, the gNB sends Sensing Report to the SF.

Editor’s Note 4: FFS whether Sensing Report is a signalling procedure.

Editor’s Note 5: FFS whether the sensing results can be included in the Sensing Response message.

8.y	Sensing Abort

Editor’s Note 1: the following may need further refinement.

The SF initiated sensing abort procedure is illustrated in Figure 8.y.1.

Figure 8.y-1: Message flow for Sensing Abort

1.	The SF sends Sensing Abort message to the gNB, which stops the indicated sensing measurement and releases any related resources.

Editor’s Note 2: FFS whether it is a class 1 or class 2 procedure.

8.z	Sensing Failure Indication

The gNB initiated sensing failure indication procedure is illustrated in Figure 8.z-1.

Figure 8.z-1: Message flow for Sensing Failure Indication

1.	The gNB sends Sensing Failure Indication message to the SF to notify that an ongoing sensing measurement can no longer be performed.

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>