3GPP TSG-RAN WG3 Meeting #130	R3-258882

# Dallas, US, Nov 17th - Nov 21st, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | - | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (BLCR for 38.300) Introduction of Ambient IoT Ph2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ambient_IoT_Solutions_Ph2-Core |  |  |  |  |  | Date: |  |  | 2025-12-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduce the features for Ambient IoT phase 2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | RAN3#130: - Add the description part of A-IoT architecture aspects for topology 2. - Add the message flows of inventory and command procedure for topology 2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The features for Ambient IoT phase 2 will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.23.2, 16.23.6.1, 16.23.6.2, 16.23.7.1, 16.23.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.413 CR 1408 TS 38.423 CR 1664 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Initial version, capture agreed TP R3-258783 and R3-258855. |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 16.23	Support of Ambient IoT

### 16.23.1	General

A-IoT radio interface provides the communication between A-IoT device(s) and A-IoT reader, as illustrated in Figure 16.23.1-1. In this release, the A-IoT reader is a gNB-reader. A-IoT radio interface can support both inventory procedure and command procedure as defined in TS 23.369 [68]. An A-IoT device monitors the R2D message as long as it has sufficient energy.

Figure 16.23.1-1: Architecture supporting the A-IoT radio interface

### 16.23.2	Architecture

The NG-RAN overall architecture as depicted in section 4.1 is applicable for Ambient IoT along the following adaptations and additions:

-	For the sake of supporting the provision of A-IoT services, the involved gNBs are enabled to support A-IoT, specifically, the gNB supports communication with A-IoT devices by means of A-IoT radio. A-IoT radio is either supported by gNB-readers or UE-readers.

-	A gNB may or may not only support communication with A-IoT devices by means of A-IoT radio.

-	A gNB serves one or multiple readers.

-	A gNB is aware of the mapping relationship between A-IoT Areas and readers by means of OAM configuration.

-	An A-IoT Area may include readers belonging to the same or different gNB.

-	One reader only belongs to one gNB.

-	One reader can be mapped to one or multiple A-IoT Areas.

-	As specified in TS 23.369 [68], the A-IoT CN node which the gNB connects to is either the AIOTF (in case of direct connectivity with the AIOTF) or the AMF (in case of indirect connectivity with the AIOTF). A gNB is aware of direct connectivity with the AIOTF via OAM.

NOTE 1:	A deployment with both direct connectivity and indirect connectivity is not expected.

NOTE 2:	A deployment with UE-reader only supports indirect connectivity.

-	In this version of the specification, A-IoT specific information is transported between the gNB and the A-IoT CN node via the NG-C interface, the protocol stack in section 4.3.1.2 applies. In case of indirect connectivity with the AIOTF, A-IoT specific information is transferred transparently via the AMF.

-	In this version of the specification, no A-IoT specific communication takes place between gNBs.

-	In this version of the specification, split gNB architecture is not supported.

The AIOTF may request the gNB to perform an A-IoT operation within a specific area (requested service area). The requested service area may be indicated by means of a list of A-IoT Areas and/or a list of readers. The A-IoT Area is represented by an A-IoT Area ID.

The AIOTF is aware of the area within which the gNB supports provision of A-IoT services by means of OAM configuration. This area may be represented as A-IoT Areas and/or the readers supported by the gNB.

The AIOTF may also be aware of the reader’s location by means of OAM configuration.

The AIOTF is aware of the mapping relationship among gNBs, readers and A-IoT areas by means of OAM configuration, as needed.

The authorization of UE-reader is performed at core network.

<<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>>

### 16.23.6	Inventory Procedure

#### 16.23.6.1	Topology 1

Figure 16.23.6-1 depicts the basic communication between the gNB and the A-IoT CN node for the Inventory procedure for Topology 1.

Figure 16.23.6-1: Inventory procedure for Topology 1

/******* Unchanged part skipped *******/

#### 16.23.6.2	Topology 2

Figure 16.23.6-2 depicts the basic communication between the UE, gNB and the AMF node for the Inventory procedure for Topology 2.

Figure 16.23.6-2: Inventory procedure for topology 2

1.	The gNB receives the Inventory Request message from the AMF.

2.	The gNB allocates A-IoT radio resources for the UE to perform A-IoT operation.

Editor’s Note 1: FFS on the details, depending on RAN2.

3.	A-IoT specific RRC signalling takes place (FFS) between the UE reader and the gNB.

Editor’s Note 2: Further details depending on RAN2.

4.	The gNB sends an Inventory Response message to the AMF.

5.	The Inventory operation over the A-IoT radio interface is performed.

6/7.	The gNB sends the inventory report received from the UE towards the AMF via Inventory Report message(s).

8.	If there is no follow-on command, the gNB will receives the A-IoT Session Release message from AMF as defined in clause 16.23.8.

### 16.23.7	Command Procedure

#### 16.23.7.1	Topology 1

Figure 16.23.7-1 depicts the basic communication between the gNB and the A-IoT CN node for the Command procedure for Topology 1.

Figure 16.23.7-1: Command procedure for Topology 1

/******* Unchanged part skipped *******/

#### 16.23.7.2	Topology 2

Figure 16.23.7-2 depicts the basic communication between the UE, gNB and the AMF node for the Command procedure for Topology 2.

Figure 16.23.7-2: Command procedure for Topology 2

The Command procedure addresses only one A-IoT device.

1.	Prior to the Command procedure, the Inventory procedure is performed involving the concerned device and potentially other A-IoT devices as specified in Figure 16.23.6.2 in step 1 to 8.

2.	The gNB receives the Command Request message from the AMF in order to send a command to a single A-IoT device via the UE.

3.	The gNB allocates A-IoT radio resources to the UE for A-IoT operation.

Editor’s Note 3: Resource allocation depending on RAN1/2.

4.	A-IoT specific RRC signalling takes place (FFS).

Editor’s Note 4: Further depending on RAN2.

5.	The UE performs the Command operation over the A-IoT radio interface.

6.	After receiving RRC Command Response from the UE, the gNB sends Command Response message towards the AMF.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>