3GPP TSG RAN WG3 Meeting #130	R3-258092

Dallas, USA, 17 - 21 November, 2025

3GPP TSG-RAN WG2 Meeting #131bis		R2-2507932

Prague, Czech Republic, Oct. 13th-17th

Title:	Reply LS on the maximum supported AIoT NAS container length

Response to:	C1-255679 from CT1

Release:	Release 19

Work Item:	Ambient_IoT_solutions

Source:	RAN2

To:	CT1

Cc:	RAN3

Contact person:	Jing HAN

hanjing8@lenovo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1	Overall description

RAN2 thanks CT1 for the LS on the maximum supported AIoT NAS container length.

RAN2 further discussed this issue with below analyses for Rel-19 Ambient IoT:

The R2D message containing the command request message is R2D Upper Layer Data Transfer message in MAC layer, and the maximum MAC PDU size is 125 bytes. In the R2D MAC PDU, the MAC header(s) occupies 6 bytes. The maximum size of one R2D NAS container is 125 bytes – 6 bytes = 119 bytes. Note that this message does not support segmentation.

The D2R message containing the command response message is D2R Upper Layer Data Transfer message in MAC layer, and the maximum MAC PDU size is 125 bytes. In the D2R MAC PDU, the MAC header(s) occupies 2 bytes and the maximum data SDU length in the D2R MAC PDU is 125 bytes - 2 bytes = 123 bytes. Considering this message support AS layer segmentation, the supported maximum size of the command response message can be larger than 123 bytes. However, from RAN2 point of view, AS layer is designed to support segmentation to enable transmission of data sizes up to 1000 bits (125 bytes) as per SA1 requirements, and not intend to support the NAS SDU size larger than SA1 requirement.

In conclusion, as to the maximum supported AIoT NAS container length mentioned in the LS, RAN2 understands

- For R2D upper layer data for a given command, the maximum size of one R2D NAS container is 119 bytes

- For the response to a given command in D2R, the maximum size of one D2R NAS container is 125 bytes considering SA1 requirement

The above conclusion is only for Rel-19 Ambient IoT.

2	Actions

To CT1

ACTION: 	RAN2 kindly asks CT1 to take above into account.

3	Dates of next RAN2 meetings

TSG-RAN2 Meeting #132		17 - 21 Nov 2025		Dallas, US

TSG-RAN2 Meeting #133		09 - 13 Feb 2026		Stor-Göteborg, SE