3GPP TSG-RAN WG3 Meeting #81bis	R3-131954

# Venice, Italia, October 7th – 11th, 2013

Agenda item:		10.1.1.1

Source:	ZTE

Title:	Text proposal on solutions for Ping-Pong

Document for:		Discussion and approval

Introduction

At RAN3 #81, the Mobility Settings Change interpretation is described in TP [1]. In this paper, some modifications are added according to the discussions in paper [2].

Text proposal

*** Fist change ***

### 4.1.1	Ping-pong event

Problem description:

Enabling wider differentiation of mobility setting may be needed in the system (homogeneous and heterogeneous scenarios), but may create issues, such as ping-pongs. Example scenarios are presented below (further scenarios are FFS).

Scenario 1:

When load balancing is used to resolve congestion in the source cell, and the Mobility Settings Change procedure is used to adapt the handover trigger point to the target cell, some UE categories may be subject to ping-pong depending on how the UE category is handled in the target cell. A UE belonging to such UE category is handed over from the congested source cell to the target cell while located far out in the edge of the target cell. While the eNB serving the target cell is aware that handing over the UE back to the congested cell within a certain time window is a ping pong event it is FFS whether the eNB serving the target cell needs additional information for further handover decisions. These decisions are typically based on a trade off between the risk for failure and ping pong.

Solutions:

The following solutions have been identified:

- Solution without additional information - The existing information such as load information, measurement configuration, QoS parameters and UE capabilities can be used to assess the offset used for a handover and likelihood of connection failure of the served UE. Therefore, current specifications enable an eNB to have information for avoiding unnecessary handovers back to the source cell.

- Solution with additional information but without pre-defined UE groups - In this solution the source eNB sends an indication in the handover request to the target eNB to give additional information about each handover

  - Signal the offset from the agreed handover trigger used for this handover.

  - Signal a timer to inform the target that it should not hand over the UE back to source within the given time.

  - Signal a group identity (defined at source as a bit string) in the Mobility Setting Change procedure; later, the target, if it accepted the new mobility settings, applies the new settings to the UEs handed over successfully with the same group identity signaled in the HO preparations.

- Solution with pre-defined UE groups - In this solution, the groups are defined in the standard. The mobility settings change procedure is extended to include negotiation of the predefined groups.

  - The eNB exchange the group ID in the handover request

  - The groups are based on commonly known parameters, like UE capabilities or release or bearer class or UE behavior (e.g. UE mobility state as known by the network)

*** Remaining text not changed ***

References

- R3-131548

- R3-131723