3GPP TSG RAN WG3 #86	R3-142780

17.11. to 21.11.2014, San Francisco, USA

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.443 | CR | 0081 | rev | - | Current version: | 11.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for using fake MBMS session on allowing PMCH without any sessions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | MBMS_LTE; TEI11 |  |  |  |  |  |  | Date: |  | 2014-11-16 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-11 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | On the one hand, PMCH carrying MCCH should be transmitted even though no MTCH exsits on it, on the other hand, for backward compatibility, PMCH configuration shall have at least one MTCH. I.e., In TS36.443, it is not allowed to configurate a PMCH without any MTCH. |  |  |  |  |  |  |  |  |
| Summary of change: |  | When all MBMS sessions stop, the PMCH carrying MCCH shall keep the last MBMS session as a fake MBMS session in oder not to delete this PMCH configuration. Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification (same release). This CR has an impact under a functional point of view. The impact can be considered isolated since it only affects PMCH transmission in Uu interface. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PMCH carrying MCCH canot be configured when no MTCH exists on it and this PMCH cannot be transmitted on the Uu interface. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS/TR 36.331 CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

8.4	MBMS Scheduling Information

8.4.1	General

The purpose of the MBMS Scheduling Information Procedure is to provide MCCH related information to the eNB.

The procedure uses non MBMS-Service-associated signalling.

8.4.2	Successful Operation

Figure 8.4.2-1. MBMS Scheduling Information procedure. Successful operation.

The MCE initiates the procedure by sending the MBMS SCHEDULING INFORMATION message to the eNB. For the PMCH carrying MCCH, when all MMBS sessions stop, the MCE shall keep the last MBMS session as a fake MBMS sessionfor this PMCH Configuration. If PMCH configuration information is contained in this message, the eNB shall do PMCH transmission according to the PMCH configuration information. If PMCH configuration information for an existing PMCH is not contained in this message, the eNB should stop PMCH transmission for the PMCH. The eNB shall store the MBSFN Area Configuration Item IE, and apply the MCCH update from the modification period defined in the MCCH Update Time IE. The eNB shall schedule the MBMS services in the MCCH according to the order defined in the MBMS Session List per PMCH IE.

8.4.3	Abnormal Conditions

Void.