3GPP TSG-RAN WG3 Meeting # 93bis	R3-162596

Sophia Antipolis, France, Oct. 10th – 14th, 2016

Agenda item:		10.4.3

Source:	LG Electronics Inc.

Title:	TP for the functions on Xn interface

Document for:		Discussion

1. Text Proposal

----------------Start of the First Change---------------

6.3.3	RAN internal interface

6.3.3.1	Xn Interface

The interface allowing to interconnect two gNBs or one gNB and one eLTE eNB with each other is referred to as the Xn interface.

Editor’s note: The applicability of this interface for a connection between two eLTE eNBs is FFS.

6.3.3.1.1	General principles

The general principles for the specification of the Xn interface are as follows:

-	the Xn interface shall be open;

-	the Xn interface shall support the exchange of signalling information between the endpoints, in addition the interface shall support data forwarding to the respective endpoints;

-	from a logical standpoint, the Xn is a point-to-point interface between the endpoints. A point-to-point logical interface should be feasible even in the absence of a physical direct connection between the endpoints.

-	the Xn interface shall support control plane and user plane separation;

-	the Xn interface shall separate Radio Network Layer and Transport Network Layer;

-	the Xn interface shall be future proof to fulfil different new requirements, support new services and new functions.

6.3.3.1.2	Xn Interface Functions

The Xn-C interface supports the following functions:

-	Xn Interface management and error handling: procedures to manage the Xn-C interface;

- Error indication;

- Setting up the Xn;

- Resetting the Xn;

- Updating the Xn configuration data;

- Xn Removal.

-  UE connected mode mobility management: procedures to manage the UE mobility for connected mode between nodes in the New RAN;

- Handover preparation;

- Handover cancellation.

-	Dual connectivity: procedures to enable usage of additional resources in a secondary node in the New RAN

Xn-U interface supports following functions:

-	Data forwarding

-	Flow control

----------------End of the First Change---------------