3GPP TSG-RAN WG2 #93-bis	R3-162625

Sophia Antipolis, France, 10-14 October 2016

Source: 	Ericsson

Title:	Procedures Analysis for the NG1 Interface

Agenda Item:	10.4.2

Document for:	Discussion and Decision

# Introduction

During RAN3-93bis it was agreed that a TP for TR38.801, capturing procedures for the NG interface, should be presented. In this contribution a TP is provided, which is based on the proposal in R3-162419.

# Text Proposal

----------------------------------------------Start of Changes----------------------------------------------

#### 6.3.2.3	NG Interface Functions

NG-C interface supports following functions:

-	Interface management: procedures to manage the NG-C interface;

-	UE context management	: procedures to manage the UE context between the RAN and CN;

-	UE mobility management: procedures to manage the UE mobility for connected mode between the RAN and CN;

-	Transport of NAS messages: procedures to transfer NAS messages between the CN and UE;

-	Paging: procedures to enable the CN to generate Paging messages sent to the RAN and to allow the RAN to page the UE in RRC_IDLE state;

-	PDU Session Management (FFS): procedures to establish, manage and remove PDU sessions that are made of data flows carrying UP traffic.

#### 6.3.2.4	NG Interface Procedures

To support the functions listed in Section 6.3.2.3 the NG interface should support the following procedures. The procedures are classified in different categories.

Interface management procedures:

- NG Setup: To establish an NG interface between the New RAN and NGC nodes (FFS)

- Configuration Updates from New RAN and NGC nodes: To update configuration of the interface from the New RAN or NGC (FFS)

- NG Reset: To reset the NG interface

- Error Indication: To report detected errors in one incoming message

UE Context Management Procedures:

- Initial Context Setup: To establish the initial UE context both in New RAN and in NGC (FFS)

- UE Context Release: To remove UE context information (FFS)

- UE Context Modification: To modify an already established UE context (FFS)

UE Mobility Management Procedures:

- Handover Preparation: Needed to prepare resource allocation from the source RAN to the NGC for a UE handing over (FFS)

- Handover Resource Allocation: Needed to reserve resources at the target RAN for a UE handing over (FFS)

- Path Switch Request: Needed to request the switch of a UP connection to a different UP GW at the NGC (FFS)

- Handover Cancel: Needed to cancel an ongoing handover preparation or an already prepared handover (FFS)

- Handover Notification: Needed to indicate to the NGC that handover has been successfully completed (FFS)

Transport of NAS Messages Procedures:

- Initial UE Message: To transport the first NAS PDU sent when no UE signaling connection is established from the UE to the NGC (FFS)

- DL NAS Transport: To transport NAS PDUs in DL (FFS)

- UL NAS Transport: To transport NAS PDUs in UL (FFS)

- Non Delivery NAS Indication: To indicate that the RAN node did not deliver a NAS PDU to the UE (FFS)

- NAS Rerouting: In order to route a NAS request to a different CN node (FFS)

Paging Procedures:

- Paging: To provide a paging instruction from the NGC to the New RAN

PDU Session Management Procedures:

- PDU Session Setup: Needed to establish a PDU Session and assign resources to it according to the QoS and other configuration parameters assigned to the flow(s) configured within the session

- PDU Session Modify: Needed to modify a previously configured PDU session (FFS)

- PDU Session Release: Needed to release a previously configured PDU Session

- PDU Session Modification Indication: Needed to switch a PDU Session from one termination point at RAN to a different termination point. (FFS)

----------------------------------------------End of Changes----------------------------------------------