3GPP TSG-RAN3 Meeting #93bis	R3-163027

# Reno, USA, 14 – 18 November 2016

Agenda Item:	30

Source: 		Fujitsu

Title: 	Change in X2 TNL performances use-case and solution

Document for:		Discussion and decision

# Introduction

During RAN#72 meeting, the Study on SON for eCoMP for LTE SID [1] was approved. It was decided that RAN3 will “Identify scenarios and implementation requirements on network side needed for autonomous SON-based CoMP coordination sets generation and update”. Furthermore during RAN3#93bis, the following use-case, from R3-162608 [2], was discussed and introduced in the TR 36.742 [3]:

Use-case description: Change in X2 backhaul caracteristics due to load

- This use-case is triggered by the CoMP controlling unit (CCU) (centralized case) or involved eNBs (distributed case).

- The purpose of this use-case is to maintain maximum average and cell edge User Packet Throughput gain following changes in X2 capacity available for inter-eNB CoMP.

In this contribution, we describe further the need of the above use-case for eCoMP operations, taking into account discussion during RAN3#93 and RAN3#93bis, and propose a solution.

# Discussion

## Use-case description

In [4] we discussed the fact that X2 performances (i.e. bandwidth and latency) are crucial for CoMP operations and are variable in time. Concerning latency, TR 36.874 [5] concluded that non-ideal backhaul latency has a strong impact on overall CoMP gain in inter-eNB scenarios. For the bandwidth we need to look at the signaling load introduced by the exchange of CoMP information between eNBs.

The signaling needed to implement CoMP schemes has been defined during Rel-12 and Rel-13 WIs. The following reporting information were added to X2AP LOAD INFORMATION message:

- CoMP Information, comprising (non-exhaustive):

  - Up to 256 CoMP Information Items, each comprising:

    - 1 Benefit Metric

    - Up to 32 CoMP Hypothesis Set Items

- Enhanced RNTP

Furthermore the following reporting information were added to Resource Status Reporting Initiation procedure:

- RSRP Measurement Report List, comprising (non-exhaustive):

  - Up to 128 RSRP Measurement Report Items, each comprising:

    - Up to 9 RSRP Measurement Results

- CSI Report, comprising (non-exhaustive):

  - Up to 128 CSI Report per Cell, each comprising:

    - Up to 4 CSI Report per CSI process, each comprising:

      - Up to 2 CSI Report per CSI Process Items, each comprising,

        - Up to 14 Subband CQI Lists

These information need to be signaled periodically over X2 in order to bring a significant gain. The periodicity for CoMP hypothesis was not defined in the specifications but can be as low as few ms. The periodicity for RSRP Measurement Report can be as low as 120ms. And the periodicity for CSI Report can be as low as 5ms. Furthermore the signaling load also depends on the number of UEs involved, which means that the load on X2 should be carefully controlled in order to maximize the number of UEs benefiting from CoMP, without impacting the transport network and the others UEs. If one of the X2 link between the cooperating nodes is overloaded, this information exchange will not be optimal and can impact the overall performances, and not only CoMP operations.

Therefore an adequate choice of the CoMP cooperating set, taking into account actual X2 performances would be beneficial.

Proposal 1: RAN3 is kindly asked to agree the corresponding modification of the use-case description in the TP provided in section 5

## Possible solution

As discussed in [4], X2 backhaul characteristics are dynamic and therefore need to be signalled over X2. This exchange of information can be performed in two different ways:

- On a periodic basis (e.g. every seconds)

- Event triggered (e.g. significant change in X2 TNL load or X2 latency)

In the case of a centralized CoMP scheme, this exchange of information will be triggered by the CCU, for each candidate eNB which may be configured in the CoMP cooperating set. In the case of a distributed CoMP scheme, this exchange of information will be triggered by any eNB which may be configured in the CoMP cooperating set. As the X2 backhaul characteristics can be different for each way (e.g. from eNB1 to eNB2 or from eNB2 to eNB1), the exchange of information should be bidirectional.

Observation 1: The changes in X2 backhaul characteristics are dynamic and need to be signaled over X2

Proposal 2: RAN3 is kindly asked to agree the corresponding solution in the TP provided in section 5

# Conclusion and proposals

In this contribution we discussed the importance of X2 performances for CoMP operations. Therefore we propose to improve the existing use-case description in the TR.

Proposal 1: RAN3 is kindly asked to agree the corresponding modification of the use-case description in the TP provided in section 5

Furthermore we propose that X2 characteristics (bandwidth and latency) should be signaled via X2.

Proposal 2: RAN3 is kindly asked to agree the corresponding solution in the TP provided in section 5

# References

[1]	RP-161181, New SI: Study on SON for eCoMP for LTE

[2] R3-162608, SON for eCoMP use-cases, Nokia, Alcatel-Lucent Shanghai Bell

[3] TR 36.742, Study on SON for eCoMP

[4] R3-162473, Use cases for TR 36.742 - SON for eCoMP, Fujitsu

[5] TR 36.874, Coordinated multi-point operation for LTE with non-ideal backhaul

# Text Proposal for TR 36.742

----------------------------------------------Start of 1st Changes----------------------------------------------

### 5.2.1	Use-case description: Monitoring X2 backhaul characteristics

This use-case is triggered by the CoMP controlling unit (CCU) (centralized case) or involved eNBs (distributed case).

The purpose of this use-case is to maintain maximum average and cell edge User Packet Throughput gain following changes in X2 capacity available for inter-eNB CoMP.

Considering the high level of cooperation and the signalling load between nodes needed for CoMP operation, X2 performances (i.e. X2 latency and X2 bandwidth) have an important impact on CoMP operations. Furthermore these performances are variable in time. For example different X2 transport routes can change the latency. And X2 bandwidth depends on the overall load of the backhaul network, but also on UEs connectivity and traffic (e.g. Dual Connectivity). Therefore an adequate choice of the CoMP cooperating set, taking into account actual X2 performances would be beneficial.

----------------------------------------------End of 1st Changes----------------------------------------------

- 6	Potential solutions

----------------------------------------------Start of 2nd Changes----------------------------------------------

- 6.1	Solution for monitoring X2 backhaul characteristics

- 6.1.1	Solution description

X2 backhaul characteristics are dynamic and therefore need to be signalled over X2. This exchange of information can be performed in two different ways:

- On a periodic basis (e.g. every seconds)

- Event triggered (e.g. significant change in X2 TNL load or X2 latency)

In the case of a centralized CoMP scheme, this exchange of information will be triggered by the CCU, for each candidate eNB which may be configured in the CoMP cooperating set. In the case of a distributed CoMP scheme, this exchange of information will be triggered by any eNB which may be configured in the CoMP cooperating set. As the X2 backhaul characteristics can be different for each way (e.g. from eNB1 to eNB2 or from eNB2 to eNB1), the exchange of information should be bidirectional.

----------------------------------------------End of 2nd Changes----------------------------------------------