3GPP TSG-RAN WG3 Meeting #95bis	R3-171177

Spokane, USA, 3rd – 7th April 2017

Agenda Item:	31.1

Source:	Ericsson

Title:	QoE measurement failure handling

Document for:	Discussions & Approval

# 1	Background

At RAN3#95 meeting the work item QoE [1] was completed. The UE Application Layer measurements at the moment consists of QoE measurement for streaming service. The measurement configuration is encapsulated in a DASH file and the size could reach 6000 b before compression.

This paper discusses an enhancement to handle the failure case during the UE Application Layer measurement initiation.

# 2	Discussion

The UE Application Layer measurements may be initiated towards RAN directly from an O&M node in generic way for a group of UEs. They may also be initiated towards RAN by receiving a "trace request" from the core network. The configuration of the measurement includes the measurement details, which is encapsulated in a container that is transparent to RAN. The configuration information typically comprises a DASH file specifying the data to be collected. The RAN then starts the measurements in the UE by sending the configuration information transparently to the UE.

The RRC protocol is used to start the measurements in the UE. The configuration information is carried in a MEASUREMENT CONTROL message in the form of a transparent container. If the configuration fails, a cause value is sent back in the MEASUREMENT CONTROL FAILURE message. RAN2 will discuss if to add more cause values for UE Application Layer measurement configuration failure, [3].

However there is no way for RAN to feedback the failure to the measurement initiating entity. To improve the QoE feature and to avoid CN may send the big sized faulty UE application layer measurement configuration further, we propose to introduce a procedure for failure reporting.

Proposal 1: RAN3 to agree to introduce the failure reporting for the UE Application Layer measurements.

CR is submitted in [2].

# 3	Proposals

Proposal 1: RAN3 to agree to introduce the failure reporting for the UE Application Layer measurements.

# 4	Reference

- [1] 	RP-161917	New WI proposal: Quality of Experience (QoE) Measurement Collection for streaming services in UTRAN, China Unicom

- [2] 	R3-171178	UE Application Layer Measurement Failure Handling, Ericsson