3GPP TSG-RAN WG3 #96							R3-171693

Hangzhou, P. R. China, 15th – 19th May 2017

Agenda Item:	10.10.2

Source: 	Vodafone

Title: 	Many to Many Connections of CUs and DUs

Document for:	Discussion and Decision

# Introduction

At the previous RAN3 #95bis meeting in Spokane, Option 2 for the High Layer Functional Split was selected and RAN3 decided to adopt a separated control and user plane for the new RAN architecture.

The separated control and user planes and separation of Control and User Function Entities [1] allows us to evolve the network in various different configurations to suit a number of applications and to cope with transmission network delays.

In this contribution, we provide a number of architectural and deployment scenarios and illustrate how the new evolved architecture could provide mobility, resistance and network flexibility.

This leads to the requirement that there is a many to many relationship between CUs and DUs, and, implicitly that the current definition of a gNode B is revisited.

# Network Resilience and Load Sharing

The following figure illustrates a practical solution of many Distributed Units (DU) or base-stations connected to two User Plane Centralised Units.

For example, these two PDCP-U units could be serving a large city such as London. During the working day the PDCP-U load is mostly associated with the DUs in the centre of London while in the evening the PDCP-U load is mostly associated with the DUs in the suburbs.

Please note that only User Plane connections are illustrated and control plane connections are omitted.

Figure 1 Multiple DUs to Multiple CUs Connections (only User Plane Connections are shown)

In figure illustrate that every DU unit has at least two connections to CU1 and CU2. These connections can be designed to provide:

- Load Balancing and load sharing between CUs, to cope with massive movement of load/traffic between DUs during daily commuting cycles

- Resilience, in case of failover of one of the CUs

The architecture illustrates that User data traffic can be hosted on two or more CUs where the PDCP-U is routed from.

Mobility between the DUs are facilitated by Xn connection between them.

Observation 1: many DUs should be able to connect to multiple CUs to provide load balancing, load sharing and resilience.

Observation 2: it does not seem sensible to describe all of (thousands of) DUs in London plus a small number of massive PDCP-U units in London as “one gNodeB”. A term such as NG-RAN is much more suitable!

# Flexible Network

Having the flexibility of separated Control and User Plane Functional Entities, gives us the flexibility to deploy the network in a number of ways. This flexibility allows us to:

- Have a simple and elegant network design by using GTP-U for User plane traffic and SCTP for the Control/signalling traffic and

- Place User Plane Functional Entities in data centres which are serviced in a cost-effective way, thus saving OPEX (Note that this does not increase U plane latency – it simply swaps NG-U latency to F1-U latency)

- Mitigate backhaul transmission delays, latencies and fluctuations by placing control entities in aggregation points closer to the DUs.

- Place the Control Functional Entitles close to the DU (base-stations) for applications that require ultra-fast signalling response

See the following Scenarios and figures:

## Scenario 1

The following figure illustrates a conventional deployment scenario, of Option 2-2, where the latencies between the DUs and the CU, is less than or approximately 8ms. In this scenario CU houses the Control Plane and User Plane Entities. The Control Plane Entity manages the RRC and PDCP-C and the User Plane Entity manages the PDCP-U.

Figure 2 Architecture Scenario1

## Scenario 2

The following figure illustrates a slightly different deployment approach: in this scenario the latencies in the transmission network are greater than 8ms, for example DUs are considerably further apart from CUs, and Control Entities are placed closer to the DUs.

Figure 3 Architecture  Scenario 2

## Scenario 3

In this scenario, irrespective of the transmission latencies and delays, the Control Entities are placed in the DUs. See the following figure. This deployment allows Ultra-fast signalling response between the UE and DU which are suitable for application such as Mobile Edge Computing etc.

This scenario also allows the RRM entity to be very “radio aware” as it has access to all the radio information within the DU.

Separately this scenario allows an eNB collocated with NR functionality to use the existing eNB RRM entity to remotely control the PDCP-U used for the fast NR/LTE “dual connection”.

Figure 4 Architecture Scenario 3

## Scenario 4

For applications which require Ultra Low User Plane Latencies, such as machine-to-machine type applications in a factory environment, Scenario 4, shown in the following figure illustrates a practical solution. In Scenario 4, The Control Plane Entity and Local RRM are placed in the DU. The S/P Gateway which houses the factory-based applications is also located in the DU, with links to the local factory Intranet.

UEs or devices that require Ultra Low Latency connections connect to the Local S/P Gateway in the DU and UEs that require conventional internet connectivity connect to the S/P Gateway in the operator’s environment.  See the following figure:

Figure 5 Architecture Scenario 4

# Scenario 5

In this scenario a Dual Connectivity [2] scenario is used to illustrate the flexibility of the network design. In this configuration, UE1 and UE 2 are (respectively) connected to CU1 and CU2. In this scenario the Control Entities are located at the DUs. See the following figure:

Figure 6 Scenario 5 Dual Connectivity Architecture

For dual connectivity of UE2, RRM2 instructs DU1 to connect to the PDCP-U unit located at CU2. Independently, for UE 1, RRM1 selects to connect to CU1.  Hence DU1 needs to connect to multiple CUs, simultaneously.

Observation 3: Dual Connectivity requires that a DU can connect to multiple User Plane CUs.

Observation 4: (as the RRM/RRC/PDPC-C is the “CU-control plane”) Dual Connectivity requires that one DU can connect to multiple Control Plane CUs.

# New Interface

In all Scenarios 1-4 shown, with the separation of the Control Plane and User Plane Entities, it can be observed that an interface exists between the two entities. This interface is very similar to that used by Release 12 Dual Connectivity control plane, but currently is not named. Here we give it the name “E1” as the working name for this interface.

Figure 7 New Interface between Control and User Entities termed  E1

Observation: to progress the work on the new RAN Architecture, the interface between the Control Plane Entity and the User Plane Entity should be specified and be named E1.

# Conclusion

The separation of the Control and User Planes and the consequently the separation of Control and User Plane PDCP Entities has opened a new set of architectural possibilities for mobile operators. Scenario 1-5 presented in this contribution illustrates the flexibility of the new RAN architecture for a number of applications.

Furthermore, we have illustrated that for resilience, load sharing, load balancing and for dual connectivity, there needs to be a many to many relationship between DUs, User plane CUs, and Control Plane Entities.

The separated Control and User Plane Entities need a standardised interface between them which is very similar to the existing Release 12 Dual connectivity Control Plane between MCG and SCG. We have used E1 as the name for this interface.

We kindly ask RAN3 to agree the following proposals:

Proposal 1: There shall be a many to many relationship between DUs and User Plane CUs

Proposal 2: There shall be a many to many relationship between DUs and Control Plane CUs

Proposal 3: There shall be a many to many relationship between Control Plane CUs and User Plane CUs

Proposal 4: The interface between the Control Plane and the User Plane Entities to be named E1.

Proposal 5: The interface between the Control Plane and the User Plane Entities shall be fully standardised.

Proposal 6: The conclusions on this TDoc are used to rework the definitions of “what is a gNB”.

# References

[1] R3-171203

[2] 3GPP TR 36.842 V12.0.0