3GPP TSG-RAN WG3 #96	Tdoc R3-171713

Hangzhou, China, 15th – 19th May 2017

Agenda Item:	10.02

Source:	Ericsson

Title:	NW slicing support at NG-C establishment and update

Document for:	Discussion, Decision

# Introduction

During recent meetings, RAN3 agreed on the overall principles for the support of network slicing at RAN level, as documented in [1]. In this contribution we detail further the need to introduce slice awareness already when the NG-C interface is established and make some proposals for updates of [1] and [2].

# Discussion

## Motivation

During the study phase, RAN3 discussed at length the need for the RAN to be slice aware and agreed that only by being made aware of the relevant network slices, can RAN applied the isolation and differentiated handling upon which the whole concept of NW slicing is based.

This is already documented in [1] where it is also highlighted that such awareness may be provided by the 5G CN:

*******

’Selection of RAN part of the network slice

-	NG RAN supports the selection of the RAN part of the network slice, by assistance information provided by the UE or the 5GC which unambiguously identifies one or more of the pre-configured network slices in the PLMN.’

*******

And more, it was established during the discussions on the provision and availability of slice selection assistance information over the radio interface, that the earlier the RAN becomes aware of relevant network slices it has or may have to handle, the better.

Putting the above facts together, we realize the following:

- It was already established during the study phase that RAN must be slice aware and that it is beneficial if this awareness is achieved as early as possible.

When it comes to providing slice awareness already during NG Setup/Configuration Update, it was however claimed that it may not be necessary.

But if we go back again to the study phase discussions, we also note that it is an established fact that the way isolation and differentiated handling is realized is up to network implementation:

***

‘How NG RAN supports resource isolation is implementation dependent.’

***

So it appears a bit difficult to understand how it can be claimed at this point that such information is not necessary, given it seems in fact quite clear that if the RAN and the 5G CN are made aware of which network slices are available already when the NG association is established, then such information can be considered early.

For example, there would be awareness in RAN of which groups network slices could in theory be served by the same AMF, which can be useful during CN instance selection. In the same way, it may be beneficial for the AMF to know already at NG setup (and then as soon as any configuration changes) if certain slices are not available in a gNB.

If a slice is not available anymore (for any reason), this could also be communicated, meaning this information would not indicate what is ‘supported’, rather what is ‘available’ or ‘applicable’.

Naturally, OAM could configure this information, if it is not provided in NGAP, but configuration via OAM may be quite tedious comparing to embedding this information in NGAP.

- Inclusion of slicing information already during NG setup appears useful as it can enable the RAN to be aware of which combinations of NW slices the AMF in question may serve and can therefore adapt its handling. In the same way AMF insight into slice availability in RAN may ease mapping of UEs to slices.

- If this information is not provided via NG Setup, it may anyway have to be configured/provisioned by OAM.

# Conclusion

In this contribution we have examined and discussed the motivations for making RAN slice aware already at NG Setup.

The following observations were made:

- It was already established during the study phase that RAN must be slice aware and that it is beneficial if this awareness is achieved as early as possible.

- Inclusion of slicing information already during NG setup appears useful as it can enable the RAN to be aware of which combinations of NW slices the AMF in question may serve and can therefore adapt its handling. In the same way AMF insight into slice availability in RAN may ease mapping of UEs to slices.

- If this information is not provided via NG Setup/Configuration Update, it may anyway have to be configured/provisioned by OAM.

Based on the observations above, we propose the following:

- Proposal 1 	It is proposed to confirm the need to include slice information (in the form of lists of S-NSSAIs) in NG Setup/Configuration Update signalling and agree to the text proposals in [4][5].

# References

3GPP TS 38.300 ‘NG Radio Access Network; Overall Description; Stage 2‘, May 2017.

3GPP TS 38.413 ‘NG RAN – NG Application Protocol (NGAP)’, May 2017.

R3-171714, ‘TP for NW slicing support at NG-C establishment and update in TS 38.300’, Ericsson, May 2017.

R3-171715, ‘TP for NW slicing support at NG-C establishment and update in TS 38.413’, Ericsson, May 2017.

# Annex A: TP for 38.413

*************** START OF MODIFIED SECTION***********

### 8.7.1	NG Setup

#### 8.7.1.1	General

The purpose of the NG Setup procedure is to exchange application level data needed for the gNB and the AMF to correctly interoperate on the NG-C interface. This procedure shall be the first NGAP procedure triggered after the TNL association has become operational. The procedure uses non-UE associated signalling.

Editor’s Note:	Further details are FFS.

#### 8.7.1.2	Successful Operation

Figure 8.7.1.2-1: NG setup: successful operation

The gNB initiates the procedure by sending an NG SETUP REQUEST message including the appropriate data to the AMF. The AMF responds with an NG SETUP RESPONSE message including the appropriate data.

If the List of Applicable S-NSSAIs IE is included in the NG SETUP REQUEST message, the AMF shall, if supported, consider this information in the realization and handling of network slicing.

If the List of Applicable S-NSSAIs IE is included in the NG SETUP RESPONSE message, the gNB shall, if supported, consider this information in the realization and handling of network slicing.

Editor’s Note:	Further details are FFS.

#### 8.7.1.3	Unsuccessful Operation

Figure 8.7.1.3-1: NG setup: unsuccessful operation

If the AMF cannot accept the setup, it should respond with an NG SETUP FAILURE message and appropriate cause value.

Editor’s Note:	Further details are FFS.

#### 8.7.1.4	Abnormal Conditions

Editor’s Note:	Further details are FFS.

### 8.7.2	gNB Configuration Update

#### 8.7.2.1	General

The purpose of the gNB Configuration Update procedure is to update application level configuration data needed for the gNB and the AMF to interoperate correctly on the NG-C interface. This procedure does not affect existing UE-related contexts, if any.

Editor’s Note:	Further details are FFS.

#### 8.7.2.2	Successful Operation

Figure 8.7.2.2-1: gNB configuration update: successful operation

The gNB initiates the procedure by sending a GNB CONFIGURATION UPDATE message to the AMF including an appropriate set of updated configuration data that it has just taken into operational use. The AMF responds with a GNB CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data.

If the List of Applicable S-NSSAIs IE is included in the GNB CONFIGURATION UPDATE message, the AMF shall, if supported, consider this information in the realization and handling of network slicing.

If the List of Applicable S-NSSAIs IE is included in the GNB CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB shall, if supported, consider this information in the realization and handling of network slicing.

***************END OF MODIFIED SECTION***************

*************** START OF MODIFIED SECTION***********

#### 9.1.8.1	NG SETUP REQUEST

Editor’s Note:	Message structure and IEs need further checking and completion. Further details FFS.

This message is sent by the gNB to transfer application layer information for an NG-C interface instance.

Direction: gNB  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | <ref> |  | YES | reject |
| Global gNB ID | M |  | <ref> | eNote: definition of Global gNB ID is FFS. It may relate to identification of cells hosted by the gNB (FFS). | YES | reject |
| gNB Name | O |  | PrintableString (SIZE(1..150,…)) | FFS | YES | ignore |
| Supported TAs |  | 1..<maxnoofTACs> |  | Supported TAs in the gNB. eNote: TAs applicable for 5G as well | GLOBAL | reject |
| >TAC | M |  | <ref> | Broadcast TAC | - |  |
| >Broadcast PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs | - |  |
| >>PLMN Identity | M |  | <ref> |  |  |  |
| Default Paging DRX | M |  | <ref> | eNote: assumption that this is common for NR and E-UTRA. (to be verified with RAN2 and is FFS) | YES | ignore |
| List of Applicable S-NSSAIs |  | 0..<maxnoofSlicesonNG> |  | Indicates the network slices applicable for the NG association | GLOBAL | ignore |
| >S-NSSAI | M |  | <ref> | NW slice identifier (FFS) | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSlicesonNG | Max number of NW slices per NG association. Value is FFS |

#### 9.1.8.2	NG SETUP RESPONSE

Editor’s Note:	Message structure and IEs need further checking and completion. Further details FFS.

This message is sent by the AMF to transfer application layer information for an NG-C interface instance.

Direction: AMF  gNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | <ref> |  | YES | reject |
| AMF Name | O |  | PrintableString (SIZE(1..150,…)) | FFS | YES | ignore |
| AMF pool and AMF code related information | M |  | <ref> | eNote: Will be similar to EPS, but details are pending discussions in SA2 (FFS) | GLOBAL | reject |
| Relative AMF Capacity | M |  | <ref> | eNote: Load balancing concepts are pending discussions in SA2 (FFS) | YES | ignore |
| List of Applicable S-NSSAIs |  | 0..<maxnoofSlicesonNG> |  | Indicates the network slices applicable for the NG association | GLOBAL | ignore |
| >S-NSSAI | M |  | <ref> | NW slice identifier (FFS) | - |  |
| Criticality Diagnostics | O |  | <ref> |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofSlicesonNG | Max number of NW slices per NG association. Value is FFS |

#### 9.1.8.3	NG SETUP FAILURE

Editor’s Note:	Further details FFS.

#### 9.1.8.4	GNB CONFIGURATION UPDATE

Editor’s Note:	Further details FFS. The List of Applicable S-NSSAIs IE shall be included with the same definition as for the NG Setup procedure, once the message is defined.

#### 9.1.8.5	GNB CONFIGURATION UPDATE ACKNOWLEDGE

Editor’s Note:	Further details FFS. The List of Applicable S-NSSAIs IE shall be included with the same definition as for the NG Setup procedure, once the message is defined.

#### 9.1.8.6	GNB CONFIGURATION UPDATE FAILURE

Editor’s Note:	Further details FFS.

***************END OF MODIFIED SECTION***************

# Annex B: TP for 38.300

*************** START OF MODIFIED SECTION***********

# X     RAN support of Network Slicing

## X.1	General Principles and Requirements

In this sub clause, the general principles and requirements related to the realization of network slicing in the NG RAN are given.

Network Slicing is a concept to allow differentiated treatment depending on each customer requirements. With slicing, it is possible for Mobile Network Operators (MNO) to consider customers as belonging to different tenant types with each having different service requirements that govern in terms of what slice types each tenant is eligible to use based on Service Level Agreement (SLA) and subscriptions.

NSSAI (Network Slice Selection Assistance Information) includes one or more S-NSSAIs (Single NSSAI). Each network slice is uniquely identified by a S-NSSAI, as defined in TR 23.799 [X].

The following key principles apply for support of Network Slicing in NG RAN:

RAN awareness of slices

-	NG RAN supports a differentiated handling of traffic for different network slices which have been pre-configured. How NG RAN supports the slice enabling in terms of NG RAN functions (i.e. the set of network functions that comprise each slice) is implementation dependent.

Selection of RAN part of the network slice

-	NG RAN supports the selection of the RAN part of the network slice, by assistance information provided by the UE or the 5GC which unambiguously identifies one or more of the pre-configured network slices in the PLMN.

Resource management between slices

-	NG RAN supports policy enforcement between slices as per service level agreements. It should be possible for a single NG RAN node to support multiple slices. The NG RAN should be free to apply the best RRM policy for the SLA in place to each supported slice.

Support of QoS

-	NG RAN supports QoS differentiation within a slice.

RAN selection of CN entity

-	For initial attach, the UE may provide assistance information to support the selection of an AMF. If available, NG RAN uses this information for routing the initial NAS to an AMF. If the UE does not provide any such information the NG RAN sends the NAS signalling to a default AMF.

-	For subsequent accesses, the UE provides a Temp ID, which is assigned to the UE by the 5GC, to enable the NG RAN to route the NAS message to the appropriate AMF as long as the Temp ID is valid (NG RAN is aware of and can reach the AMF which is associated with the Temp ID). Otherwise, the methods for initial attach applies.

Resource isolation between slices

-	the NG RAN supports resource isolation between slices. NG RAN resource isolation may be achieved by means of RRM policies and protection mechanisms that should avoid that shortage of shared resources in one slice breaks the service level agreement for another slice. It should be possible to fully dedicate NG RAN resources to a certain slice. How NG RAN supports resource isolation is implementation dependent.

Slice Availability

-	Some slices may be available only in part of the network. Awareness in the NG RAN of the slices supported in the cells of its neighbours may be beneficial for inter-frequency mobility in connected mode. It is assumed that the slice configuration does not change within the UE’s registration area.

-	The NG RAN and the 5GC are responsible to handle a service request for a slice that may or may not be available in a given area. Admission or rejection of access to a slice may depend by factors such as support for the slice, availability of resources, support of the requested service by other slices.

Support for UE associating with multiple network slices simultaneously

-	In case a UE is associated with multiple slices simultaneously, only one signalling connection is maintained.

Granularity of slice awareness

-	Slice awareness in NG RAN is introduced at PDU session level, by indicating the S-NSSAI corresponding to the PDU Session, in all signalling containing PDU session resource information.

Validation of the UE rights to access a network slice

- It is the responsibility of the 5GC to validate that the UE has the rights to access a network slice.  Prior to receiving the Initial Context Setup Request message, the NG RAN may be allowed to apply some provisional/local policies, based on awareness of which slice the UE is requesting access to. During the initial context setup, the NG RAN is informed for all network slices for which resources are being requested.

## X.2    CN Instance and NW Slice Selection

### X.2.1	   CN-RAN interaction and internal RAN aspects

NG RAN selects AMF based on a Temp ID or assistance information provided by the UE over RRC. The mechanisms used in the RRC protocol are described in the next sub clause.

Table X.2.1-1 AMF selection based on Temp ID and assistance information

| Temp ID | Assistance Info | AMF selection by NG RAN |
| --- | --- | --- |
| NA or invalid | NA | Default AMF is selected |
| NA or invalid | Present | Selects AMF which supports UE requested slices |
| Valid | NA or present | Selects AMF per CN identity information in Temp ID |

### X.2.2	   Radio Interface Aspects

FFS (RAN2)

## X.3    Resource Isolation and Management

Resource isolation enables specialized customization and avoids one slice affecting another slice.

Hardware/software resource isolation is up to implementation. Each slice may be assigned with either shared or dedicated radio resource up to RRM implementation and SLA.

To enable differentiated handling of traffic for network slices with different SLA:

-	NG RAN is configured with a set of different configurations for different network slices;

-	To select the appropriate configuration for the traffic for each network slice, NG RAN receives relevant information indicating which of the configurations applies for this specific network slice.

## X.4    Signalling Aspects

[Editor’s note: It is FFS whether signalling procedures will be kept in this section or moved to other sections describing procedures]

### X.4.1	   General

In this sub clause, signalling flows related to the realization of network slicing in the NG RAN are given.

### X.4.1	   CN Instance and NW Slice Selection

FFS

### X.4.2	   UE Context Handling

Following the initial access, the establishment of the RRC connection and the selection of the correct AMF, the AMF establishes the complete UE context by sending the Initial Context Setup Request message to the NG RAN over NG-C.

The message contains the S-NSSAI as part of the PDU session/s resource description.

Upon successful establishment of the UE context and allocation of PDU resources to the relevant NW slice/s, the NG RAN responds with the Initial Context Setup Response message.

Figure X.4.2-1: Network Slice-aware Initial Context Setup

### X.4.3	   PDU Session Handling

When new PDU sessions need to be established or existing ones modified or released, the 5GC requests the NG RAN to allocate/release resources relative to the relevant PDU sessions by means of the PDU Session Setup/Modify/Release procedures over NG-C. In case of network slicing, S-NSSAI information is added per PDU session, so NG RAN is enabled to apply policies at PDU session level according to the SLA represented by the network slice, while still being able to apply (for example) differentiated QoS within the slice.

NG RAN confirms the establishment/modification/release of a PDU session associated to a certain NW slice by responding with the PDU Session Setup/Modify/Release Response message over the NG-C interface.

Figure 8.8.3-1: Network Slice-aware PDU Session Setup/Modify/Release

### X.4.4	   Mobility

To make mobility slice-aware in case of Network Slicing, S-NSSAI is introduced as part of the PDU session information that is transferred during mobility signalling. This enables slice-aware admission and congestion control.

### X.4.4.1 Connected Mode Mobility

An example for the case of active mode mobility across different Registration Areas, is shown in Figure X.4.2.1-1 for the case of 5GC involved handover.

Figure X.4.4.1-1: Active mode CN involved mobility across different Registration Areas

Change from R3-171395 (for clause 16.3 Network Slicing)

## X.4    Signalling Aspects

### X.4.1	   General

In this sub clause, signalling flows related to the realization of network slicing in the RAN are given.

### X.4.1	   CN Instance and NW Slice Selection

RAN selects AMF based on a Temp ID or assistance information provided by the UE over RRC.

Figure X.4.1-1: AMF instance selection

In case a Temp ID is not available, the RAN uses the information provided by the UE during RRC connection establishment to select the appropriate AMF instance (details are FFS and subject to RAN2 standardization). If such information is also not available, the RAN routes the UE to a default AMF instance.

***************END OF MODIFIED SECTION***************