3GPP TSG-RAN WG3 #96	Tdoc R3-171716

Hangzhou, China, 15th – 19th May 2017

Agenda Item:	10.02

Source:	Ericsson

Title:	NW slicing support during PDU session resource management

Document for:	Discussion, Decision

# Introduction

During recent meetings, RAN3 agreed on how to introduce network slice awareness at RAN level with regards to PDU session resource management signalling. In this contribution we recap the recent agreements and propose more details for [1].

# Discussion

## Current Assumptions

As already documented in [2], it was agreed that slice awareness, in the form of S-NSSAI, should be introduced as part of each PDU session resource information, to be included where PDU session information is transferred on network interfaces.

- There is an agreement to introduce S-NSSAI per PDU session as part of the PDU session management procedures defined in NG-RAN specifications.

The above implies that S-NSSAI should be introduced in at least the NG procedures:

- Initial Context Setup;

- PDU Session Resource Setup.

Implying the presence of S-NSSAI in at least the following IE:

- PDU Session Resource Setup List;

Other procedures and IEs are FFS.

# Conclusion

In this contribution we have examined and discussed the need to add S-NSSAI at least in signalling carrying the information to establish resources to support a PDU session, therefore it is proposed:

- Proposal 1 	Agree that S-NSSAI should be added to Initial Context Setup and PDU Session Resource Setup, as part of the PDU Session Resource Setup List or equivalent information (depending how this IE is defined at the end);

- Proposal 2 	Agree to the text proposal in [3].

# References

3GPP TS 38.413 ‘NG RAN – NG Application Protocol (NGAP)’, May 2017.

3GPP TS 38.300 ‘NG Radio Access Network; Overall Description; Stage 2‘, May 2017.

R3-171717, ‘TP for NW slicing support during PDU session resource management’, Ericsson, May 2017.

# Annex A: TP to TS 38.413

*************** START OF MODIFIED SECTION***********

### 8.2.1	PDU Session Resource Setup

#### 8.2.1.1	General

The purpose of the PDU Session Resource Setup procedure is to assign resources on Uu and NG-U for one or several PDU Session Resources and the corresponding QoS flows, and to setup corresponding Data Radio Bearers for a given UE. The procedure uses UE-associated signalling.

Editor’s Note:	Further details are FFS.

#### 8.2.1.2	Successful Operation

Figure 8.2.1.2-1: PDU session resource setup: successful operation

The AMF initiates the procedure by sending a PDU SESSION RESOURCE SETUP REQUEST message to the gNB.

The PDU SESSION RESOURCE REQUEST message shall contain the information required by the gNB to setup PDU session related NG-RAN configuration consisting of at least one PDU Session Resource for each PDU Session Resource to setup included in PDU Session Resource To Be Setup Item IE.

If the S-NSSAI IE is included in the PDU Session Resource To Be Setup Item IE, the gNB shall, if supported, use this information for network slicing operation for the concerned PDU session.

Editor’s Note:	Further details are FFS.

#### 8.2.1.3	Unsuccessful Operation

Editor’s Note:	Further details are FFS.

#### 8.2.1.4	Abnormal Conditions

Editor’s Note:	Further details are FFS.

***************END OF MODIFIED SECTION***************

*************** START OF MODIFIED SECTION***********

### 8.3.1	Initial Context Setup

#### 8.3.1.1	General

The purpose of the Initial Context Setup procedure is to procedure is to establish the necessary overall initial UE Context including PDU Session resources. The procedure uses UE-associated signalling.

#### .3.1.2	Successful Operation

Figure 8.3.1.2-1: Initial Context Setup: successful operation

The AMF initiates the procedure by sending an INITIAL CONTEXT SETUP REQUEST message to the gNB.

The INITIAL CONTEXT SETUP REQUEST message shall contain within the PDU Session Resource To Be Setup Item IE the information required by the gNB to build the PDU Session resource configuration associated to least one PDU session.

If the S-NSSAI IE is included in the PDU Session Resource To Be Setup Item IE, the gNB shall, if supported, use this information for network slicing operation for the concerned PDU session.

After sending the INITIAL CONTEXT SETUP RESPONSE message, the procedure is terminated in the gNB.

Editor’s Note:	Further details are FFS.

***************END OF MODIFIED SECTION***************

*************** START OF MODIFIED SECTION***********

#### 9.2.2.3	PDU Session Resource Setup List

Editor’s Note: Most probably this IE will be incorporated directly into the message structure and not defined as a separate IE. In principle, it is applicable for NGAP messages for PDU Session Resource Control. Further details FFS.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Resource To Be Setup List |  | 1 |  |  | YES | reject |
| >PDU Session Resource To Be Setup Item IEs |  | 1 .. <maxnoof PDUSessionResources> |  |  | EACH | reject |
| >>PDU Session ID (FFS) | M |  | <ref> |  | - |  |
| >>S-NSSAI (FFS) | O |  | <ref> |  | - |  |
| >>PDU Session Resource Aggregated Maximum Bitrate | M |  | <ref> |  | - |  |
| >>Transport Layer Address | FFS |  | <ref> |  | - |  |
| >>GTP-TEID | FFS |  | <ref> | 5GC TEID. | - |  |
| >>NAS-PDU | M |  | <ref> | eNote: interaction with NAS signalling is FFS. | - |  |
| >>Bearer Type (FFS) | M |  | <ref> | eNote: IP, unstructured or Ethernet | - |  |
| >>QoS Flows To Be Setup List |  | 1 |  |  | - |  |
| >>>QoS Flows To Be Setup Item IEs |  | 1 .. <maxnoof QoSFlows> |  |  | - |  |
| >>>>QoS Flow Indicator | M |  | <ref> |  | EACH | reject |
| >>>>QoS Flow Level QoS Parameters | FFS |  | <ref> |  | EACH | reject |

***************END OF MODIFIED SECTION***************