3GPP TSG-RAN3 Meeting #96	R3-171843

Hangzhou, China, 15 – 19 May, 2017

Title: 	TP on Paging message delivery over F1 to 38.470

Source: 	Huawei

Agenda item:	10.10.1

Document for:	Discussion

Introduction

This paper provides text proposal of paging message delivery over F1 to TS 38.470.

Text proposal

----------------------------------------------------1st Change ---------------------------------------------------------------

5.x	Paging Delivery

Figure 5.x-1: Paging Delivery procedure

The Paging Delivery procedure is initiated by gNB-CU for delivering paging information to gNB-DU in order that gNB-DU sending paging message to UE.

The gNB-CU includes RRC paging message as a RRC container and paging occasions of cells in the PAGING DELIVERY message. Upon receiving the PAGING DELIVERY message, the gNB-DU sends the RRC paging message to UE on paging occasions in the cells.