3GPP TSG-RAN WG3 #97	R3-173089

Berlin, Germany, EU, 21 - 25 August 2017

Agenda item:	10.8.1.4

Source:	Nokia, Nokia Shanghai Bell

Title:	S-RLF handling for MR-DC

Document for:	Decision

# 1	Introduction

During the RAN2 NR AdHoc meeting in Qingdao, the following agreements were made while discussing the actions performed by the network upon S-RLF.

Agreements:

1	For all SCG failure cases, the UE maintains the current measurement configurations from both the MN and the SN (i.e. UE does not take autonomous action) and UE continues measurements based on configuration from the MN.

FFS whether the UE continues measurements based on configuration from the SN

2	UE includes in the SCGFailureInformation message the measurement results available according to current measurement configuration of both the MN and the SN.

3	The MN handles the SCGFailureInformation and may decide to keep, change, or release the SN/SCG. The measurement results according to the SN configuration can, in all cases, be forwarded to the old SN and/or to the new SN.

FFS if a different SN can interpret the measurement results based on the configuration from the old SN

4	The UE includes two measurement results in the SCGFailureInformation message: 1) measurement results according to the MN configuration encoded in LTE RRC format, and 2) measurement results according to the SN configuration

FFS Whether the measurement results according to the SN configuration are encoded in NR RRC format and included in a container

In this discussion paper, we discuss the actions performed by the network following an SCG RLF.

A similar paper has also been submitted to RAN2 #99 meeting.

# 2	Background

In LTE DC, a UE monitors the radio quality of the SeNB PSCell via. similar radio link monitoring (RLM) procedures as for MeNB PCell. This is called secondary radio link monitoring (S-RLM) and the occurrence of UE detecting the radio link failure (RLF) of PSCell is referred to as secondary RLF (S-RLF). When S-RLF occurs, the UE stops utilizing the radio resources of the SeNB (i.e., suspends the Secondary Cell Group (SCG) by no longer attempting to receive DL or send UL towards the SeNB) and indicates the failure to MeNB via uplink (using the message “secondary cell group (SCG) failure report”), which contains information of the latest measurements of frequencies configured by SN.

In LTE DC, it was up to MeNB to act per the S-RLF indication (“SCG failure report”), and MeNB was fully aware of the UE measurement configuration and was controlling the SeNB SCell mobility fully, and could react to the occurrence of the S-RLF. However, with LTE-NR DC, the SgNB is in control of the intra SgNB mobility, and the SgNB leg may also fail differently due to, for example, the beam management procedures with higher carrier frequencies or due to LOS blockage occurring or non-delivery of HO command etc. Also, since the SgNB is responsible for handling the mobility within the NR system, the MeNB shall not even comprehend the measurement events configured by the SgNB, the MeNB may not even be fully aware of how to react to the S-RLF indication if received from the UE.

Hence, MN should not make a decision about change/release the SN without offering the first chance to SN.

Figure 1 : Illustration of a typical LTE-NR DC deployment scenario

The problem is best illustrated with the above figure. There are two scenarios under which an S-RLF could occur.

- When there is permanent loss of 5G coverage e.g.: when a UE moves deep into the grey area of the above figure.

- When there is temporary loss of 5G coverage e.g.: when a UE encounters a coverage hole while moving from one white ellipse to the other of the above figure.

While in case (a), the UE is less likely to report a strong candidate PScell in the SCG failure information to MN, in case (b), the UE is very likely to report a strong candidate PScell. Hence, it is very important in case (b) for the MN to forward the SCG failure information to SN and wait for a confirmation from SN before making a decision to change/release the SN.

SN also has already allocated the resources for the UE’s MR-DC bearer and it is appropriate to provide SN the first chance to check if the UE can be recovered in SN before changing SN or releasing the resources. If the SN finds out that the candidate cell reported by the UE in SCG failure report belongs to a different SN, it can also trigger a SN initiated SN change towards MN.

In this context, we make the following proposals

Proposal 1: After the MN receives the SCG failure report from UE due to S-RLF, the MN shall forward the SCGFailureInformation message to the old SN over X2/Xn.

Proposal 2: The old SN processes the SCG failure information and decides either one of the following: retain the UE in the same SN, trigger SN change or inform MN about the absence of a candidate PSCell.

Proposal 3: The old SN informs the MN about the decision. Only after a confirmation from the old SN that retaining or change of SN is not possible, the MN shall take a decision to change or remove the UE’s DRB from the old SN.

# 3	Conclusion

In this discussion paper, we have discussed the need for the network to prioritize the actions of MN and SN just after SCG RLF.

Proposal 1: After the MN receives the SCG failure report from UE due to S-RLF, the MN shall forward the SCGFailureInformation message to the old SN over X2/Xn.

Proposal 2: The old SN processes the SCG failure information and decides either one of the following: retain the UE in the same SN, trigger SN change or inform MN about the absence of a candidate PSCell.

Proposal 3: The old SN informs the MN about the decision. Only after a confirmation from the old SN that retaining or change of SN is not possible, the MN shall take a decision to change or remove the UE’s DRB from the old SN.

Proposal (1) can be based on the existing RLF REPORT message. However, the fact that in case of MR-DC the feedback from the SgNB is needed, it may be more reasonable to define a new class-1 procedure. We leave this decision for the discussion in RAN3.

# References

- 3GPP TS 36.133: Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management, 3GPP

- R2-1706855: UE measurements upon S-RLF for MR-DC, Nokia, Alcatel-Lucent Shanghai Bell