3GPP TSG-RAN WG3 #97	Tdoc R3-173250

Berlin, Germany, 21st – 25th August 2017

Agenda Item:	22.1

Source:	Ericsson, AT&T, Intel

Title:	Resolution of E1 open issues – standardization

Document for:	Discussion, Decision

# Introduction

During RAN #76 a new study item (SI) on “Separation of CP and UP for split option 2 of NR” was approved [1]. In the previous RAN3 meeting, the SI was discussed and a list of open issues was endorsed [2]. In this contribution, we address the open issues related to the second objective of the study item.

# Discussion

The second objective of the SI is reported in the following [1].

- “Identifying details solutions e.g. introducing a standardised control plane interface between the CU-CP and CU-UP part of the gNB to enable the possibility of optimizing the physical location of different RAN functions based on the scenario and desired performance.”

Related to this objective, the following open issues were listed in the previous RAN3 meeting [2].

- Standardize a new i/f between CU-CP and CU-UP?

- SI Objective 2 is fulfilled?

There are two possible solutions for realising the separation of CU-CP and CU-UP: (1) by implementation or (2) by standardizing an open interface.

- Separation of CU-CP and CU-UP by implementation: This approach has the benefit of requiring lower standardization effort. From the standardization point-of-view, this approach only introduces the requirement that the split between CU and DU as well as the F1 interface design should avoid introducing limitations that affect the separation of CU-CP and CU-UP. This approach implies that there would be a proprietary interaction (e.g., a proprietary interface) between CU-CP and CU-UP. This implies that if multiple vendors are chosen for the deployment of CU-CP and CU-UP, then an agreement should be in place between the CU-CP vendor and CU-UP vendor. This may be inefficient in many scenarios and greatly limit the flexibility for network deployment. Failure to achieve such customised design may create interoperability issues if the separation of CU-CP and CU-UP is left for implementation.

Observation 1	The separation of CU-CP and CU-UP by implementation requires lower standardization effort, but requires customized implementation and is exposed to the risk of non-interoperability.

- Separation of CU-CP and CU-UP by standardizing an open interface: This approach requires more standardization effort. However, it is worth noting that a standard interface between CU-CP and CU-UP is expected to be simple as its basic set of functionalities appear to be limited to data radio bearer management. The data radio bearer management can include the functions needed to perform: (1) setup, modification, and release of the data radio bearers, (2) configuration and activation of the security keys, and (3) configuration of the mapping between QoS flows and data radio bearers. Therefore, the standardization effort is expected to be small. It will then be up to RAN3 to decide whether any more functions need to be added to the E1 for further optimisation. A standard interface would then leave great flexibility for the operators in the network deployment phase. It was pointed out that an open interface may limit the radio performance, but it is worth noting that the radio performance is mostly driven by the lower-layer (scheduler) in the DU. Therefore, no significant impact on the radio performance is expected by the introduction of an open interface between CU-CP and CU-UP.

Observation 2	An open interface between CU-CP and CU-UP allows for more flexibility in the network deployment, without impacting the radio performance and requiring significant standardization effort.

The above arguments lead to the following proposals:

Proposal 1	RAN3 to standardize an open interface between CU-CP and CU-UP.

Proposal 2	Objective 2 of the SI is resolved.

Proposal 3	RAN3 is kindly asked to agree on the text proposal in Annex I.

# Conclusion

In this paper, we discussed the separation of CP and UP for split option 2 of NR.

Observation 1	The separation of CU-CP and CU-UP by implementation requires lower standardization effort, but limits the network deployment flexibility.

Observation 2	An open interface between CU-CP and CU-UP allows for more flexibility in the network deployment, without impacting the radio performance and requiring significant standardization effort.

The proposals are summarized below.

Proposal 1	RAN3 to standardize an open interface between CU-CP and CU-UP.

Proposal 2	Objective 2 of the SI is resolved.

Proposal 3	RAN3 is kindly asked to agree on the text proposal in Annex I.

# References

RP-171421, Separation of CP and UP for split option 2 of NR, Ericsson

R3-1712638, Way forward on CP-UP split, Ericsson, AT&T, Vodafone

# Annex I: TP for TR 38.xxx

Start of Text Proposal for TR 38.xxx

6 CP-UP separation: solutions

The second objective of the SI is reported in the following [1]:

- “Identifying details solutions e.g. introducing a standardised control plane interface between the CU-CP and CU-UP part of the gNB to enable the possibility of optimizing the physical location of different RAN functions based on the scenario and desired performance

There are two possible solutions for realising the separation of CU-CP and CU-UP: (1) by implementation or (2) by standardizing an open interface. In the following, we discuss the advantages and drawbacks of these approaches.

6.1 Separation of CU-CP and CU-UP by implementation

This approach has the benefit of requiring lower standardization effort. From the standardization point-of-view, this approach only introduces the requirement that the split between CU and DU as well as the F1 interface design should avoid introducing limitations that affect the separation of CU-CP and CU-UP. This approach implies that there would be a proprietary interaction (e.g., a proprietary interface) between CU-CP and CU-UP. This implies that if multiple vendors are chosen for the deployment of CU-CP and CU-UP, then an agreement should be in place between the CU-CP vendor and CU-UP vendor. This may be inefficient in many scenarios and greatly limit the flexibility for network deployment. Failure to achieve such customised design may create interoperability issues if the separation of CU-CP and CU-UP is left for implementation.

6.2 Standardization of an open interface

This approach requires more standardization effort. However, it is worth noting that a standard interface between CU-CP and CU-UP is expected to be simple as its basic set of functionalities appear to be limited to data radio bearer management. The data radio bearer management can include the functions needed to perform: (1) setup, modification, and release of the data radio bearers, (2) configuration and activation of the security keys, and (3) configuration of the mapping between QoS flows and data radio bearers. Therefore, the standardization effort is expected to be small. It will then be up to RAN3 to decide whether any more functions need to be added to the E1 for further optimisation. A standard interface would then leave great flexibility for the operators in the network deployment phase. It was pointed out that an open interface may limit the radio performance, but it is worth noting that the radio performance is mostly driven by the lower-layer (scheduler) in the DU. Therefore, no significant impact on the radio performance is expected by the introduction of an open interface between CU-CP and CU-UP.

6.3 Conclusions

Two solutions for realizing the separation of CU-CP and CU-UP were identified: by implementation and by standardization of an open interfaces. The benefits and drawbacks of the two approaches were discussed. It was concluded that the standardization of an open interface offers more flexibility for network deployment. Therefore, a new open interface between CU-CP and CU-UP should be standardized. Based on these considerations, the second objective of the study item is successfully fulfilled.

End of Text Proposal for TR 38.xxx