3GPP TSG-RAN WG3 #97	Tdoc R3-173254

Berlin, Germany, 21st – 25th August 2017

Agenda Item:	22.2

Source:	Ericsson, Vodafone, AT&T

Title:	Xn handover in disaggregated gNB with E1 interface

Document for:	Discussion, Decision

# Introduction

In this contribution, we present the stage 2 call-flow for the Xn handover in a disaggregated gNB, with the E1 interface.

# Discussion

We assume that the Xn handover involves changing both the CU-CP and the CU-UP. This is the most comprehensive and complex of all the possible handover scenarios. The other handover procedures can be logically derived from the procedure illustrated in this contribution (e.g., intra-CU-CP and/or intra-CU-UP handover procedures). The Xn handover call-flow is shown in Fig. 1.

The following terminology is utilized to describe the entities involved in the procedure:

- S-gNB: source gNB

- T-gNB: target gNB

- S-DU: source gNB-DU

- S-CU-UP: source gNB-CU-UP

- S-CU-CP: source gNB-CU-CP

- T-DU: target gNB-DU

- T-CU-UP: target CU-UP

- T-CU-CP: target CU-CP

The call-flow can be logically divided into three phases.

- The first phase is the Xn handover preparation:

  - UE sends a RRC MEASUREMENT REPORT to S-DU. The event that triggers the measurement report depends on the UE measurement configuration.

  - S-DU employs the F1AP UL RRC TRANSFER message to forward the RRC measurement report to S-CU-CP.

  - S-CU-CP takes the handover decision, based on the content of the RRC measurement report. S-gNB (S-CU-CP) sends the XnAP ‘HANDOVER REQUEST’ message to T-gNB (T-CU-CP), passing necessary information to prepare the handover at the target side.

  - T-CU-CP performs admission control, creates a UE context, identifies T-DU, and selects T-CU-UP. Then, T-CU-CP sends the F1-AP CONTEXT SETUP REQUEST message to T-DU, including UE context information and CU-UP-UL-TEID for data radio bearers.

  - T-DU performs admission control, configures the lower-layers, and creates a local UE context (including a C-RNTI for the UE). Then, T-DU sends the F1-AP CONTEXT SETUP RESPONSE message to T-CU-CP, including lower-layers configuration, C-RNTI, and DU-DL-TEID for data radio bearers.

  - T-CU-CP sends the E1-AP BEARER SETUP message to T-CU-UP, including information for security configuration and QoS-flows to DRB mapping, and DU-DL-TEID.

  - T-CU-UP applies the configuration received from T-CU-CP. Then, T-CU-UP sends the E1AP BEARER SETUP RESPONSE message to T-CU-CP.

  - T-gNB (T-CU-CP) sends the XnAP HANDOVER REQUEST ACKNOWLEDGE to S-gNB (S-CU-CP). The XnAP HANDOVER REQUEST ACKNOWLEDGE message includes a transparent container to be sent to the UE as an RRC message to perform the handover (includes C-RNTI). The XnAP HANDOVER REQUEST ACKNOWLEDGE message may also include RNL/TNL information for the data forwarding tunnels (if applicable).

NOTE: It is possible that steps 4-7 are performed in parallel if the T-CU-CP knows the DU-DL-TEID.

- Fig. 1: Xn handover in the disaggregated gNB deployment.

- The second phase is the Xn handover execution:

- S-CU-CP sends the F1AP UE CONTEXT MODIFICATION REQUEST message to S-DU. The F1AP UE CONTEXT MODIFICATION REQUEST message includes a transparent container that carries the RRC message to perform handover, which was generated by T-CU-CP and ciphered and integrity protected by S-CU-CP. The F1AP UE CONTEXT MODIFICATION REQUEST message contains a specific field value to inform S-DU about the handover.

- S-DU sends the F1AP UE CONTEXT MODIFICATION RESPONSE message to S-CU-CP to confirm that the handover is accepted.

- S-DU sends the RRC CONNECTION RECONFIGURATION MESSAGE to the UE.

- S-CU-CP sends the E1AP BEARER MODIFICATION message to S-CU-UP. The E1AP BEARER MODIFICATION message may include the RNL/TNL information for the data forwarding tunnels (if applicable).

- S-CU-UP sends the E1AP BEARER MODIFICATION ACK message to S-CU-CP, including the uplink PDCP SN receiver status and the downlink PDCP SN transmitter status.

- S-gNB (S-CU-CP) sends the XnAP SN STATUS TRANFER message to T-gNB (T-CU-CP) to convey the uplink PDCP SN receiver status and the downlink PDCP SN transmitter status.

- T-CU-CP sends the E1AP SN STATUS TRASNFER message to T-CU-UP to convey the uplink PDCP SN receiver status and the downlink PDCP SN transmitter status.

- Data forwarding may start from S-CU-UP to T-CU-UP (if applicable).

- UE sends a RANDOM ACCESS preamble to T-DU. It may employ a dedicated RACH preamble if that was included in the RRC connection reconfiguration message.

- T-DU sends a RANDOM ACCESS RESPONSE to UE.

- UE sends the RRC CONNECTION CONFIGURATION COMPLETE to T-DU (include the C-RNTI to identify the UE).

- T-DU employs the F1AP UL RRC TRANSFER message to forward the RRC connection reconfiguration complete message to T-CU-CP.

- T-gNB (T-CU-CP) sends a NGAP PATH SWITCH REQUEST message to AMF to inform that the UE has changed cell. AMF should then contact the UPF to modify PDU session (not shown).

- AMF confirms the NGAP PATH SWITCH REQUEST message with the NGAP PATH SWITCH REQUEST ACKNOWLEDGE message.

NOTE: Steps 10 and 13 may not be required in some cases, because F1AP and E1AP messages are transmitted using a reliable transport (SCTP/IP).

- The last phase is the resource release in the source:

- T-gNB (T-CU-CP) sends the XnAP UE CONTEXT RELEASE message to S-gNB (S-CU-CP). The XnAP UE CONTEXT RELEASE message informs S-gNB (S-CU-CP) that the handover was successful.

- S-CU-CP sends the F1AP UE CONTEXT RELEASE message to S-DU, to allow S-DU to release the resources allocated to the UE.

- S-CU-UP sends the E1AP DATA FORWARD COMPLETE message to S-CU-CP as soon as the data forwarding to the target has been completed.

- S-CU-CP sends the E1AP BEARER RELEASE message to S-CU-UP to release the data radio bearers and release the corresponding resources.

# Conclusion

We conclude with the following proposals:

Proposal 1	RAN3 is kindly asked to agree with the proposed call-flow for Xn handover in disaggregated gNB deployment with E1 interface.

Proposal 2	RAN3 is kindly asked to agree with the TP in Annex I.

# Annex I: TP for TR 38.xxx

Start of Text Proposal for TR 38.xxx

7.5.1 Xn Handover

This section presents the Xn handover that involves changing both the CU-CP and the CU-UP. It is the most comprehensive of the handover scenarios. The other handover procedures can be logically derived from the procedure illustrated in this section. The Xn handover call-flow is shown in Figure 7.5.1-1.

The following terminology is utilized to describe the entities involved in the procedure:

- S-gNB: source gNB

- T-gNB: target gNB

- S-DU: source DU

- S-CU-UP: source CU-UP

- S-CU-CP: source CU-CP

- T-DU: target DU

- T-CU-UP: target CU-UP

- T-CU-CP: target CU-CP

The call-flow can be logically divided into three phases.

- The first phase is the Xn handover preparation:

  - The UE sends a RRC MEASUREMENT REPORT to the S-DU. The event that triggers the measurement report depends on the UE measurement configuration.

  - The S-DU employs the F1AP UL RRC TRANSFER message to forward the RRC measurement report to the S-CU-CP.

  - The S-CU-CP takes the handover decision, based on the content of the RRC measurement report. S-gNB (S-CU-CP) sends the XnAP ‘HANDOVER REQUEST’ message to the T-gNB (T-CU-CP), passing necessary information to prepare the handover at the target side.

  - The T-CU-CP performs admission control, creates a UE context, identifies T-DU, and selects T-CU-UP. Then, T-CU-CP sends the F1-AP CONTEXT SETUP REQUEST message to the T-DU, including UE context information and CU-UP-UL-TEID for data radio bearers.

  - The T-DU performs admission control, configures the lower-layers, and creates a local UE context (including a C-RNTI for the UE). Then, T-DU sends the F1-AP CONTEXT SETUP RESPONSE message to the T-CU-CP, including lower-layers configuration, C-RNTI, and DU-DL-TEID for data radio bearers.

  - The T-CU-CP sends the E1-AP BEARER SETUP message to the T-CU-UP, including information for security configuration and QoS-flows to DRB mapping, and DU-DL-TEID.

  - The T-CU-UP applies the configuration received from T-CU-CP. Then, T-CU-UP sends the E1AP BEARER SETUP RESPONSE message to the T-CU-CP.

  - The T-gNB (T-CU-CP) sends the XnAP HANDOVER REQUEST ACKNOWLEDGE to the S-gNB (S-CU-CP). The XnAP HANDOVER REQUEST ACKNOWLEDGE message includes a transparent container to be sent to the UE as an RRC message to perform the handover (includes C-RNTI). The XnAP HANDOVER REQUEST ACKNOWLEDGE message may also include RNL/TNL information for the data forwarding tunnels (if applicable).

NOTE: It is possible that steps 4-7 are performed in parallel if the T-CU-CP knows the DU-DL-TEID.

- Fig. 7.5.1-1: Xn handover in the disaggregated gNB deployment.

- The second phase is the Xn handover execution:

- The S-CU-CP sends the F1AP UE CONTEXT MODIFICATION REQUEST message to the S-DU. The F1AP UE CONTEXT MODIFICATION REQUEST message includes a transparent container that carries the RRC message to perform handover, which was generated by T-CU-CP and ciphered and integrity protected by S-CU-CP. The F1AP UE CONTEXT MODIFICATION REQUEST message contains a specific field value to inform S-DU about the handover.

- The S-DU sends the F1AP UE CONTEXT MODIFICATION RESPONSE message to S-CU-CP to confirm that the handover is accepted.

- The S-DU sends the RRC CONNECTION RECONFIGURATION MESSAGE to the UE.

- The S-CU-CP sends the E1AP BEARER MODIFICATION message to the S-CU-UP. The E1AP BEARER MODIFICATION message may include the RNL/TNL information for the data forwarding tunnels (if applicable).

- The S-CU-UP sends the E1AP BEARER MODIFICATION ACK message to the S-CU-CP, including the uplink PDCP SN receiver status and the downlink PDCP SN transmitter status.

- The S-gNB (S-CU-CP) sends the XnAP SN STATUS TRANFER message to T-gNB (T-CU-CP) to convey the uplink PDCP SN receiver status and the downlink PDCP SN transmitter status.

- The T-CU-CP sends the E1AP SN STATUS TRASNFER message to the T-CU-UP to convey the uplink PDCP SN receiver status and the downlink PDCP SN transmitter status.

- Data forwarding may start from S-CU-UP to the T-CU-UP (if applicable).

- The UE sends a RANDOM ACCESS preamble to the T-DU. It may employ a dedicated RACH preamble if that was included in the RRC connection reconfiguration message.

- The T-DU sends a RANDOM ACCESS RESPONSE to UE.

- The UE sends the RRC CONNECTION CONFIGURATION COMPLETE to the T-DU (include the C-RNTI to identify the UE).

- The T-DU employs the F1AP UL RRC TRANSFER message to forward the RRC connection reconfiguration complete message to T-CU-CP.

- The T-gNB (T-CU-CP) sends a NGAP PATH SWITCH REQUEST message to the AMF to inform that the UE has changed cell. AMF should then contact the UPF to modify PDU session (not shown).

- The AMF confirms the NGAP PATH SWITCH REQUEST message with the NGAP PATH SWITCH REQUEST ACKNOWLEDGE message.

NOTE: Steps 10 and 13 may not be required in some cases, because F1AP and E1AP messages are transmitted using a reliable transport (SCTP/IP).

- The last phase is the resource release in the source:

- T-gNB (T-CU-CP) sends the XnAP UE CONTEXT RELEASE message to S-gNB (S-CU-CP). The XnAP UE CONTEXT RELEASE message informs S-gNB (S-CU-CP) that the handover was successful.

- S-CU-CP sends the F1AP UE CONTEXT RELEASE message to S-DU, to allow T-DU to release the resources allocated to the UE.

- S-CU-UP sends the E1AP DATA FORWARD COMPLETE message to S-CU-CP as soon as the data forwarding to the target has been completed.

- S-CU-CP sends the E1AP BEARER RELEASE message to S-CU-UP to release the data radio bearers and release the corresponding resources.

End of Text Proposal for TR 38.xxx