3GPP TSG-RAN WG3 Meeting #97	R3-173323

# Berlin, Germany, 21st -25th  May 2017

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.423 | CR | 1040 | rev | 1 | Current version: | 14.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support 4T bps in X2AP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2017-08-11 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | X2AP need to be extended to support maximum data rate at 4T bps. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the Extended Bit Rate to support maximum data rate 4T bps. Update the related IE groups. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for 4T bps as maximum data rate. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.10, 9.2.12, 9.2.xx(new), 9.3.5, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 36.413 CR 1527 |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 9.2.10	GBR QoS Information

This IE indicates the maximum and guaranteed bit rates of a GBR E-RAB for downlink and uplink.

NOTE:	The SeNB regards the GBR QoS Information IE as an E-RAB level parameter also for E-RABs configured with the split bearer option, although for the split bearer option the bitrates signalled by the MeNB are typically not equal to the bitrates signalled by the MME for that E-RAB (see TS 36.300 [15]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| E-RAB Maximum Bit Rate Downlink | M |  | Bit Rate 9.2.11 | Maximum Bit Rate in DL (i.e. from EPC to E-UTRAN) for the bearer. Details in TS 23.401 [12]. If the Extended E-RAB Maximum Bit Rate Downlink IE is included, the E-RAB Maximum Bit Rate Downlink IE shall be ignored. | – | – |
| E-RAB Maximum Bit Rate Uplink | M |  | Bit Rate 9.2.11 | Maximum Bit Rate in UL (i.e. from E-UTRAN to EPC) for the bearer. Details in TS 23.401 [12]. If the Extended E-RAB Maximum Bit Rate Uplink IE is included, the E-RAB Maximum Bit Rate Uplink IE shall be ignored. | – | – |
| E-RAB Guaranteed Bit Rate Downlink | M |  | Bit Rate 9.2.11 | Guaranteed Bit Rate (provided that there is data to deliver) in DL (i.e. from EPC to E-UTRAN) for the bearer. Details in TS 23.401 [12]. If the Extended E-RAB Guaranteed Bit Rate Downlink IE is included, the E-RAB Guaranteed Bit Rate Downlink IE shall be ignored. | – | – |
| E-RAB Guaranteed Bit Rate Uplink | M |  | Bit Rate 9.2.11 | Guaranteed Bit Rate (provided that there is data to deliver) in UL (i.e. from E-UTRAN to EPC) for the bearer. Details in TS 23.401 [12]. If the Extended E-RAB Guaranteed Bit Rate Uplink IE is included, the E-RAB Guaranteed Bit Rate Uplink IE shall be ignored. | – | – |
| Extended E-RAB Maximum Bit Rate Downlink | O |  | Extended Bit Rate 9.2.xx | Maximum Bit Rate in DL (i.e. from EPC to E-UTRAN) for the bearer. Details in TS 23.401 [12]. | – | – |
| Extended E-RAB Maximum Bit Rate Uplink | O |  | Extended Bit Rate 9.2.xx | Maximum Bit Rate in UL (i.e. from E-UTRAN to EPC) for the bearer. Details in TS 23.401 [12]. | – | – |
| Extended E-RAB Guaranteed Bit Rate Downlink | O |  | Extended Bit Rate 9.2.xx | Guaranteed Bit Rate (provided that there is data to deliver) in DL (i.e. from EPC to E-UTRAN) for the bearer. Details in TS 23.401 [12]. | – | – |
| Extended E-RAB Guaranteed Bit Rate Uplink | O |  | Extended Bit Rate 9.2.xx | Guaranteed Bit Rate (provided that there is data to deliver) in UL (i.e. from E-UTRAN to EPC) for the bearer. Details in TS 23.401 [12]. | – | – |

### 9.2.11	Bit Rate

This IE indicates the number of bits delivered by E-UTRAN in UL or to E-UTRAN in DL or by UE in sidelink within a period of time, divided by the duration of the period. It is used, for example, to indicate the maximum or guaranteed bit rate for a GBR E-RAB, or an aggregated maximum bit rate.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Bit Rate | M |  | INTEGER (0..10,000,000,000) | The unit is: bit/s |

### 9.2.12	UE Aggregate Maximum Bit Rate

On Handover Aggregate Maximum Bitrate is transferred to the target eNB. In Dual Connectivity, UE Aggregate Maximum Bit Rate is split into MeNB UE Aggregate Maximum Bit Rate and SeNB UE Aggregate Maximum Bit Rate which are enforced by MeNB and SeNB respectively as specified in TS 36.300 [15]. The UE Aggregate Maximum Bitrate is applicable for all Non-GBR bearers per UE which is defined for the Downlink and the Uplink direction and provided by the MME to the eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| UE Aggregate Maximum Bit Rate Downlink | M |  | Bit Rate 9.2.11 | If the Extended UE Aggregate Maximum Bit Rate Downlink IE is included, the UE Aggregate Maximum Bit Rate Downlink IE shall be ignored. | – | – |
| UE Aggregate Maximum Bit Rate Uplink | M |  | Bit Rate 9.2.11 | If the Extended UE Aggregate Maximum Bit Rate Uplink IE is included, the UE Aggregate Maximum Bit Rate Uplink IE shall be ignored. | – | – |
| Extended UE Aggregate Maximum Bit Rate Downlink | O |  | Extended Bit Rate 9.2.xx | UE Aggregate Maximum Bit Rate in DL. Details in TS 23.401 [12]. | – | – |
| Extended UE Aggregate Maximum Bit Rate Uplink | O |  | Extended Bit Rate 9.2.xx | UE Aggregate Maximum Bit Rate in UL. Details in TS 23.401 [12]. | – | – |

------------------------------------------

Skip to next change

------------------------------------------

### 9.2.xx	Extended Bit Rate

This IE indicates the number of bits delivered by E-UTRAN in UL or to E-UTRAN in DL within a period of time, divided by the duration of the period. It is used, for example, to indicate the maximum or guaranteed bit rate for a GBR bearer, or an aggregated maximum bit rate.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Extended Bit Rate |  |  | INTEGER (10,000,000,001..4,000,000,000,000,...) | The unit is: bit/s |

### 9.3.5	Information Element definitions

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

X2AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) x2ap (2) version1 (1) x2ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-E-RAB-Item,

id-Number-of-Antennaports,

id-MBSFN-Subframe-Info,

id-PRACH-Configuration,

id-CSG-Id,

id-MDTConfiguration,

id-SignallingBasedMDTPLMNList,

id-MultibandInfoList,

id-FreqBandIndicatorPriority,

id-NeighbourTAC,

id-Time-UE-StayedInCell-EnhancedGranularity,

id-MBMS-Service-Area-List,

id-HO-cause,

id-eARFCNExtension,

id-DL-EARFCNExtension,

id-UL-EARFCNExtension,

id-M3Configuration,

id-M4Configuration,

id-M5Configuration,

id-MDT-Location-Info,

id-AdditionalSpecialSubframe-Info,

id-UEID,

id-enhancedRNTP,

id-ProSeUEtoNetworkRelaying,

id-M6Configuration,

id-M7Configuration,

id-OffsetOfNbiotChannelNumberToDL-EARFCN,

id-OffsetOfNbiotChannelNumberToUL-EARFCN,

id-AdditionalSpecialSubframeExtension-Info,

id-BandwidthReducedSI,

id-extended-e-RAB-MaximumBitrateDL,

id-extended-e-RAB-MaximumBitrateUL,

id-extended-e-RAB-GuaranteedBitrateDL,

id-extended-e-RAB-GuaranteedBitrateUL,

id-extended-uEaggregateMaximumBitRateDownlink,

id-extended-uEaggregateMaximumBitRateUplink,

maxnoofBearers,

maxCellineNB,

maxEARFCN,

maxEARFCNPlusOne,

newmaxEARFCN,

maxInterfaces,

------------------------------------------

Skip to next change

------------------------------------------

#### -- E

EARFCN ::= INTEGER (0..maxEARFCN)

EARFCNExtension ::= INTEGER(maxEARFCNPlusOne..newmaxEARFCN, ...)

ECGI ::= SEQUENCE {

pLMN-Identity				PLMN-Identity,

eUTRANcellIdentifier		EUTRANCellIdentifier,

iE-Extensions				ProtocolExtensionContainer { {ECGI-ExtIEs} } OPTIONAL,

...

}

ECGI-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

EnhancedRNTP ::= SEQUENCE {

enhancedRNTPBitmap			BIT STRING (SIZE(12..8800, ...)),

rNTP-High-Power-Threshold	RNTP-Threshold,

enhancedRNTPStartTime		EnhancedRNTPStartTime  OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { EnhancedRNTP-ExtIEs} } OPTIONAL,

...

}

EnhancedRNTP-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

EnhancedRNTPStartTime ::= SEQUENCE {

startSFN				INTEGER (0..1023, ...),

startSubframeNumber		INTEGER (0..9, ...),

iE-Extensions			ProtocolExtensionContainer { { EnhancedRNTPStartTime-ExtIEs} } OPTIONAL,

...

}

EnhancedRNTPStartTime-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

ENB-ID ::= CHOICE {

macro-eNB-ID	BIT STRING (SIZE (20)),

home-eNB-ID		BIT STRING (SIZE (28)),

... ,

short-Macro-eNB-ID		BIT STRING (SIZE(18)),

long-Macro-eNB-ID		BIT STRING (SIZE(21))

}

EncryptionAlgorithms ::= BIT STRING (SIZE (16, ...))

EPLMNs ::= SEQUENCE (SIZE(1..maxnoofEPLMNs)) OF PLMN-Identity

E-RAB-ID ::= INTEGER (0..15, ...)

E-RAB-Level-QoS-Parameters ::= SEQUENCE {

qCI								QCI,

allocationAndRetentionPriority	AllocationAndRetentionPriority,

gbrQosInformation				GBR-QosInformation													OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { E-RAB-Level-QoS-Parameters-ExtIEs} }	OPTIONAL,

...

}

E-RAB-Level-QoS-Parameters-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RAB-List ::= SEQUENCE (SIZE(1.. maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RAB-ItemIEs} }

E-RAB-ItemIEs 	X2AP-PROTOCOL-IES ::= {

{ ID id-E-RAB-Item	 CRITICALITY ignore 	TYPE E-RAB-Item 	PRESENCE mandatory },

...

}

E-RAB-Item ::= SEQUENCE {

e-RAB-ID				E-RAB-ID,

cause						Cause,

iE-Extensions				ProtocolExtensionContainer { {E-RAB-Item-ExtIEs} } OPTIONAL,

...

}

E-RAB-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

EUTRA-Mode-Info ::= CHOICE {

fDD		FDD-Info,

tDD		TDD-Info,

...

}

EUTRANCellIdentifier ::= BIT STRING (SIZE (28))

EUTRANTraceID		::= OCTET STRING (SIZE (8))

EventType ::= ENUMERATED{

change-of-serving-cell,

...

}

ExpectedUEBehaviour ::= SEQUENCE {

expectedActivity		ExpectedUEActivityBehaviour OPTIONAL,

expectedHOInterval		ExpectedHOInterval			OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { ExpectedUEBehaviour-ExtIEs} } OPTIONAL,

...

}

ExpectedUEBehaviour-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

ExpectedUEActivityBehaviour ::= SEQUENCE {

expectedActivityPeriod					ExpectedActivityPeriod					OPTIONAL,

expectedIdlePeriod						ExpectedIdlePeriod						OPTIONAL,

sourceofUEActivityBehaviourInformation	SourceOfUEActivityBehaviourInformation	OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { ExpectedUEActivityBehaviour-ExtIEs} } OPTIONAL,

...

}

ExpectedUEActivityBehaviour-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

ExpectedActivityPeriod ::= INTEGER (1..30|40|50|60|80|100|120|150|180|181,...)

ExpectedIdlePeriod ::= INTEGER (1..30|40|50|60|80|100|120|150|180|181,...)

ExpectedHOInterval ::= ENUMERATED {

sec15, sec30, sec60, sec90, sec120, sec180, long-time,

...

}

ExtendedULInterferenceOverloadInfo ::= SEQUENCE {

associatedSubframes							BIT STRING (SIZE (5)),

extended-ul-InterferenceOverloadIndication	UL-InterferenceOverloadIndication,

iE-Extensions								ProtocolExtensionContainer { { ExtendedULInterferenceOverloadInfo-ExtIEs} } OPTIONAL,

...

}

ExtendedULInterferenceOverloadInfo-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

ExtendedBitRate	::= INTEGER (10000000001..4000000000000,...)

------------------------------------------

Skip to next change

------------------------------------------

#### -- G

GBR-QosInformation ::= SEQUENCE {

e-RAB-MaximumBitrateDL			BitRate,

e-RAB-MaximumBitrateUL			BitRate,

e-RAB-GuaranteedBitrateDL		BitRate,

e-RAB-GuaranteedBitrateUL		BitRate,

iE-Extensions					ProtocolExtensionContainer { { GBR-QosInformation-ExtIEs} } OPTIONAL,

...

}

GBR-QosInformation-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

-- Extension for maximum bitrate > 10Gbps --

{ ID id-extended-e-RAB-MaximumBitrateDL	CRITICALITY ignore	EXTENSION ExtendedBitRate	PRESENCE optional}|

{ ID id-extended-e-RAB-MaximumBitrateUL	CRITICALITY ignore	EXTENSION ExtendedBitRate	PRESENCE optional}|

{ ID id-extended-e-RAB-GuaranteedBitrateDL	CRITICALITY ignore	EXTENSION ExtendedBitRate	PRESENCE optional}|

{ ID id-extended-e-RAB-GuaranteedBitrateUL	CRITICALITY ignore	EXTENSION ExtendedBitRate	PRESENCE optional},

...

}

GlobalENB-ID ::= SEQUENCE {

pLMN-Identity			PLMN-Identity,

eNB-ID					ENB-ID,

iE-Extensions			ProtocolExtensionContainer { {GlobalENB-ID-ExtIEs} } OPTIONAL,

...

}

GlobalENB-ID-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

GTPtunnelEndpoint ::= SEQUENCE {

transportLayerAddress			TransportLayerAddress,

gTP-TEID						GTP-TEI,

iE-Extensions					ProtocolExtensionContainer { {GTPtunnelEndpoint-ExtIEs} } OPTIONAL,

...

}

GTPtunnelEndpoint-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

GTP-TEI					::= OCTET STRING (SIZE (4))

GUGroupIDList		::= SEQUENCE (SIZE (1..maxPools)) OF GU-Group-ID

GU-Group-ID			::= SEQUENCE {

pLMN-Identity		PLMN-Identity,

mME-Group-ID		MME-Group-ID,

iE-Extensions		ProtocolExtensionContainer { {GU-Group-ID-ExtIEs} } OPTIONAL,

...

}

GU-Group-ID-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

------------------------------------------

Skip to next change

------------------------------------------

#### -- U

UEAggregateMaximumBitRate ::= SEQUENCE {

uEaggregateMaximumBitRateDownlink	BitRate,

uEaggregateMaximumBitRateUplink		BitRate,

iE-Extensions						ProtocolExtensionContainer { {UEAggregate-MaximumBitrate-ExtIEs} } OPTIONAL,

...

}

UEAggregate-MaximumBitrate-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-extended-uEaggregateMaximumBitRateDownlink	CRITICALITY ignore	EXTENSION ExtendedBitRate	PRESENCE optional}|

{ ID id-extended-uEaggregateMaximumBitRateUplink	CRITICALITY ignore	EXTENSION ExtendedBitRate	PRESENCE optional},

...

}

UE-ContextKeptIndicator ::= ENUMERATED {

true,

...

}

------------------------------------------

Skip to next change

------------------------------------------

### 9.3.7	Constant definitions

-- **************************************************************

--

#### -- Constant definitions

--

-- **************************************************************

X2AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) x2ap (2) version1 (1) x2ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM X2AP-CommonDataTypes;

------------------------------------------

Skip to next change

------------------------------------------

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-E-RABs-Admitted-Item														ProtocolIE-ID ::= 0

id-E-RABs-Admitted-List														ProtocolIE-ID ::= 1

id-E-RAB-Item																ProtocolIE-ID ::= 2

id-E-RABs-NotAdmitted-List													ProtocolIE-ID ::= 3

id-E-RABs-ToBeSetup-Item													ProtocolIE-ID ::= 4

id-Cause																	ProtocolIE-ID ::= 5

------------------------------------------

Skip to next change

------------------------------------------

id-OffsetOfNbiotChannelNumberToDL-EARFCN									ProtocolIE-ID ::= 177

id-OffsetOfNbiotChannelNumberToUL-EARFCN									ProtocolIE-ID ::= 178

id-AdditionalSpecialSubframeExtension-Info									ProtocolIE-ID ::= 179

id-BandwidthReducedSI														ProtocolIE-ID ::= 180

id-MakeBeforeBreakIndicator													ProtocolIE-ID ::= 181

id-UE-ContextReferenceAtWT													ProtocolIE-ID ::= 182

id-WT-UE-ContextKeptIndicator												ProtocolIE-ID ::= 183

id-UESidelinkAggregateMaximumBitRate										ProtocolIE-ID ::= 184

id-uL-GTPtunnelEndpoint														ProtocolIE-ID ::= 185

id-extended-e-RAB-MaximumBitrateDL											ProtocolIE-ID ::= 186

id-extended-e-RAB-MaximumBitrateUL											ProtocolIE-ID ::= 187

id-extended-e-RAB-GuaranteedBitrateDL										ProtocolIE-ID ::= 188

id-extended-e-RAB-GuaranteedBitrateUL										ProtocolIE-ID ::= 189

id-extended-uEaggregateMaximumBitRateDownlink								ProtocolIE-ID ::= 190

id-extended-uEaggregateMaximumBitRateUplink									ProtocolIE-ID ::= 191

END