# 3GPP TSG RAN WG3 meeting #97bis	R3-173710

# Prague, Czech, October 9th -13th 2017

Agenda Item:	10.13

Source: 	Huawei

Title:	Details procedures for NRPPa

Document for:	 pCR

# Introduction

The following procedure attempt to capture the following WA:

WA: Adopt applicable parts of LPPa into NRPPa and make them applicable for NG-RAN nodes with a NG interface towards the 5GC.

WA: There is a single protocol (supporting both RATs) between the NG-RAN Node and the termination point in the 5GC

First, the WA can be turn to agreements.

The below in the pCR, the NRPPa procedure are reported with the principle that the SMLC (still kept E-SMLC for now as convenience) asking to the NG-RAN Node to report the positioning method independently if it is an ng-eNB or a gNB. It is up to RAN to provide the adequate reporting based on NG-RAN Node. However some IE are FFS because it is needed to check whether they apply to E-UTRA and NR cells

From our understanding OTDOA procedures should be also included for ng-eNB only as well as the Inter-RAT Measurement Results and the WLAN Measurement Results. We had indicated this dedicated behaviour of ng-eNB by “NG-RAN node, i.e. the ng-eNB, in the procedural text, The existing cause value ‘Requested Item not Supported’ and the Error Indication procedure seems to be enough to cover the case where OTDOA procedure request is sent to a gNB. The Inter-RAT Measurement Results and the WLAN Measurement Results are optional IE with criticality ‘ignored’, the gNB will ignore these IE on return only the cell ID associated in this case. However it is debatable if the feature support should be clarified at Stage 2 or/and additional abnormal condition should be described at stage3.

The related NB IoT IEs are also removed, in ‘strikethrough’ mode and not in tracking change. NB IoT is not supported by ng-eNB in rel-15. The IEs should be removed at TP integration or revision can be provided.

# pCR

3GPP TS 38.455 V0.1.1 (2017-08)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

NG-RAN;

NR Positioning Protocol A (NRPPa)

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

<keyword[, keyword, …]>

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2017, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	6

2	References	6

3	Definitions, symbols and abbreviations	6

3.1	Definitions	6

3.2	Symbols	6

3.3	Abbreviations	7

4	General	7

4.1	Procedure specification principles	7

4.2	Forwards and backwards compatibility	7

4.3	Specification notations	7

5	NRPPa services	7

6	Services expected from lower layer	7

7	Functions of NRPPa	7

8	NRPPa procedures	8

8.1	Elementary procedures	8

8.2	Location Information Transfer Procedures	8

8.2.1	E-CID Measurement Initiation	8

8.2.1.1	General	8

8.2.1.2	Successful Operation	9

8.2.1.3	Unsuccessful Operation	10

8.2.2	E-CID Measurement Failure Indication	10

8.2.2.1	General	10

8.2.2.2	Successful Operation	10

8.2.2.3	Unsuccessful Operation	10

8.2.3	E-CID Measurement Report	10

8.2.3.1	General	10

8.2.3.2	Successful Operation	11

8.2.3.3	Unsuccessful Operation	11

8.2.4	E-CID Measurement Termination	11

8.2.4.1	General	11

8.2.4.2	Successful Operation	11

8.2.4.3	Unsuccessful Operation	12

8.2.x	OTDOA Information Exchange	12

8.2.x.1	General	12

8.2.x.2	Successful Operation	12

8.2.5.3	Unsuccessful Operation	12

8.3	Management Procedures	13

8.3.1	Error Indication	13

8.3.1.1	General	13

8.3.1.2	Successful Operation	13

8.3.1.3	Abnormal Conditions	13

9	Elements for NRPPa Communication	13

9.1.x	Messages for Location Information Transfer Procedures	14

9.1.x.y	E-CID MEASUREMENT INITIATION REQUEST	14

9.1.x.y	E-CID MEASUREMENT INITIATION RESPONSE	14

9.1.x.y	E-CID MEASUREMENT INITIATION FAILURE	15

9.1.x.y	E-CID MEASUREMENT FAILURE INDICATION	15

9.1.x.y	E-CID MEASUREMENT REPORT	15

9.1.x.y	E-CID MEASUREMENT TERMINATION COMMAND	16

9.1.x.y	OTDOA INFORMATION REQUEST	16

9.1.1.8	OTDOA INFORMATION RESPONSE	17

9.1.x.y	OTDOA INFORMATION FAILURE	17

9.1.z	Messages for Management Procedures	17

9.1.z.y	ERROR INDICATION	17

9.U	Information Element definitions	18

9.U.v	General	18

9.U.v	Cause	18

9.U.v	Criticality Diagnostics	19

9.U.v	Message Type	20

9.2.4	NRPPa Transaction ID	20

9.2.5	E-CID Measurement Result	21

9.2.x	ECGI (FFS)	22

9.2.x	NCGI (FFS)	22

9.2.x	OTDOA Cell Information	22

9.2.x	RAN Access Point Position	24

9.2.x	PRS Muting Configuration	25

9.2.x	Cell Portion ID	26

9.2.x	Inter-RAT Measurement Result	26

9.2.15	WLAN Measurement Result	28

9.2.19	PRS Frequency Hopping Configuration	28

10	Handling of unknown, unforeseen and erroneous protocol data	29

Annex <X> (informative): Change history	31

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document specifies the control plane radio network layer signalling procedures between a NG-RAN node and the E-SMLC. NRPPa supports the concerned functions by signalling procedures defined in this document.

Editors note: The termination point in the 5GC side needs to be defined by SA2. All instances of the term E-SMLC in this draft specification will be replaced by the termination point decided by SA2.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.413:" NG Radio Access Network (NG-RAN); NG Application Protocol (NGAP)".

[x]	3GPP TR 25.921 (version.7.0.0): "Guidelines and principles for protocol description and error handling".

[x]	3GPP TS 36.211:"Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Physical Channels and Modulation".

[x]	3GPP TS 23.032:"Technical Specification Group Services and System Aspects; Universal Geographical Area Description (GAD)".

[x] 	3GPP TS 36.133:"Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

# 3	Definitions, symbols and abbreviations

Delete from the above heading those words which are not applicable.

Clause numbering depends on applicability and should be renumbered accordingly.

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

CID	Cell-ID (positioning method)

E-CID	Enhanced Cell-ID (positioning method)

# 4	General

## 4.1	Procedure specification principles

Editor’s Note: Intention is that this section is based on section 4.1 in 36.455.

## 4.2	Forwards and backwards compatibility

Editor’s Note: Intention is that this section is based on section 4.2 in 36.455.

## 4.3	Specification notations

Editor’s Note: Intention is that this section is based on section 4.3 in 36.455.

# 5	NRPPa services

Editor’s Note: Intention is that this section is based on section 5 in 36.455.

# 6	Services expected from lower layer

Editor’s Note: Intention is that this section is based on section 6 in 36.455.

Within 5G RAN, NRPPa protocol uses the services provided by the NGAP protocol. An NRPPa message is carried inside an NGAP message.

NGAP signalling is described in TS 38.413 [2].

# 7	Functions of NRPPa

Editor’s Note: Intention is that this section is based on section 7 in 36.455.

The NRPPa protocol provides the following functions:

-	E-CID Location Information Transfer. This function allows the NG-RAN node to exchange location information with the E-SMLC for the purpose of E-CID positioning.

-	Reporting of General Error Situations. This function allows reporting of general error situations, for which function specific error messages have not been defined.

The mapping between the above functions and NRPPa EPs is shown in the table below.

Table 7-1: Mapping between NRPPa functions and NRPPa EPs

| Function | Elementary Procedure(s) |
| --- | --- |
| E-CID Location Information Transfer | a) E-CID Measurement Initiation b) E-CID Measurement Failure Indication c) E-CID Measurement Report d) E-CID Measurement Termination |
| Reporting of General Error Situations | Error Indication |

# 8	NRPPa procedures

Editor’s Note: Intention is that this section is based on section 8 in 36.455.

## 8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

# Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- |
|  |  | Response message | Response message |  |
| E-CID Measurement Initiation | E-CID MEASUREMENT INITIATION REQUEST | E-CID MEASUREMENT INITIATION RESPONSE | E-CID MEASUREMENT INITIATION FAILURE |
| OTDOA Information Exchange | OTDOA INFORMATION REQUEST | OTDOA INFORMATION RESPONSE | OTDOA INFORMATION FAILURE |  |

# Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| E-CID Measurement Failure Indication | E-CID MEASUREMENT FAILURE INDICATION |
| E-CID Measurement Report | E-CID MEASUREMENT REPORT |
| E-CID Measurement Termination | E-CID MEASUREMENT TERMINATION COMMAND |
| Error Indication | ERROR INDICATION |

## 8.2	Location Information Transfer Procedures

### 8.2.1	E-CID Measurement Initiation

#### 8.2.1.1	General

The purpose of E-CID Measurement Initiation procedure is to allow the E-SMLC to request the NG-RAN node to report E-CID measurements used by E-SMLC to compute the location of the UE.

#### 8.2.1.2	Successful Operation

Figure 8.2.1.2-1: E-CID Measurement Initiation procedure, successful operation

The E-SMLC initiates the procedure by sending an E-CID MEASUREMENT INITIATION REQUEST message. If the NG-RAN node is able to initiate the requested E-CID measurements, it shall reply with the E-CID MEASUREMENT INITIATION RESPONSE message.

The Measured Results IE shall be included in the E-CID Measurement Result IE of the E-CID MEASUREMENT INITIATION RESPONSE message when measurement results other than the "Cell-ID" have been requested.

If the Report Characteristics IE is set to "OnDemand", the eNB shall return the result of the measurement in the E-CID MEASUREMENT INITIATION RESPONSE message including, if available, the E-UTRAN Access Point Position IE in the E-CID Measurement Result IE, and the E-SMLC shall consider that the E-CID measurements for the UE has been terminated by the eNB. If available, the eNB shall include the Cell Portion ID IE in the E-CID MEASUREMENT INITIATION RESPONSE message. Upon reception of the Cell Portion ID IE, the E-SMLC may use the value as the cell portion for the measurement.

Editor’s note: TP from LPPa not checked yet - FFS

If the Report Characteristics IE is set to "OnDemand" and the Inter-RAT Measurement Quantities IE is included in the E-CID MEASUREMENT INITIATION REQUEST message, the NG-RAN node, i.e. the ng-eNB, shall, if supported, provide the corresponding measurements, if available in the NG-RAN node, i.e. the ng-eNB, in the Inter-RAT Measurement Result IE in E-CID MEASUREMENT INITIATION RESPONSE message.

If the Report Characteristics IE is set to "OnDemand" and the WLAN Measurement Quantities IE is included in the E-CID MEASUREMENT INITIATION REQUEST message, the NG-RAN node, i.e. the ng-eNB, shall, if supported, provide the corresponding measurements, if available in the NG-RAN node, i.e. the ng-eNB, in the WLAN Measurement Result IE in E-CID MEASUREMENT INITIATION RESPONSE message.

#### If the Report Characteristics IE is set to "Periodic", the eNB shall initiate the requested measurements and shall reply with the E-CID MEASUREMENT INITIATION RESPONSE message without including either the E-CID Measurement Result IE or the Cell Portion ID IE in this message. The eNB shall then periodically initiate the E-CID Measurement Report procedure for the measurements, with the requested reporting periodicity.

Editor’s note: TP from LPPa not checked yet - FFS

#### 8.2.1.3	Unsuccessful Operation

Figure 8.2.1.3-1: E-CID Measurement Initiation procedure, unsuccessful operation

If the NG-RAN node is not able to initiate at least one of the requested E-CID measurements, the NG-RAN node shall respond with an E-CID MEASUREMENT INITIATION FAILURE message.

### 8.2.2	E-CID Measurement Failure Indication

#### 8.2.2.1	General

The purpose of the E-CID Measurement Failure Indication procedure is for the NG-RAN node to notify the E-SMLC that the E-CID measurements previously requested with the E-CID Measurement Initiation procedure can no longer be reported.

#### 8.2.2.2	Successful Operation

Figure 8.2.2.2-1: E-CID Measurement Failure Indication, successful operation

Upon reception of the E-CID MEASUREMENT FAILURE INDICATION message, the E-SMLC shall consider that the E-CID measurements for the UE have been terminated by the NG-RAN node.

#### 8.2.2.3	Unsuccessful Operation

Not applicable.

### 8.2.3	E-CID Measurement Report

#### 8.2.3.1	General

The purpose of E-CID Measurement Report procedure is for the NG-RAN node to provide the E-CID measurements for the UE to the E-SMLC.

#### 8.2.3.2	Successful Operation

Figure 8.2.3.2-1: E-CID Measurement Report procedure, successful operation

The NG-RAN node initiates the procedure by sending an E-CID MEASUREMENT REPORT message. The E-CID MEASUREMENT REPORT message contains the E-CID measurement results according to the measurement configuration in the respective E-CID MEASUREMENT INITIATION REQUEST message.

The Measured Results IE shall be included in the E-CID Measurement Result IE of the E-CID MEASUREMENT REPORT message when measurement results other than the "Cell-ID" have been requested.

If available, the eNB shall include the E-UTRAN Access Point Position IE which is the configured estimated serving antenna position in the E-CID Measurement Result IE within the E-CID MEASUREMENT REPORT message. Upon reception of this E-UTRAN Access Point Position IE, the E-SMLC may use the value as the geographical position of the E-UTRAN access point.

If available, the eNB shall include the Cell Portion ID IE in the E-CID MEASUREMENT REPORT message. Upon reception of the Cell Portion ID IE, the E-SMLC may use the value as the cell portion for the measurement.

Editor’s note: TP from LPPa note checked - FFS

#### 8.2.3.3	Unsuccessful Operation

Not applicable.

### 8.2.4	E-CID Measurement Termination

#### 8.2.4.1	General

The purpose of E-CID Measurement Termination procedure is to terminate periodical E-CID measurements for the UE performed by the NG-RAN node.

#### 8.2.4.2	Successful Operation

Figure 8.2.4.2-1: E-CID Measurement Termination procedure, successful operation

The E-SMLC initiates the procedure by generating an E-CID MEASUREMENT TERMINATION COMMAND message.

#### 8.2.4.3	Unsuccessful Operation

Not applicable.

### 8.2.x	OTDOA Information Exchange

#### 8.2.x.1	General

The purpose of the OTDOA Information Exchange procedure is to allow the E-SMLC to request the NG-RAN node, i.e. the ng-eNB, to transfer OTDOA information to the E-SMLC.

#### 8.2.x.2	Successful Operation

Figure 8.2.5.2-1: OTDOA Information Exchange procedure, successful operation

The E-SMLC initiates the procedure by sending an OTDOA INFORMATION REQUEST message. The NG-RAN node, i.e. the ng-eNB, responds with OTDOA INFORMATION RESPONSE message that contains the available OTDOA information applicable to the relevant cells/TPs.

#### 8.2.5.3	Unsuccessful Operation

Figure 8.2.5.3-1: OTDOA Information Exchange procedure, unsuccessful operation

If the NG-RAN node, i.e. the ng-eNB, does not have any OTDOA information to report, the NG-RAN node, i.e. the ng-eNB, shall respond with an OTDOA INFORMATION FAILURE message.

## 8.3	Management Procedures

#### 8.3.1	Error Indication

#### 8.3.1.1	General

The Error Indication procedure is initiated by a node to report detected errors in one incoming message, provided they cannot be reported by an appropriate failure message.

#### 8.3.1.2	Successful Operation

# Figure 8.3.1.2-1: Error Indication procedure, E-SMLC originated, successful operation

# Figure 8.3.1.2-2: Error Indication procedure, NG-RAN node originated, successful operation

When the conditions defined in clause 10 are fulfilled, the Error Indication procedure is initiated by an ERROR INDICATION message sent from the receiving node.

The ERROR INDICATION message shall contain at least either the Cause IE or the Criticality Diagnostics IE.

#### 8.3.1.3	Abnormal Conditions

Not applicable.

# 9	Elements for NRPPa Communication

Editor’s Note: Intention is that this section is based on section 9 in 36.455.

Sub clauses 9.1 and 9.2 describe the structure of the messages and information elements required for the NRPPa protocol in tabular format. Sub clause 9.3 provides the corresponding ASN.1 definition.

The following attributes are used for the tabular description of the messages and information elements: Presence, Range Criticality and Assigned Criticality. Their definition and use can be found in TS 38.413 [2].

NOTE:	The messages have been defined in accordance to the guidelines specified in TR 25.921 [x].

### 9.1.x	Messages for Location Information Transfer Procedures

#### 9.1.x.y	E-CID MEASUREMENT INITIATION REQUEST

This message is sent by E-SMLC to initiate E-CID measurements.

Direction: E-SMLC  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.x |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.x |  | - |  |
| E-SMLC Measurement ID | M |  | INTEGER(1..15,…) |  | YES | reject |
| Report Characteristics | M |  | ENUMERATED(OnDemand, Periodic,…) |  | YES | reject |
| Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED(120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, 60min,…) |  | YES | reject |
| Measurement Quantities |  | 1 .. <maxnoMeas> |  |  | EACH | reject |
| >Measurement Quantities Item | M |  | ENUMERATED (Cell-ID, Angle of Arrival, Timing Advance Type 1, Timing Advance Type 2, RSRP, RSRQ,…) | FFS | - | - |
| Inter-RAT Measurement Quantities |  | 0 .. <maxnoMeas> |  |  | EACH | ignore |
| >Inter-RAT Measurement Quantities Item | M |  | ENUMERATED(GERAN, UTRAN ,…) |  |  |  |
| WLAN Measurement Quantities |  | 0 .. <maxnoMeas> |  |  | EACH | ignore |
| >WLAN Measurement Quantities Item | M |  | ENUMERATED (WLAN, ...) |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is 63. |

| Condition | Explanation |
| --- | --- |
| ifReportCharacteristicsPeriodic | This IE shall be present if the Report Characteristics IE is set to the value “Periodic”. |

Editor’s Note: It needs to be clarified if all Measurement Quantities Item IE are applicable for NR i.e. could be shared by E-UTRA and NR

#### 9.1.x.y	E-CID MEASUREMENT INITIATION RESPONSE

This message is sent by eNB to indicate that the requested E-CID measurement is successfully initiated.

Direction: NG-RAN node  E-SMLC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.x |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.x |  | - |  |
| E-SMLC UE Measurement ID | M |  | INTEGER(1..15,…) |  | YES | reject |
| NG-RAN node UE Measurement ID | M |  | INTEGER(1..15,…) |  | YES | reject |
| E-CID Measurement Result | O |  | 9.2.x |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.x |  | YES | ignore |
| Cell Portion ID | O |  | 9.2.x |  | YES | ignore |
| Inter-RAT Measurement Result | O |  | 9.2.x |  | YES | ignore |
| WLAN Measurement Result | O |  | 9.2.x |  | YES | ignore |

#### 9.1.x.y	E-CID MEASUREMENT INITIATION FAILURE

This message is sent by NG-RAN node to indicate that the requested E-CID measurement cannot be initiated.

Direction: NG-RAN node  E-SMLC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.x |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.x |  | - |  |
| E-SMLC UE Measurement ID | M |  | INTEGER(1..15,…) |  | YES | reject |
| Cause | M |  | 9.2.x |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.x |  | YES | ignore |

#### 9.1.x.y	E-CID MEASUREMENT FAILURE INDICATION

This message is sent by NG-RAN node to indicate that the previously requested E-CID measurement can no longer be reported.

Direction: NG-RAN node  E-SMLC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.x |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.x |  | - |  |
| E-SMLC UE Measurement ID | M |  | INTEGER(1..15,…) |  | YES | reject |
| NG-RAN node UE Measurement ID | M |  | INTEGER(1..15,…) |  | YES | reject |
| Cause | M |  | 9.2.x |  | YES | ignore |

#### 9.1.x.y	E-CID MEASUREMENT REPORT

This message is sent by NG-RAN node to report the results of the requested E-CID measurement.

Direction: NG-RAN node  E-SMLC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.x |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.x |  | - |  |
| E-SMLC UE Measurement ID | M |  | INTEGER(1..15,…) |  | YES | reject |
| NG-RAN node UE Measurement ID | M |  | INTEGER(1..15,…) |  | YES | reject |
| E-CID Measurement Result | M |  | 9.2.x |  | YES | ignore |
| Cell Portion ID | O |  | 9.2.x |  | YES | ignore |

#### 9.1.x.y	E-CID MEASUREMENT TERMINATION COMMAND

This message is sent by the E-SMLC to terminate the requested E-CID measurement.

Direction: E-SMLC  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.x |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.x |  | - |  |
| E-SMLC UE Measurement ID | M |  | INTEGER(1..15,…) |  | YES | reject |
| NG-RAN node UE Measurement ID | M |  | INTEGER(1..15,…) |  | YES | reject |

#### 9.1.x.y	OTDOA INFORMATION REQUEST

This message is sent by E-SMLC to request OTDOA information.

Direction: E-SMLC  NG-RAN node, i.e. the ng-eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.x |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.x |  | - |  |
| OTDOA Information Type |  | 1 .. <maxnoOTDOAtypes> |  |  | EACH | reject |
| >OTDOA Information Item | M |  | ENUMERATED (pci, cellid, tac, earfcn, prsBandwidth, prsConfigIndex, cpLength, noDlFrames, noAntennaPorts, sFNInitTime, …, e-UTRANAccessPointPosition, prsmutingconfiguration, prsid, tpid, tpType, crsCPlength, MBSFNsubframeConfiguration, nPRSConfiguration, offsetNBChanneltoEARFCN, operationModeInfo, NPRS-ID, dlBandwidth, multipleprsConfigurationsperCell, prsOccasionGroup, prsFrequencyHoppingConfiguration, repetitionNumberofSIB1-NB) |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoOTDOAtypes | Maximum no. of OTDOA information types that can be requested and reported with one message. Value is 63. |

#### 9.1.1.8	OTDOA INFORMATION RESPONSE

This message is sent by NG-RAN node, i.e. the ng-eNB, to provide OTDOA information.

Direction: NG-RAN node, i.e. the ng-eNB  E-SMLC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.x |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.x |  | - |  |
| OTDOA Cells |  | 1 .. <maxCellineNB> |  | Served cells/TPs that broadcast PRS. May be used to signal multiple PRS configurations per cell (up to 3 are supported in this release). | GLOBAL | ignore |
| >OTDOA Cell Information | M |  | 9.2.x |  | - | - |
| Criticality Diagnostics | O |  | 9.2.x |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is FFS. |

Editor’s Note: the maxCellineNB is FFS previously 256 extended to 3840

#### 9.1.x.y	OTDOA INFORMATION FAILURE

This message is sent by NG-RAN node, i.e. the ng-eNB, to indicate that the OTDOA information cannot be provided.

Direction: NG-RAN node, i.e. the ng-eNB  E-SMLC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.x |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.x |  | - |  |
| Cause | M |  | 9.2.x |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.x |  | YES | ignore |

### 9.1.z	Messages for Management Procedures

#### 9.1.z.y	ERROR INDICATION

This message is used to indicate that some error has been detected in the NG-RAN node or in the E-SMLC.

Direction: E-SMLC  NG-RAN node and NG-RAN node  E-SMLC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.x |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.x |  | – |  |
| Cause | O |  | 9.2.x |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.x |  | YES | ignore |

## 9.U	Information Element definitions

### 9.U.v	General

When specifying information elements which are to be represented by bit strings, if not otherwise specifically stated in the semantics description of the concerned IE or elsewhere, the following principle applies with regards to the ordering of bits:

-	The first bit (leftmost bit) contains the most significant bit (MSB);

-	The last bit (rightmost bit) contains the least significant bit (LSB);

-	When importing bit strings from other specifications, the first bit of the bit string contains the first bit of the concerned information.

### 9.U.v	Cause

The purpose of the cause information element is to indicate the reason for a particular event for the whole protocol.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Cause Group | M |  |  |  |
| >Radio Network Layer |  |  |  |  |
| >>Radio Network Layer Cause | M |  | ENUMERATED (Unspecified, Requested Item not Supported, Requested Item Temporarily not Available, ... ) |  |
| >Protocol |  |  |  |  |
| >>Protocol Cause | M |  | ENUMERATED (Transfer Syntax Error, Abstract Syntax Error (Reject), Abstract Syntax Error (Ignore and Notify), Message not Compatible with Receiver State, Semantic Error, Unspecified, Abstract Syntax Error (Falsely Constructed Message),...) |  |
| >Misc |  |  |  |  |
| >>Miscellaneous Cause | M |  | ENUMERATED (Unspecified, ...) |  |

The meaning of the different cause values is described in the following table. In general, "not supported" cause values indicate that the concerned capability is missing. On the other hand, "not available" cause values indicate that the concerned capability is present, but insufficient resources were available to perform the requested action.

| Radio Network Layer cause | Meaning |
| --- | --- |
| Unspecified | Sent when none of the above cause values applies but still the cause is Radio Network Layer related |
| Requested Item not Supported | The NG-RAN node does not support the requested measurement object, or cannot provide the requested information item. |
| Requested Item Temporarily not Available | The NG-RAN node can temporarily not provide the requested measurement object or information item. |

| Protocol cause | Meaning |
| --- | --- |
| Abstract Syntax Error (Reject) | The received message included an abstract syntax error and the concerned criticality indicated "reject" (see sub clause 10.3) |
| Abstract Syntax Error (Ignore and Notify) | The received message included an abstract syntax error and the concerned criticality indicated "ignore and notify" (see sub clause 10.3) |
| Abstract syntax error (falsely constructed message) | The received message contained IEs or IE groups in wrong order or with too many occurrences (see sub clause 10.3) |
| Message not Compatible with Receiver State | The received message was not compatible with the receiver state (see sub clause 10.4) |
| Semantic Error | The received message included a semantic error (see sub clause 10.4) |
| Transfer Syntax Error | The received message included a transfer syntax error (see sub clause 10.2) |
| Unspecified | Sent when none of the above cause values applies but still the cause is Protocol related |

| Miscellaneous cause | Meaning |
| --- | --- |
| Unspecified | Sent when none of the above cause values applies and the cause is not related to any of the categories Radio Network Layer, Transport Network Layer or Protocol. |

### 9.U.v	Criticality Diagnostics

The Criticality Diagnostics IE is sent by the NG-RAN node or E-SMLC when parts of a received message have not been comprehended or were missing, or if the message contained logical errors. When applicable, it contains information about which IEs were not comprehended or were missing. The conditions for inclusion of the NRPPa Transaction ID IE are described in clause 10.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Procedure Code | O |  | INTEGER (0..255) | Procedure Code is to be used if Criticality Diagnostics is part of Error Indication procedure, and not within the response message of the same procedure that caused the error. |
| Triggering Message | O |  | ENUMERATED (initiating message, successful outcome, unsuccessful outcome) | The Triggering Message is used only if the Criticality Diagnostics is part of Error Indication procedure. |
| Procedure Criticality | O |  | ENUMERATED (reject, ignore, notify) | This Procedure Criticality is used for reporting the Criticality of the Triggering message (Procedure). |
| NRPPa Transaction ID | O |  | 9.2.4 |  |
| Information Element Criticality Diagnostics |  | 0 .. <maxNrOfErrors> |  |  |
| >IE Criticality | M |  | ENUMERATED (reject, ignore, notify) | The IE Criticality is used for reporting the criticality of the triggering IE. The value "ignore'" shall not be used. |
| >IE ID | M |  | INTEGER (0..65535) | The IE ID of the not understood or missing IE. |
| >Type Of Error | M |  | ENUMERATED (not understood, missing, …) |  |

| Range bound | Explanation |
| --- | --- |
| maxNrOfErrors | Maximum no. of IE errors allowed to be reported with a single message. The value for maxNroOfErrors is 256. |

### 9.U.v	Message Type

The Message Type IE uniquely identifies the message being sent. It is mandatory for all messages.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Procedure Code | M |  | INTEGER (0..255) |  |
| Type of Message | M |  | CHOICE (Initiating Message, Successful Outcome, Unsuccessful Outcome, …) |  |

### 9.2.4	NRPPa Transaction ID

The NRPPa Transaction ID IE is used to associate all the messages belonging to the same procedure. Messages belonging to the same procedure shall use the same NRLPPa Transaction ID.

The NRPPa Transaction ID is determined by the initiating peer of a procedure.

The NRPPa Transaction ID shall uniquely identify a procedure among all ongoing parallel procedures using the same procedure code, and initiated by the same protocol peer.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| NRPPa Transaction ID | M |  | INTEGER (0..32767) |  |

### 9.2.5	E-CID Measurement Result

The purpose of the E-CID Measurement Result information element is to provide the E-CID measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Serving Cell ID | M |  | FFS |  |
| >ECGI |  |  |  |  |
| >>ECGI |  |  | 9.2.x FFS | E-UTRA Cell Identifier of the serving cell |
| >NCGI |  |  |  |  |
| >>NCGI |  |  | FFS | NR Cell Identifier of the serving cell |
| Serving Cell TAC | M |  | OCTET STRING(2) | Tracking Area Code of the serving cell |
| RAN Access Point Position | O |  | 9.2.x FFS | The configured estimated geographical position of the antenna of the cell. |
| Measured Results |  | 0 .. <maxnoMeas> | FFS |  |
| >CHOICE Measured Results Value | M |  |  |  |
| >>Value Angle of Arrival | M |  | INTEGER (0..719) | According to mapping in TS 36.133 [8] |
| >>Value Timing Advance Type 1 | M |  | INTEGER (0..7690) | According to mapping in TS 36.133 [8] |
| >>Value Timing Advance Type 2 | M |  | INTEGER (0..7690) | According to mapping in TS 36.133 [8] |
| >>Result RSRP |  | 1 .. <maxCellReport> |  |  |
| >>>PCI | M |  | INTEGER (0..503, …) | Physical Cell Identifier of the reported cell |
| >>>EARFCN | M |  | INTEGER (0..65535, ..., 65536..262143) | Corresponds to NDL for FDD and NDL/UL for TDD in ref. TS 36.104 [5] |
| >>>ECGI | O |  | ECGI 9.2.x | E-UTRAN Cell Global Identifier of the reported cell |
| >>>Value RSRP | M |  | INTEGER(0..97, …) |  |
| >>Result RSRQ |  | 1 . <maxCellReport> |  |  |
| >>>PCI | M |  | INTEGER (0..503, …) | Physical Cell Identifier of the reported cell |
| >>>EARFCN | M |  | INTEGER (0..65535, ..., 65536..262143) | Corresponds to NDL for FDD and NDL/UL for TDD in ref. TS 36.104 [5] |
| >>>ECGI | O |  | ECGI 9.2.x | E-UTRAN Cell Global Identifier of the reported cell |
| >>>Value RSRQ | M |  | INTEGER(0..34, …) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is 63. |
| maxCellReport | Maximum no. of cells that can be reported with one message. Value is 9. |

Editor’s Note: the encoding and the NR part of the Serving Cell ID is FFS

Editor’s Note: It needs to be checked if the RAN Access Point Position IE has common encoding for NR and E-UTRA

Editor’s Note: It needs to be discussed if the Measured Results IE is dedicated to E-UTRA or could be encoded with NR e.g. PCI is dedicated to E-UTRA but other measurement may be common FFS

### 9.2.x	ECGI (FFS)

The E-UTRA Cell Global Identifier (ECGI) is used to globally identify a cell.

Editor’s Note: value is FFS pending to the progress in 38.413

### 9.2.x	NCGI (FFS)

The NR Cell Global Identifier (NCGI) is used to globally identify a cell.

Editor’s Note: value is FFS pending to the progress in 38.413

### 9.2.x	OTDOA Cell Information

This IE contains OTDOA information of a cell/TP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| OTDOA Cell Information |  | 1 .. <maxnoOTDOAtypes> |  |  |
| >CHOICE OTDOA Cell Information Item | M |  |  |  |
| >>PCI | M |  | INTEGER (0..503, …) | Physical Cell ID |
| >>Cell ID | M |  | ECGI 9.2.x |  |
| >>TAC | M |  | OCTET STRING(2) | Tracking Area Code |
| >>EARFCN | M |  | INTEGER (0..65535, ..., 65536..262143) | Corresponds to NDL for FDD and NDL/UL for TDD in ref. TS 36.104 [5] |
| >>PRS Bandwidth | M |  | ENUMERATED (bw6, bw15, bw25, bw50, bw75, bw100, ...) | Transmission bandwidth of PRS |
| >>PRS Configuration Index | M |  | INTEGER (0..4095, ...) | PRS Configuration Index, ref TS 36.211 [6] |
| >>CP Length | M |  | ENUMERATED (Normal, Extended,...) | Cyclic prefix length of the PRS |
| >>Number of DL Frames | M |  | ENUMERATED (sf1, sf2, sf4, sf6,…) | Number of consecutive downlink subframes NPRS with PRS, ref TS 36.211 [6] |
| >>Number of Antenna Ports | M |  | ENUMERATED(n1-or-n2, n4,…) | Number of used antenna ports, where n1-or-n2 corresponds to 1 or 2 ports, n4 corresponds to 4 ports |
| >>SFN Initialisation Time | M |  | BIT STRING (64) | Time in seconds relative to 00:00:00 on 1 January 1900 (calculated as continuous time without leap seconds and traceable to a common time reference) where binary encoding of the integer part is in the first 32 bits and binary encoding of the fraction part in the last 32 bits. The fraction part is expressed with a granularity of 1 /2**32 second. |
| >RAN Access Point Position | M |  | 9.2.x | The configured estimated geographical position of the antenna of the cell/TP. |
| >>PRS Muting Configuration | M |  | 9.2.x | The configuration of positioning reference signals muting pattern. |
| >>PRS-ID | M |  | INTEGER (0..4095,…) | PRS ID, ref TS 36.211 [6]. |
| >>TP-ID | M |  | INTEGER (0..4095,…) | Identity of the transmission point. This IE together with the PCI and/or PRS-ID may be used to identify the transmission point in case the same physical cell ID is shared by multiple transmission points. |
| >>TP Type | M |  | ENUMERATED (prs-only-tp, …) | A TP which transmits PRS only. |
| >>Number of DL Frames-Extended | M |  | INTEGER (1..160,…) | Number of consecutive downlink subframes NPRS with PRS, ref TS 36.211 [6]. |
| >>CRS CP Length | M |  | ENUMERATED (Normal, Extended,...) | Cyclic prefix length of the CRS. |
| >>MBSFN subframe Configuration | M |  | 9.2.14 | The MBSFN subframe configuration. |
| >>NPRS configuration | M |  | 9.2.16 | The NPRS configuration. |
| >>Offset of NB-IoT Channel Number to DL EARFCN | M |  | Offset of NB-IoT Channel Number to EARFCN 9.2.18 | Corresponds to MDL in TS 36.104 [5] |
| >>operationModeInfo | M |  | ENUMERATED (inband, guardband, standalone,...) |  |
| >>NPRS-ID | M |  | INTEGER (0..4095,…) | NPRS ID, ref TS 36.211 [6]. |
| >>DL Bandwidth | M |  | ENUMERATED (bw6, bw15, bw25, bw50, bw75, bw100, ...) | DL transmission bandwidth expressed in units of resource blocks NRB, ref TS 36.104 [5]. |
| >>PRS Occasion Group | M |  | ENUMERATED (og2, og4, og8, og16, og32, og64, og128, ...) | PRS occasion group in a PRS period, ref TS 36.211 [6]. |
| >>PRS Frequency Hopping Configuration | M |  | 9.2.19 | PRS frequency hopping configuration. |
| >>Repetition Number of SIB1-NB | M |  | ENUMERATED (r4, r8, r16, ...) | Repetition Number of SIB1-NB, refer to TS36.213 [12]. Value r4 corresponds to 4 repetitions, r8 to 8 repetitions, and r16 to 16 repetitions. |

| Range bound | Explanation |
| --- | --- |
| maxnoOTDOAtypes | Maximum no. of OTDOA information types that can be requested and reported with one message. Value is 63. |

### 9.2.x	RAN Access Point Position

The RAN Access Point Position IE is used to identify the geographical position of an RAN Access Point. It is expressed as ellipsoid point with altitude and uncertainty ellipsoid according to TS 23.032 [x].

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Latitude Sign | M |  | ENUMERATED (North, South) |  |
| Degrees Of Latitude | M |  | INTEGER (0..223-1) | The IE value (N) is derived by this formula: N223 X /90  N+1 X being the latitude in degrees (0°.. 90°). |
| Degrees Of Longitude | M |  | INTEGER (-223..223-1) | The IE value (N) is derived by this formula: N224 X /360  N+1 X being the longitude in degrees (-180°..+180°). |
| Direction of Altitude | M |  | ENUMERATED (Height, Depth) |  |
| Altitude | M |  | INTEGER (0..215-1) | The relation between the value (N) and the altitude (a) in meters it describes is N a < N+1, except for N=215-1 for which the range is extended to include all greater values of (a). |
| Uncertainty semi-major | M |  | INTEGER (0..127) | The uncertainty "r" is derived from the "uncertainty code" k by r = 10x(1.1k-1). |
| Uncertainty semi-minor | M |  | INTEGER (0..127) | The uncertainty "r" is derived from the "uncertainty code" k by r = 10x(1.1k-1). |
| Orientation of major axis | M |  | INTEGER (0..179) |  |
| Uncertainty Altitude | M |  | INTEGER (0..127) | The uncertainty altitude “h” expressed in metres is derived from the “uncertainty code” k, by: h=45x(1.025k-1). |
| Confidence | M |  | INTEGER(0..100) | In percentage |

### 9.2.x	PRS Muting Configuration

The PRS Muting Configuration IE is used to describe the configuration of PRS muting patterns for the concerned cell/TP, according to TS 36.211 [x] and TS 36.133 [x].

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE PRS Muting Configuration | M |  |  |  |
| >Two | M |  | BIT STRING (2) | If a bit is set to "0", it indicates that the PRS is muted in the corresponding PRS positioning occasion (numbering from any sub frame for which SFN=0) in a periodic cycle of length equal to the length of the bit string |
| >Four | M |  | BIT STRING (4) | Same as above |
| >Eight | M |  | BIT STRING (8) | Same as above |
| >Sixteen | M |  | BIT STRING (16) | Same as above |
| >thirty-two | M |  | BIT STRING (32) | Same as above |
| >sixty-four | M |  | BIT STRING (64) | Same as above |
| >one-hundred-and-twenty-eight | M |  | BIT STRING (128) | Same as above |
| >two-hundred-and-fifty-six | M |  | BIT STRING (256) | Same as above |
| >five-hundred-and-twelve | M |  | BIT STRING (512) | Same as above |
| >one-thousand-and-twenty-four | M |  | BIT STRING (1024) | Same as above |

### 9.2.x	Cell Portion ID

This parameter gives the current Cell Portion associated with the target UE. The Cell Portion ID is the unique identifier for a cell portion within a cell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Cell Portion ID | M |  | INTEGER (0..255, …, 256..4095) |  |

### 9.2.x	Inter-RAT Measurement Result

The purpose of the Inter-RAT Measurement Result information element is to provide the Inter-RAT measurement results.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Inter-RAT Measured Results |  | 1.. <maxnoMeas> |  |  |
| >CHOICE Inter-RAT Measured Results Value | M |  |  |  |
| >>Result GERAN | M | 1..<maxGERANMeas> |  |  |
| >>>ARFCN of BCCH | M |  | INTEGER (0..1023, ...) |  |
| >>>Physical CellId GERAN | M |  | INTEGER (0..63, ...) |  |
| >>>RSSI | M |  | INTEGER(0..63, ...) |  |
| >>Result UTRAN |  | 1..<maxUTRANMeas> |  |  |
| >>>UARFCN | M |  | INTEGER (0..16383, ...) |  |
| >>>CHOICE Physical CellId UTRA | M |  |  |  |
| >>>>Physical CellId UTRA FDD | M |  | INTEGER (0..511, ...) |  |
| >>>>Physical CellId UTRA TDD | M |  | INTEGER (0..127, ...) |  |
| >>>UTRA RSCP | O |  | INTEGER(-5..91, ...) |  |
| >>>UTRA EcNo | O |  | INTEGER(0..49, ...) | This IE applies to FDD only. |

| Range bound | Explanation |
| --- | --- |
| maxnoMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is 63. |
| maxGERANMeas | Maximum no. of GERAN cells that can be reported with one message. Value is 8. |
| maxUTRANMeas | Maximum no. of UTRAN cells that can be reported with one message. Value is 8. |

### 9.2.x	MBSFN subframe Configuration

The MBSFN subframe Configuration IE describes the MBSFN subframe configuration for the concerned cell/TP.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| MBSFN subframe Configuration Value |  | 1 .. < maxMBSFN-Allocations > |  |  |
| >Radio Frame Allocation Period | M |  | ENUMERATED (n1, n2, n4, n8, n16, n32) | Radio frame allocation period, ref TS 36.331 [10] |
| >Radio Frame Allocation Offset | M |  | INTEGER (0..7) | Radio frame allocation offset, ref TS 36.331 [x] |
| > CHOICE Subframe Allocation | M |  |  | Subframe allocation, ref TS 36.331 [x] |
| >>oneFrame | M |  | BIT STRING (SIZE(6)) |  |
| >>fourFrames | M |  | BIT STRING (SIZE(24)) |  |

| Range bound | Explanation |
| --- | --- |
| maxMBSFN-Allocations | Maximum number of MBSFN frame allocations with different offset as defined in TS36.331 [10]. Value is 8. |

### 9.2.15	WLAN Measurement Result

The WLAN Measurement Result information element provides the WLAN measurement results.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| WLAN Measured Results |  | 1.. <maxnoMeas> |  |  |
| >WLAN RSSI | M |  | INTEGER (0..141, ...) |  |
| >SSID | O |  | OCTET STRING (SIZE(1..32)) | Includes the SSID field as defined in subclause 8.4.2.2 of IEEE 802.11™ [11]. |
| >BSSID | M |  | OCTET STRING (SIZE(6)) | Includes the BSSID field as defined in subclause 8.2.4.3.4 of IEEE 802.11™ [11]. |
| >HESSID | O |  | OCTET STRING (SIZE(6)) | Includes the HESSID field as defined in subclause 8.4.2.94 of IEEE 802.11™ [11]. |
| >Operating Class | O |  | INTEGER (0..255) | Indicates the WLAN Operating Class as defined in IEEE 802.11™ [11]. |
| >Country Code |  |  | ENUMERATED (unitedStates, europe, japan, global, …) | Indicates the WLAN country code as defined in IEEE 802.11™ [11]. |
| >WLAN Channel List |  | 0..1 |  |  |
| >>WLAN Channel List Item |  | 1..<maxWLANchannels> |  |  |
| >>>WLAN Channel |  |  | INTEGER (0..255) | Indicates the WLAN channel number as defined in IEEE 802.11™ [11]. |
| >WLAN Band | O |  | ENUMERATED (band2dot4, band5, …) | Indicates the WLAN band as defined in IEEE 802.11™ [11]. |

| Range bound | Explanation |
| --- | --- |
| maxnoMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is 63. |
| maxWLANchannels | Maximum no. of WLAN channels that can be reported within one list. Value is 16. |

### 9.2.19	PRS Frequency Hopping Configuration

The PRS Frequency Hopping Configuration IE is used to describe the configuration of PRS frequency hopping for the concerned cell/TP, according to TS 36.211 [x].

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Number of Frequency Hopping Bands | M |  | ENUMERATED (twobands, fourbands, ...) | Number of bands for frequency hopping. |
| Band Positions |  | 1.. <maxnoFreqHoppingBandsMinusOne,...> |  |  |
| >NarrowBand Index | M |  | INTEGER (0..15, ...) | Narrowband Index |

| Range bound | Explanation |
| --- | --- |
| maxnoFreqHoppingBandsMinusOne | Maximum no. of frequency hopping bands minus one. Value is 7. |

# 10	Handling of unknown, unforeseen and erroneous protocol data

Editor’s Note: Intention is that this section is based on section 10 in 36.455.

# Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2017-08-23 | RAN3#97 | R3-173238 |  |  |  | TS skeleton agreed | v0.0.0 |
| 2017-08-25 | RAN3#97 | R3-173374 |  |  |  | TS 38.455 V0.1.0 | v0.1.0 |