3GPP TSG-RAN WG3 #97bis					R3-173876

Prague, Czech Republic, 9-13 October 2017

Agenda Item:	10.8.1.2

Source: 	Vodafone Group

Title: 	X2-AP Procedures Affected by Cell ID Length

Document for:	Discussion and Decision

# X2AP Procedures Affected by NR Cell ID Length and Variable Length gNB ID

# Introduction

In the last RAN3 meeting #97, a working agreement was reached to have 36 bits for the new NR gNodeB Cell ID.

In a Dual Connectivity scenario, such as Architecture Option 3X, where a Master eNodeB is connected with X2 interface to a secondary NR cell, X2-AP procedures, as described in detail in TS 36.423, are used for signalling and control from one node to the other, as shown in the following Figure 1:

Figure 1 Option 3X Architecture with X2 interface

# Cell ID Length Conflict

Currently the LTE Cell’s ID is 28 bits long and the proposed NR Cell ID would be 36 bits long. See Figure 2.

Figure 2 LTE and NR Cell ID Length

In our view a large number of X2-AP procedures and features that rely on the exchange of Cell ID between one node to another could be affected by the difference in Cell ID lengths and could potentially cause conflict in deployment.

# Procedure Affected by Cell ID length

The following table, which is an extract of TS 36.423, illustrates X2-AP procedures that could be affected by difference in Call ID lengths:

Table 1 X2-AP messages that carry ECGI and which are impacted by EN-DC

| Clause | Procedure | Direction | Description | Presence of ECGI in R14 message | Need to use NR cell ID instead of ECGI? | Need gNB ID instead of eNB ID |
| --- | --- | --- | --- | --- | --- | --- |
| 9.1.1.1 | HANDOVER REQUEST | eNB target eNB | When handing over a EN-DC configuration, the SgNB ID needs to be included | ECGI in this message is OK because the handover is between eNB master cells. | No | Yes |
| 9.1.2.1 | LOAD INFORMATION | eNB1 eNB2 | Id of the Source cell is used | Mandatory | Yes | No |
| 9.1.2.3 | X2 setup request | eNB1 eNB2 | E-UTRAN Cell Global Identifier of the neighbour cells are used | Mandatory | Yes | Yes |
| 9.1.2.4 | X2 SETUP RESPONSE | eNB2 eNB1 | E-UTRAN Cell Global Identifier of the neighbour cell is used | Mandatory | Yes | Yes |
| 9.1.2.8 | ENB CONFIGURATION UPDATE | eNB1 eNB2 | E-UTRAN Cell Global Identifier of the neighbour cell is used | Mandatory | Yes | No |
| 9.1.2.11 | RESOURCE STATUS REQUEST | eNB1 eNB2 | Cell to report Cell ID | Mandatory | Yes | No |
| 9.1.2.12 | RESOURCE STATUS RESPONSE | eNB2 eNB1 | Cell ID is used to request resource information | Mandatory | Yes | No |
| 9.1.2.13 | RESOURCE STATUS FAILURE | eNB2 eNB1 | Cell ID is used to report failure information | Mandatory | Yes | No |
| 9.1.2.14 | RESOURCE STATUS UPDATE | eNB2 eNB1 | Cell ID is used to obtain Cell update | Mandatory | Yes | No |
| 9.1.2.15 | MOBILITY CHANGE REQUEST | eNB1 eNB2 | Two cell IDs are used to request change in mobility | Mandatory | Yes | No |
| 9.1.2.16 | MOBILITY CHANGE ACKNOWLEDGE | eNB2 eNB1 | Two cell IDs are used to request change acknowledgement | Mandatory | Yes | No |
| 9.1.2.17 | MOBILITY CHANGE FAILURE | eNB2 eNB1 | Cell id are used to indicate failure | Mandatory | Yes | No |
| 9.1.2.18 | RLF INDICATION | eNB2 eNB1 | Cell ID is used to indicate failed cell | Mandatory | Yes | No |
| 9.1.2.19 | HANDOVER REPORT | eNB1 eNB2 | During handover, Cell ID of a failed cell is sent to the first eNodeB | Mandatory | Yes | No |
| 9.1.2.20 | CELL ACTIVATION REQUEST | eNB1 eNB2 | The ID of the cell to be activated is sent from one eNodeB to another | Mandatory | Yes | No |
| 9.1.2.21 | CELL ACTIVATION RESPONSE | eNB2 eNB1 | The activated cell ID is sent back to the first eNodeB | Mandatory | Yes | No |
| 9.1.2.23 | X2 RELEASE | eNB1 eNB2 | To indicate unavailability of signalling between two nodes | Mandatory | No | Yes |
| 9.1.2.25 | X2 REMOVAL RESPONSE | eNB1 eNB2 | Removal of signalling between two nodes | Mandatory | No | Yes |
| 9.1.3.x | Dual connectivity with SeNB | MeNB SeNB | New messages are defined for dual connectivity with SgNB | Mandatory | n/a | n/a |

Note: the impact of the contents of IE encodings in clause 9.2 of TS 36.423 still need to be checked (e.g. 9.2.8, 9.2.21, 9.2.68)

The conflict may arise when one node request cell information from another node.

As an example for a “Resource Status Request” procedure, clause 9.1.2.11, where the Master node instruct a NR node to perform a measurement of a particular cell, the ECGI or Cell ID of the target cell is sent to the NR node. In this situation, the Master cell may not be able to successfully resolved the NR Cell’s ID due to difference in Cell ID lengths, leading to operational problems.

# Proposals

Clearly it is very important that a solution to this issue is found. Some alternatives include:

- Revert the working assumption on NR Cell ID length and use the same structure(s) for NR Cell ID as for ECGI, or,

- Add extra IEs to the X2 messages to either carry the “extra bits” needed to construct the NR Cell ID, or,

- Add extra IEs to the X2 messages to carry the full NR Cell ID, and, define an EGCI (e.g. “all ones”) that means that the receiving node has to read the NR Cell ID and ignore the ECGI IE, or

- Define a whole set of new X2 messages that use can carry a mix of NR Cell IDs and ECGIs, or

- Mandate that NR cells used in EN-DC broadcast the NR Cell ID with a ‘null’ field in the middle placed so that a “ECGI” and “eNB ID” can be unambiguously derived from the NR Cell ID for use in X2 signalling, or,

- Etc.

In addition, the length of the gNB ID needs to be discussed

Suggested way forward:

- adopt an alternative “e” and liaise this information to (at least) RAN 2 and SA5.

# Conclusion

Based on the brief analysis presented in this contribution, we believe that having different Cell ID length between LTE and NR cells could cause conflict in control signalling and difficulties in deployment.

RAN3 is kindly asked to discuss this problem and in order to allow completion of the X2-AP stage 3 signalling in the next meeting, agree on a way forward at THIS meeting.