3GPP TSG RAN WG3 Meeting #97b	Tdoc R3-173997

Prague, Czech Republic, 09-13 October 2017

Agenda Item:	22.2

Source:	Ericsson, Vodafone, AT&T, Charter Communications Inc.

Title:	Idle to connected state transition with E1

Document for:	pCR for TR 38.806

# Introduction

In this contribution, we present the stage 2 call-flow for the state transition in a disaggregated gNB (CU-DU) architecture in which the CU is separated in a control plane part (CU-CP) and user plane part (CU-UP) connected via an E1 interface.

# Discussion

Below is a proposed call flow for IDLE to CONNECTED state transitions. The call flow is based on the current E-UTRAN/EPC call flow. All the internal steps in the 5GC are not shown.

Before proceeding with the description of the procedure, an observation that is relevant to some of the steps is that we assume that the CU-CP assigns the UL GTP TEID of the CU-UP. This is possible because we foresee that the CU-CP controls the operation of the CU-UP, which also implies that the CU-CP acquires detailed knowledge of the CU-UPs (including the UL GTP TEID) via configuration (during the E1 setup phase).

Observation 1	CU-CP allocates the UL GTP TEID of the CU-UP

The call-flow has the following steps:

1-2. UE performs random access procedure.

3. UE sends an RRC CONNECTION REQUEST. This message will at least include an CN UE temporary identifier.

4a. Network responds with a L2 contention resolution message. In the L2 message the content of the RRC CONNECTION REQUEST is echoed back to the UE. The DU does not need to decode the RRC message. This message can be sent before or after step 4b.

4b. The DU forwards the UE RRC message to the CU-CP in an F1-AP INITIAL UL RRC MESSAGE TRANSFER. This message includes additional information such as a F1-AP UE identifier (FFS), UE assigned C-RNTI and lower layer configuration.

NOTE:	The exact content of this message is FFS.

5. In case the CU-CP decide to accept the UE it will generate an RRC CONNECTION SETUP message and send this message in an F1-AP DL RRC TRANSFER message to the DU. In addition to the RRC message the F1-AP message contains an F1-AP UE identifier. The content of the RRC message may include information receive from the DU in previous step.

6-7. The RRC connection is setup over the radio.

8. The DU forwards the UE RRC message to the CU-CP. The forwarded RRC message may contain NAS information, information related to CN node selection and slicing information etc.

9. The CU-CP sends an NG-AP INTIAL UE MESSAGE to the 5GC. This message may contain initial NG-AP UE addressing information.

10. In case the 5GC decides to setup the UE context it will send an NG-AP INITIAL CONTEXT SETUP REQUEST message. This message contains among other UE radio access capabilities, default QoS information, UE security information etc. similar to the corresponding S1-AP message.

11. At this point the RAN receives the UE context and may initiate several procedures which some may happen in parallel.

11a-14a. These steps correspond to the security mode command procedure triggering the setup of UE security. From this point on subsequent radio signalling or data will be encrypted and the signalling will be integrity protected. These steps can be performed in parallel to 11b-14b.

11b-14b.  Involves setting up resources in the DU and CU-UP for the particular UE. In the call flow, it is shown that the CU-CP first send F1-AP UE CONTEXT SETUP REQUEST to the DU and after receiving the F1-AP UE CONTEXT SETUP RESPONSE the CU-CP setups up a bearer in the CU-UP. In this case it is assumed that the CU-CP is aware of what UL GTP TEID of the CU-UP so that this can be included in the F1-AP UE CONTEXT SETUP REQUEST. Step 13b-14b is only performed in case of a split CU deployment.

NOTE:	It is FFS in which order the F1 and E1 procedures should be performed or if they can be performed in parallel.

NOTE:	It is possible to merge steps 11a and 11b.

In step 11b the DU is provided with UE context information including UE radio access capabilities, UE DRB, UL GTP TEID and QoS related information. The DU then configures and allocates resources for the UE. In step 12b the lower layer configuration of the DRBs are provided to the CU-CP.

15. The CU generates an RRC CONNECTION RECONFIGURATION message and sends it down to the DU. This step need to be performed after 11b-12b since the RRC message contains lower layer configuration of the DRBs provided by the DU. Step 15 can happen before 14a and 14b.

16-17. RRC reconfiguration procedure. After this procedure, UL data transmission can start.

18. The DU encapsulates the RRC message in F1-AP UL RRC MESSAGE TRANSFER message and sends to the CU-CP.

19. The CU-CP acknowledges the context setup request from the 5GC. It also provides the 5GC with GTP TEIDs for the CU-UP.

# Conclusion

We conclude with the following proposals:

Proposal 1	RAN3 is kindly asked to agree with the proposed call-flow for the successful state transition from IDLE to CONNECTED.

Proposal 2	RAN3 is kindly asked to agree with the TP in Annex I.

# Annex I: TP for TR 38.806

Start of Text Proposal for TR 38.806

7.5.3 State transition

Below is the call flow for IDLE to CONNECTED state transition. The call flow is based on the current E-UTRAN/EPC call flow. All the internal steps in the 5GC are not shown.

Figure 7.5-1: Call flow for IDLE to CONNECTED state transition

The call-flow has the following steps:

1-2. UE performs random access procedure.

3. UE sends an RRC CONNECTION REQUEST. This message will at least include an CN UE temporary identifier.

4a. Network responds with a L2 contention resolution message. In the L2 message the content of the RRC CONNECTION REQUEST is echoed back to the UE. The DU does not need to decode the RRC message. This message can be sent before or after step 4b.

4b. The DU forwards the UE RRC message to the CU-CP in an F1-AP INITIAL UL RRC MESSAGE TRANSFER. This message includes additional information such as a F1-AP UE identifier (FFS), UE assigned C-RNTI and lower layer configuration.

NOTE:	The exact content of this message is FFS.

NOTE:	It is possible to merge steps 11a and 11b.

5. In case the CU-CP decides to accept the UE it will generate an RRC CONNECTION SETUP message and send this message in an F1-AP DL RRC TRANSFER message to the DU. In addition to the RRC message the F1-AP message contains an F1-AP UE identifier. The content of the RRC message may include information received from the DU in previous step.

6-7. The RRC connection is setup over the radio.

8. The DU forwards the UE RRC message to the CU-CP. The forwarded RRC message may contain NAS information, information related to CN node selection and slicing information etc.

9. The CU-CP sends an NG-AP INTIAL UE MESSAGE to the 5GC. This message may contain initial NG-AP UE addressing information.

10. In case the 5GC decides to setup the UE context it will send an NG-AP INITIAL CONTEXT SETUP REQUEST message. This message contains among other UE radio access capabilities, default QoS information, UE security information etc. similar to the corresponding S1-AP message.

11. At this point the RAN receives the UE context and may initiate several procedures which some may happen in parallel.

11a-14a. These steps correspond to the security mode command procedure triggering the setup of UE security. From this point on subsequent radio signalling or data will be encrypted and the signalling will be integrity protected. These steps can be performed in parallel to 11b-14b.

11b-14b.  Involves setting up resources in the DU and CU-UP for the particular UE. In the call flow, it is shown that the CU-CP first sends F1-AP UE CONTEXT SETUP REQUEST to the DU and after receiving the F1-AP UE CONTEXT SETUP RESPONSE the CU-CP setups up a bearer in the CU-UP. In this case it is assumed that the CU-CP is aware of what UL GTP TEID of the CU-UP so that this can be included in the F1-AP UE CONTEXT SETUP REQUEST.

NOTE:	It is FFS in which order the F1 and E1 procedures should be performed or if they can be performed in parallel.

NOTE:	It is possible to merge steps 11a and 11b.

In step 11b the DU is provided with UE context information including UE radio access capabilities, UE DRB, UL GTP TEID and QoS related information. The DU then configures and allocates resources for the UE. In step 12b the lower layer configuration of the DRBs are provided to the CU-CP.

15. The CU generates an RRC CONNECTION RECONFIGURATION message and sends it down to the DU. This step need to be performed after 11b-12b since the RRC message contains lower layer configuration of the DRBs provided by the DU. Step 15 can happen before 14a and 14b.

16-17. RRC reconfiguration procedure. After this procedure, UL data transmission can start.

18. The DU encapsulates the RRC message in F1-AP UL RRC MESSAGE TRANSFER message and sends to the CU-CP.

19. The CU-CP acknowledges the context setup request from the 5GC. It also provides the 5GC with GTP TEIDs for the CU-UP.

End of Text Proposal for TR 38.806