3GPP TSG-RAN Meeting #97-bis 	R3-174133

# Prague, Czechia, EU, 9-13 Oct 2017

Agenda item:	10.8.1.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Completion of the RRC tunnelling in MR-DC

Document for:	Decission

# 1	Introduction

At RAN3 meeting #97-bis, corrections to the RRC transfer has been discussed [1]. This document provides the necessary changes to the baseline CR to TS 36.423.

# References

- R3-174132, RAN3 #97-bis

# Text proposal for baseline CR for TS 36.423

First change, omitted text not changed

### 8.X.Y	RRC Transfer

#### 8.X.Y.1	General

The purpose of the RRC Transfer procedure is to deliver an LTE RRC message encapsulated in a PDCP-C PDU to the SgNB so that it may then be forwarded to the UE, or from the SgNB, if it was received from the UE. Delivery status may also be provided from the SgNB to the MeNB using the RRC Transfer.

The procedure is also to enable transfer of the NR RRC message container with the NR measurements from the MeNB to the SgNB, when received from the UE.

The procedure uses UE-associated signalling.

#### 8.X.Y.2	Successful Operation

Figure 8.X.Y.2-1: RRC Transfer procedure, successful operation.

The MeNB initiates the procedure by sending the RRC TRANSFER message to the SgNB or the SgNB initiates the procedure by sending the RRC TRANSFER message to the MeNB.

#### 8.X.Y.3	Abnormal Conditions

In case of the MCG split SRBs, the receiving node may ignore the message, if the MeNB has not indicated possibility of RRC transfer at the bearer setup.

Next change, omitted text not changed

#### 9.1.Z.1	RRC TRANSFER

This message is sent by the MeNB to the SgNB to transfer an RRC message.

Direction: MeNB  SgNB or SgNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB | YES | reject |
| SgNB UE X2AP ID | M |  | gNB UE X2AP ID 9.2.XX | Allocated at the SgNB | YES | reject |
| MCG split SRB |  | 0..1 |  |  |  |  |
| > RRC Container | M |  | OCTET STRING | RRC message encapsulated in a PDCP-C PDU and ciphered with the key of the MeNB | YES | reject |
| > SRB Type | M |  | ENUMERATED (srb1, srb2, ...) | The SRB type | YES | reject |
| > Delivery status | O |  | [FFS] |  | YES | reject |
| NR UE Measurement Report |  | 0..1 |  |  |  |  |
| > RRC Container | M |  | OCTET STRING | NR UE Measurement report (see TS 38.331 [X]) | YES | reject |

Next change, omitted text not changed

-- **************************************************************

--

#### -- RRC Transfer

--

-- **************************************************************

# RrcTransfer ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{{RrcTransfer-IEs}},

...

}

# RrcTransfer-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID						CRITICALITY ignore	TYPE UE-X2AP-ID					PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID						CRITICALITY ignore	TYPE UE-X2AP-ID					PRESENCE mandatory}|

{ ID id-MCGSplitSRB						CRITICALITY ignore	TYPE MCGSplitSRB				PRESENCE optional}|

{ ID id-UENRMeasurements								CRITICALITY ignore	TYPE UENRMeasurement					PRESENCE optional},

...

}

Next change, omitted text not changed

MCGSplitSRB ::= SEQUENCE {

rrcContainer 				RRCContainer,

srbType						SRBType,

deliveryStatus				[FFS],

iE-Extensions		ProtocolExtensionContainer { {MCGSplitSRB-ExtIEs} } OPTIONAL,

...

}

#### MCGSplitSRB-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

#### ...

#### }

UENRMeasurement ::= SEQUENCE {

uENRMeasurements			RRCContainer,

iE-Extensions		ProtocolExtensionContainer { {UENRMeasurement-ExtIEs} } OPTIONAL,

...

}

#### UENRMeasurement-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

#### ...

#### }

Remaining text not changed