3GPP TSG RAN WG3 Meeting #97bis	R3-174201

Prague, Czech Republic, 09 – 13 October 2017

Title:	LS on the need for EPS Bearer ID knowledge in NG-RAN for inter-system handover from 5GS to EPS

Release:	Rel-15

Work Item:	5GS_Ph1

Source:	RAN3

To:	SA2

Cc:

Contact person:

Name:			Lixiang Xu

E-mail Address:	lx.xu@samsung.com

Attachments:

# 1	Overall description

RAN3 has studied thoroughly the indirect data forwarding for inter system handover from 5GS to EPS and come to the following conclusions:

- SMF indicates to gNB at PDU session creation time the mapped E-RAB ID per QoS Flow for those PDU sessions which are subject to handover.

- At handover time, gNB makes the final decision for which E-RAB ID(s) it proposes data forwarding and builds the source -to-target container accordingly including the corresponding E-RAB ID(s) parameter.

- The target eNB can decide and assign TEID/TNL address for the E-RAB(s) for which it accepts the data forwarding.

- The data forwarding between gNB <-> UPF is per PDU session level. Then UPF maps the data received from one tunnel (for the PDU session) to the tunnel(s) corresponding to the mapped EPS bearer(s).

Therefore, RAN3 would like to kindly update SA2 that the NG-RAN needs to be aware of the mapped E-RAB IDs of the Qos flows to be configured before initiating the inter-system handover preparation procedure for handover to EPS.

# 2	Actions

To SA2:

ACTION: RAN3 kindly asks SA2 to take above into account and update stage 2 specifications if needed.

# 3	Dates of next TSG RAN WG3 meetings

RAN3#98	27th November – 1st December 2017	Reno, USA

RAN3#Adhoc	22nd – 26th January 2018		Sophia Antipolis, France