3GPP TSG RAN WG3 Meeting #97b	Tdoc R3-173991

Prague, Czech Republic, 09-13 October 2017

Agenda Item:	22.1

Source:	Ericsson

Title:	Introduction to Rapporteur Update TR 38.806

Document for:	pCR TR 38.806

# Introduction

This contribution provides a Rapporteur update to TR 38.806. We propose two updates to TR 38.806 [1].

The first update is related to the terminology. At the last meeting, we captured an FFS about the terminology to adopt in the study item:

“Editor’s note: Definition and naming of CU-C, CU-U and DU are FFS”.

We propose for the study item to maintain the current terminology that is used throughout TR 38.806: CU-CP, CU-UP and DU. The reason is that the title of the TR clearly indicates that the CP-UP split applies to split option 2 of NR. Therefore, we do not see any possible misunderstandings. We then propose to include the definitions of CU-CP, CU-UP and DU with reference to TS 38.401.

The second update is related to the cardinality between CU-CP and CU-UP. At the last meeting, we agreed on the following cardinality:

A gNB may consist of a CU-CP, multiple CU-UPs and multiple DUs;

- The CU-CP is connected to the DU through the F1-C interface;

- The CU-UP is connected to the DU through the F1-U interface;

- The CU-UP is connected to the CU-CP through the E1 interface;

- One DU is connected to only one CU-CP;

NOTE: for resiliency, a DU may be connected to multiple CU-CPs by appropriate implementation.

This text does not reflect the one-to-many cardinality between CU-CP and CU-UP that is shown in Figure 7-2-1. Therefore, we propose to capture this cardinality, using a terminology that is like the one that is currently used for the cardinality between CU-CP and DU:

- One CU-UP is connected to only one CU-CP;

NOTE: a physical UP implementation may include several logical CU-UP functions.

We then propose the following:

Proposal 1	RAN3 to agree with the following updates to TR 38.806:

    - Maintain the current terminology: CU-CP, CU-UP and DU;

    - Add the definitions of CU-CP, CU-UP and DU;

    - Describe the cardinality between CU-CP and CU-UP.

# References

TR 38.806 v0.1.0, Study on separation of CP and UP for split option 2 of NR