3GPP TSG-RAN WG3 Meeting #98	R3-174928

Reno, Nevada, USA, Nov. 27th – Dec.1st, 2017

Agenda item:		10.8.1.8

Source:	LG Electronics Inc.

Title:	Stage 2: TP on UE-AMBR for EN-DC

Document for:		Discussion

1. Introduction

This contribution provides the text proposal on UE-AMBR for EN-DC.

Text proposal for TS 37.340

************************* First change, the unchange text is skipped********************************

8.1	QoS aspects

In EN-DC, the E-UTRAN QoS framework defined in TS 36.300 [2] applies:

-	E-RAB (and concerning S1-U bearer) is established between the EPC and the SN for SCG bearers and SCG split bearers.

-	X2-U is established between the MN and the SN for MCG split bearers and SCG split bearers.

-	DRB is established between the SN and the UE for SCG bearers, MCG split bearers and SCG split bearers.

-	The MN decides the split of DL UE AMBR limits and UL UE AMBR limits among the MN and the SN, respectively, and indicates DL UE AMBR limits and UL UE AMBR limits to be respected by the SN.

-	The node that hosts the PDCP entity enforces the respective DL UE AMBR limits.

-	The node enforces the respective UL UE AMBR limits. For split bearers and SCG split bearers, if the node is not configured to serve the uplink, it ignores the indicated UL UE-AMBR.

In MR-DC with 5GC:

-	The NG-RAN QoS framework defined in TS 38.300 [3] applies.

-	QoS flows belonging to the same PDU session may be mapped to different bearer types (see subclause 4.2.2) and as a result there may be two different SDAP entities configured for the same PDU session: one at the MN and another one at the SN, in which case the MN decides which QoS flows are assigned to the SDAP entity in the SN.

-	The node that hosts an SDAP entity decides how to map QoS flows to DRBs.

-	If the SDAP entity is hosted by the MN and the MN decides that SCG resources are to be configured it provides QoS flow to DRB mapping information and the respective per QoS flow information to the SN.

-	If the SDAP entity is hosted by the SN, the MN provides sufficient QoS related information to enable the SN to configure appropriate SCG resources and to request the configuration of appropriate MCG resources. The MN may offer MCG resources to the SN and may indicate for GBR QoS flows the amount offered to the SN on a per QoS flow level.

To support PDU sessions mapped to different bearer types, MR-DC with 5GC provides the possibility for the MN to request the 5GC:

-	For some PDU sessions of a UE: Direct the User Plane traffic of the whole PDU session either to the MN or to the SN. In that case, there is a single NG-U tunnel termination at the NG-RAN for such PDU session.

-	The MN may request to change this assignment during the life time of the PDU session.

-	For some other PDU sessions of a UE: Direct the User Plane traffic of a subset of the QoS flows of the PDU session to the SN (respectively MN) while the rest of the QoS flows of the PDU session is directed to the MN (respectively SN). In that case, there are two NG-U tunnel terminations at the NG-RAN for such PDU session.

-	The MN may request to change this assignment during the life time of the PDU session.

Editor’s note: It is FFS whether the MN may request to change PDU session between single and two NG-U tunnel terminations at the NG-RAN.

************************* End of change********************************