3GPP TSG-RAN WG3 Meeting #99	R3-180749

# Athens, Grace, 26th Feburary- 2nd March  2018

Agenda Item:	10.16

Source:	Ericsson

Title:	Notification Control

Document for:	pCR

# 1	Discussion

The GBR QoS Flow parameter, “Notification Control” is an optional IE sent from CN to RAN, indicating that “whether notifications are requested from the RAN when the GFBR can no longer be fulfilled for a QoS flow during the lifetime of the QoS flow.”

Corresponding, in TS 38.413, the “PDU Session Resource Notify” has introduced. It is a class 2 procedure and uses the UE associated signalling.

In 5G DC, some QoS flows in the PDU session may be setup in another Node (e.g. SN). When a GBR QoS flow with “Notification Control” set is established in SN, for example, SN node needs to notify the MN node when the QoS requirement can no longer be fulfilled”. Thus a similar class 2 procedure is needed.

The same applies to F1 when there is a CU/DU separation. As it is often the lower layer (DU) who monitors if the QoS requirement is fulfilled or not.

Proposal 1: RAN3 to include a class 2 procedure in Xn, to indicate when the GBR QoS requirement can no longer be fulfilled or be fulfilled again.

Proposal 2: RAN3 to include a class 2 procedure in F1, to indicate from DU to CU when the GBR QoS requirement can no longer be fulfilled or be fulfilled again.

In DC 5G, there are a few different configurations, e.g.

- NG-U could either be terminated at MN, PDCP (and SDAP) protocol in SN + RLC/LCH configuration in MN

- NG-U could either be terminated at SN, PDCP (and SDAP) protocol in SN + RLC/LCH configuration in MN

- NG-U could either be terminated at MN, PDCP (and SDAP) protocol in MN + RLC/LCH configuration in SN

- NG-U could either be terminated at MN, PDCP (and SDAP) protocol in MN + RLC/LCH configuration in SN

That is to say that the Notify message is sometimes sent from MN node to SN node, and sometimes sent from SN node to MN node.

Proposal 3: The class 2 Notify procedure in Xn should be defined to be sent from on NG-RAN node to another NG-RAN node, decoupled from the MN node and SN node role.

Currently the “PDU Session Resource Notify” defined in TS 38.413 includes two different cases, one is when NG-RAN node desires to release certain QoS flows, another is when NG-RAN wants to notify the CN that the QoS requirement for certain QoS flow can no longer be fulfilled.

In our opinion, the procedure suits the purpose for notifying the CN, but is not suitable to be used to release the QoS flow. “The list of QoS flows which are released by the NG-RAN node” is included in the message, but are the QoS flows released? What RAN should do for these released QoS flows? TS 38.413 says that upon the reception of the message, “SMF normally initiate the appropriate release procedure”, in order to release those QoS flows.

From the above we can see that three NGAP messages are needed if to use the “PDU Session Resource Notify” to release the QoS flow. It is totally up to SMF to initiate a release procedure or not. But what would happen

- if for some reason that SMF does not initiate the release procedure?

- If SMF at the same time initiate a modification procedure for the involved QoS flows?

For sure when NG-RAN node wishes to release certain QoS flow, it means that RAN cannot keep the QoS flows any more. RAN needs to handle after having sent release in the “notify” message, but CN initiate “modification” for the QoS flows NG-RAN considered to be released, or there is no “release” coming from CN.

On the other hand, we do have a “PDU Session Resource Modify Indication” procedure initiated by NG-RAN Node.

If NG-RAN node uses the above procedure to release the QoS flows, it is only 2 step messages and there is no ambiguity. It is a very robust handling.

It would be much clearer and more robust if we use the PDU Session Resource Notify procedure to notify that the QoS requirement can no longer be fulfilled, or it is being fulfilled again, and it is CN to determine how to handle the related QoS flow. It is a class 2 message.

And we use the PDU Session Resource Modify Indication when NG-RAN decides that certain QoS flow are released. It is a class 1 message.

Proposal 4: RAN3 to agree to use “PDU Session Resource Notify” procedure only be used for Notification purpose, to indicate from NG-RAN node to 5GC when the GBR QoS requirement can no longer be fulfilled or be fulfilled again.

Proposal 5: RAN3 to agree to modify the “PDU Session Resource Modify Indication” procedure to handle the NG-RAN node initiated release of certain QoS flows or PDU sessions.

# 2	Proposals

Proposal 1: RAN3 to include a class 2 procedure in Xn, to indicate when the GBR QoS requirement can no longer be fulfilled or be fulfilled again.

Proposal 2: RAN3 to include a class 2 procedure in F1, to indicate from DU to CU when the GBR QoS requirement can no longer be fulfilled or be fulfilled again.

Proposal 3: The class 2 Notify procedure in Xn should be defined to be sent from on NG-RAN node to another NG-RAN node, decoupled from the MN node and SN node role.

Proposal 4: RAN3 to agree to use “PDU Session Resource Notify” procedure only be used for Notification purpose, to indicate from NG-RAN node to 5GC when the GBR QoS requirement can no longer be fulfilled or be fulfilled again.

Proposal 5: RAN3 to agree to modify the “PDU Session Resource Modify Indication” procedure to handle the NG-RAN node initiated release of certain QoS flows or PDU sessions.

pCRs are submitted in [1] as well.

# 3	Reference

[1]	R3-180380, Notification Control, Ericsson

# Text Proposal for NGAP TS 38.413 v0.5.0

<<<<<<<<<<<<<<<<<<<< Begin Text Proposal >>>>>>>>>>>>>>>>>>>>

### 8.2.4	PDU Session Resource Notify

#### 8.2.4.1	General

The purpose of the PDU Session Resource Notify procedure is to notify that the already established QoS flow(s) or PDU session(s) for a given UE are released or not fulfilled/fulfilled by the NG-RAN node for which a notification is issued. The procedure uses UE-associated signalling.

#### 8.2.4.2	Successful Operation

Figure 8.2.4.2-1: PDU session resource notify

The NG-RAN node initiates the procedure by sending a PDU SESSION RESOURCE NOTIFY message.

The PDU SESSION RESOURCE NOTIFY message shall contain the information of PDU sessions or QoS flows which are released or not fulfilled anymore or are/ fulfilled again by the NG-RAN node.

-	For each PDU session of which some QoS flows are not fulfilled any more / fulfilled again or released by the NG-RAN node, the PDU Session Resource Notify Transfer IE shall be included to report for each PDU session of which some QoS flow(s) are released or not fulfilled any more / fulfilled again [FFS]:

1. 	The list of QoS flows which are released by the NG-RAN node, if any, shall be included in the QoS Flows Release Notify List IE.

2. The list of GBR QoS flows which are not fulfilled any more or fulfilled again by the NG-RAN node, if any, shall be included in the QoS Flows Notify List IE.

-	For each PDU session which is not fulfilled any more or be fulfilled again by the NG-RAN node, the PDU Session Resource Notify Transfer IE shall be included to report cause in the Notification Cause IE.

-	For each PDU session which is released by the NG-RAN node, the PDU Session Resource Notify Transfer IE shall be included to report the release cause in the Cause IE.

Upon reception of the PDU SESSION RESOURCE NOTIFY message, the AMF shall, for each PDU session indicated in the PDU Session ID IE, transfer transparently the PDU Session Resource Notify Transfer IE to each SMF associated with the concerned PDU session. [FFS] Upon reception of PDU Session Resource Notify Transfer IE, SMF normally initiate the appropriate release or modify procedure on the core network side for the PDU session(s) or QoS flow(s) identified in the PDU SESSION RESOURCE NOTIFY message [FFS].

Editor’s Note:	Further details are FFS.

#### 8.2.4.3	Abnormal Conditions

Editor’s Note:	Further details are FFS.

------------------------------------------

Skip to next change

------------------------------------------

#### 9.2.1.7	PDU SESSION RESOURCE NOTIFY

Editor’s Note:	Message structure and IEs need further checking and completion. Further details FFS.

This message is sent by the NG-RAN node to notify that the already established QoS flow(s) or PDU session(s) for a given UE are released or not fulfilled any more / fulfilled again by the NG-RAN node.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| PDU Session Resource Notify List |  | 0..1 |  |  | YES | reject |
| >PDU Session Resource Notify Item IEs |  | 1..<maxnoofPDUSessions> |  |  | EACH | reject |
| >>PDU Session ID | M |  | <ref> |  | - |  |
| >>PDU Session Resource Notify Transfer | M |  | 9.3.1.18 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions allowed towards one UE. Value is FFS. |

#### 9.3.1.18	PDU Session Resource Notify Transfer

Editor’s Note: 	Further details FFS.

This IE is sent transparently by the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE PDU Session Resource Notify Transfer | M |  |  |  | YES | reject |
| >QoS Flows Notify |  |  |  |  | - |  |
| >>QoS Flows Notify List |  | 1..<maxnoofQoSFlows> |  |  | - |  |
| >>>QoS Flow Indicator | M |  | <ref> |  | EACH | reject |
| >>>Notification Cause | M |  | ENUMERATED(Fullfilled, Not Fulfilled, …) |  | - | - |
| >>>Assistance Information [FFS] | O |  | <ref> |  | - |  |
| >>QoS Flows Release List [FFS] | O |  | QoS Flow List <ref> |  | YES | ignore |
| >PDU Session Resource Release |  |  |  |  | - |  |
| >>Cause [FFS] | M |  | 9.3.1.2 |  | - |  |
| >PDU Session Resource Notify |  |  |  |  | - | - |
| >> Notification Cause | M |  | ENUMERATED(Fullfilled, Not Fulfilled, …) |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is FFS. |

<<<<<<<<<<<<<<<<<<<< End Text Proposal  >>>>>>>>>>>>>>>>>>>>