3GPP TSG RAN WG3 #99                                               	R3-181063

Athens, Greece, 26 Feb – 2 Mar, 2018

Source: 	KDDI Corporation

Title:	IAB architecture consideration on SA/NSA and LTE access link aspects

Tdoc Type:	Discussion

Agenda Item:	24. Study on Integrated Access and Backhaul for NR

Document for:	Discussion and decision

Release:	Rel-15

Introduction

In NR-AH1801, there was a discussion regarding NSA support and LTE access link support [1]. In this contribution, we list up possible scenarios for NSA support and LTE access link, and identify the impact to NR and LTE specification.

Discussion

  - Possible scenarios

Possible scenarios the following six cases (2 options * 3 options) should be considered.

- IAB node 2 options:  SA(without LTE capability) or NSA(with LTE capability)

- UE access 3 options: SA UE or NSA UE or LTE UE

|  | IAB Node | UE access |
| --- | --- | --- |
| Case 1 | SA operation (without LTE capability) | SA UE |
| Case 2 |  | NSA UE |
| Case 3 |  | LTE UE |
| Case 4 | NSA operation (with LTE capability) | SA UE |
| Case 5 |  | NSA UE |
| Case 6 |  | LTE UE |

Table 1: Possible scenarios for NSA and LTE access link

  - IAB node with SA operation

Figure 1 shows possible architectures for IAB node with SA operation (e.g. Case1~3).  This Figure also shows the interfaces between IAB nodes. For example, in case1, relay links (Brue lines) are used to convey NG-U/C, and Xn.NG-U/C is for SA UE, and Xn is for IAB nodes to exchange its neighbor NR cell lists, handover coordination or other information.

Figure 1 : Architectures for “IAB node with SA operation”

Figure 2 : Protocol stacks for “IAB node with SA operation”

Figure 2 shows its protocol stacks. Regarding NR uu interface between IAB node and IAB donor, there would be discussion for Layer 2 or Layer3, or CU/DU separation, these aspects can be discussed separately.

Proposal 1 :

For IAB nodes with SA operation to accommodate SA UE and NSA UE and LTE UE, NR backhaul link should be designed to carry the following interface data

- NG-U/C : interface between IAB nodes and NGC

- Xn : interface between IAB nodes and interface IAB nodes and IAB donor

- S1-U/C : interface between IAB nodes(which have eNB capability) and EPC

- X2 : interface between IAB nodes and LTE eNB

  - IAB node with NSA operation

Figure 3 shows possible architectures for IAB node with NSA operation (e.g. Case4~6).  Difference between SA operation and NSA operation is the LTE link between IAB node and LTE eNB (purple line in Figure 3). IAB node might bypass some traffic to LTE link (purple line). Considering LTE bandwidth is much narrower compared to NR bandwidth, U-plane related high capacity required traffic (NG-U, S1-U) might not be bypassed to LTE link. However C-plane related traffic (NG-C, Xn, S1-C, X2) can be bypassed to LTE link. LTE link tend to have lager coverage with lower frequency, so this bypass solution improve IAB robustness.

Observation 1 :

Considering IAB nodes with NSA operation, LTE link might be used to bypass the following C-plane related interface to improve IAB robustness

- NG-C : interface between IAB nodes and NGC

- Xn : interface between IAB nodes and interface IAB nodes and IAB donor

- S1-C : interface between IAB nodes(which have eNB capability) and EPC

- X2 : interface between IAB nodes(which have eNB capability)

Figure 4 shows a protocol stack how to bypass C-plane related traffic to LTE. In case where the number of NR hops is large, this bypass solution contributes shorter latency. However the current LTE specification has no functionality forwarding “data from one UE” to “another UE”. So, realizing this bypass solution requires some enhancement to the current LTE specification.

Observation 2 :

Realizing “the LTE bypass solution” requires some enhancement to the current LTE specification

Considering observation 1 and 2, we propose that

Proposal 2 :

For IAB nodes with SA operation, RAN2/3 study “LTE bypass solution” which enables forwarding data from one IAB node to another IAB node, evaluate its impact to LTE specification

Figure 3 Architectures for “IAB node with NSA operation”

Figure 4 Protocol stacks for “IAB node with NSA operation”

Proposal

In this contribution, we list up possible scenarios for NSA support and LTE access link, and discuss its architecture and protocol stacks, and propose that;

Proposal 1 :

For IAB nodes with SA operation to accommodate SA UE and NSA UE and LTE UE, NR backhaul link should be designed to carry the following interface data

- NG-U/C : interface between IAB nodes and NGC

- Xn : interface between IAB nodes and interface IAB nodes and IAB donor

- S1-U/C : interface between IAB nodes(which have eNB capability) and EPC

- X2 : interface between IAB nodes and LTE eNB

Proposal 2 :

For IAB nodes with SA operation, RAN2/3 study “LTE bypass solution” which enables forwarding data from one IAB node to another IAB node, evaluate its impact to LTE specification

References

[1] R2-1801621	Proposals on IAB Use Cases and Deployment Scenarios 	AT&T, Ericsson, Intel, KDDI, Nokia, Nokia Shanghai Bell, Qualcomm, Samsung