3GPP TSG-RAN WG3 #99   						                  R3-181077

Athens, Greece, 26th February – 2nd March 2018

Agenda item:	10.10.7

Source:	CMCC

Title:	(TP for SA BL CR) QoS handling over F1

Document for:	Discussion and Decision

Introduction

Discussion on QoS handling over F1 has spanned several meetings but no agreement was reached. At RAN3 AH 1801 meeting, online discussion cannot converge with a common understanding on the definition of DRB QoS configuration.  Offline discussion was scheduled and the summary was captured in [1]. In this contribution, we further discuss this issue starting from the outcome of offline discussion at last meeting and striving for an agreement.

Discussion

Offline discussion in [1] focused on two questions: 1) Whether CU provides to DU, QoS profile for each DRB and/or for each flow in a DRB; 2) Where QoS profile to be provided consists of 5QI, ARP, Non-standardized QoS flow descriptor, GBR QoS flow information (or parts thereof). Three alternatives were identified as below:

Alt.1: CU provides to DU, QoS profile for each flow

i.e., CU takes QoS profile info for each flow received over NG, and just forwards them as-is to DU

Alt.2: CU provides to DU, QoS profile for each DRB

i.e., CU takes QoS profile info for each flow received over NG, determines an “aggregate QoS profile” for each DRB, and forwards them to DU

Alt.3: CU provides to DU, QoS profile for each flow and DRB

i.e., Alt.1 plus Alt.2

It can be seen that QoS granularity of all alternatives at DU are DRBs, and there is only one “aggregate QoS profile” for a DRB. It seems this is a common understanding in RAN3. The only left issue is about who (CU and/or DU) determines the “aggregate QoS profile” for a DRB from the individual QoS profiles for each flows mapped to a DRB.

Observation 1: RAN3 has a common understanding on the QoS granularity at DU and definition of DRB-level profile.

Considering the fact that the mapping between QoS flows and radio bearers is performed by the gNB-CU, the granularity of data delivery over F1 is radio bearer. Generally, the QoS flow can be invisible to the gNB-DU. So it is reasonable for gNB-CU to perform QoS configuration and provide DRB QoS profile to gNB-DU. It is argued in [2] that if proposing gNB-CU to determine the QoS configuration for DRB, gNB-CU should be aware of a number of information, including DU processing resource status, DU resource prediction per QoS profile and DU implementation details, etc. In light of the observation, a statement was made in [2] that the inter-vendor interoperability can hardly be achieved if alternative 2 applies. However, the statement does not hold since what actually happens in CU is grouping a number of QoS flows with similar QoS types/profiles into one DRB and then CU decides an aggregated QoS profile which may consist of 5QI, ARP, Non-standardized QoS flow descriptor, GBR QoS flow information etc by taking into account the QoS requirement of each flow. It does not necessarily need to have the knowledge of DU information.

By contrast, if alternative 1 is applies, there is one key issue has to be addressed which we have raised in previous contribution but haven’t got answers yet. That is how CU to perform the QoS flow to DRB mapping provided that the “aggregated DRB profile” is determined by DU. Since gNB-CU is not aware of the QoS configuration, only blind or very coarse mapping between QoS flow to DRB can be performed at gNB-CU. In other words, CU will perform mapping from several QoS flows to a DRB, nevertheless, the profile of which it does not know at all. It may happen that CU maps a QoS flow to the DRB which does not tailored for implementing the QoS profile.

In light of above analysis, it should be gNB-DU to decide the “aggregate QoS profile” for a DRB”, gNB-DU could accept or reject the addition of a DRB based on “aggregate QoS profile” and its own resources.

Proposal 1: gNB-CU decides the “aggregate QoS profile” for a DRB”

Proposal 2: gNB-CU signals DRB level QoS configurations to the gNB-DU over F1, and then the gNB-DU follows the DRB level QoS profile or rejects the DRB.

As gNB-DU decides the DRB scheduling policy which guarantees QoS requirement, it may be beneficial to transfer the QoS profile for each flow and the mapping information from gNB-CU to gNB-DU to enable optimized scheduling. But this information can be set optional and treated as complementary information to the DRB level QoS configuration.

Proposal 3: QoS profile for each flow can be transferred from gNB-CU to gNB-DU as optional IE to enable efficient scheduling.

In summary, alternative 2 plus alternative 1 can be considered but alternative 2 should be mandatory while alternative 1 is optional. The corresponding TP to TS 38.473 SA BLCR is provided in the annex.

Conclusion

Based on the discussion on QoS handling alternatives, the following proposals are made:

Observation 1: RAN3 has a common understanding on the QoS granularity at DU and definition of DRB-level profile.

Proposal 1: gNB-CU decides the “aggregate QoS profile” for a DRB”

Proposal 2: gNB-CU signals DRB level QoS configurations to the gNB-DU over F1, and then the gNB-DU follows the DRB level QoS profile or rejects the DRB.

Proposal 3: QoS profile for each flow can be transferred from gNB-CU to gNB-DU as optional IE to enable efficient scheduling.

Reference

R3-180606, Summary of offline discussion on CU-DU QoS handling , NTT Docomo

R3-180518, UE context Setup over the F1, Ericsson

- Annex: Text Proposal for SA BLCR 38.473

---------------------------------------------------Start of the change--------------------------------------------

- 9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| Target cell ID (FFS) | FFS |  | FFS |  | FFS | FFS |
| PSCell ID | O |  | NCGI 9.3.1.12 | PSCell Identifier in SgNB | YES | Ignore |
| Cell Type [FFS] | M |  | FFS | Includes the cell type (PSCell, PCell, normal Cell) [FFS] | YES | reject |
| CU to DU RRC Information | M |  | 9.3.1.25 |  | YES | reject |
| DRX Cycle | O |  | DRX Cycle 9.3.1.24 |  | YES | ignore |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the MeNB Resource Coordination Information IE as defined in subclause 9.1.X.1 of TS 36.423 [9]. | YES | reject |
| SCell To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Setup Item IEs |  | 0 .. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NCGI 9.3.1.12 | SCell Identifier in SgNB | - | - |
| SRB to Be Setup List |  | 0..1 |  |  |  |  |
| >SRB to Be Setup Item IEs |  | 1 .. <maxnoofSRBs>] |  |  |  |  |
| >SRB ID | M |  | 9.3.1.7 |  |  |  |
| DRB to Be Setup List |  | 1 |  |  | YES | reject |
| >DRB to Be Setup Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>DRB QoS profile | M |  |  | Shall be used for gNB to convey DRB-Level QoS Parameters | - |  |
| >>Flow-level QoS profile | O |  |  |  |  |  |
| >>Flow-to-DRB mapping | O |  |  |  |  |  |
| >>E-UTRAN QoS | O |  | 9.3.1.19 | Shall be used for EN-DC case to convey E-RAB Level QoS Parameters |  |  |
| >> Tunnels to be setup List |  | 1 |  |  |  |  |
| >>>Tunnels to Be Setup Item IEs |  | 1 .. <maxnoofULTunnels> |  |  |  |  |
| >>>>UL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 64. |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofULTunnels | Maximum no. of tunnels allowed towards one DRB, the maximum value is 2. |

------------------------------------------------Start of the next change----------------------------------------

#### 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| PSCell ID | O |  | NCGI 9.3.1.12 | PSCell Identifier in SgNB | YES | Ignore |
| Cell Type [FFS] | M |  | FFS | Includes the cell type (PSCell, PCell, normal Cell) [FFS] | YES | reject |
| DRX Cycle | O |  | DRX Cycle 9.3.1.24 |  | YES | ignore |
| CU to DU RRC Information | O |  | 9.3.1.25 |  | YES | reject |
| Transmission Stop Indicator | O |  | 9.3.1.11 |  | YES | ignore |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the MeNB Resource Coordination Information IE as defined in subclause 9.2.116 of TS 36.423 [9]. | YES | reject |
| RRC-Container | O |  | 9.3.1.6 |  | YES | ignore |
| SCell To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Setup Item IEs |  | <0 .. maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NCGI 9.3.1.12 | SCell Identifier in SgNB | - | - |
| SRB to Be Setup List |  | 0..1 |  |  |  |  |
| >SRB to Be Setup Item IEs |  | <1..maxnoofSRBs> |  |  |  |  |
| >>SRB ID | M |  | 9.3.1.7 |  |  |  |
| DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Setup Item IEs |  | <1 .. maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>DRB QoS profile | M |  |  | Shall be used for gNB to convey DRB-Level QoS Parameters | - |  |
| >>Flow-level QoS profile | O |  |  |  |  |  |
| >>Flow-to-DRB mapping | O |  |  |  |  |  |
| >>E-UTRAN QoS | O |  | 9.3.1.19 | Shall be used for EN-DC case to convey E-RAB Level QoS Parameters |  |  |
| >> Tunnels to be setup List |  | 1 |  |  |  |  |
| >>Tunnels to Be Setup Item IEs |  | 1 .. <maxnoofULTunnels> |  |  |  |  |
| >>>UL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - | - |
| DRB to Be Modified List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Modified Item IEs |  | <1 .. maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>DRB QoS profile | M |  |  | Shall be used for gNB to convey DRB-Level QoS Parameters | - |  |
| >>Flow-level QoS profile | O |  |  |  |  |  |
| >>Flow-to-DRB mapping | O |  |  |  |  |  |
| >>E-UTRAN QoS | O |  | 9.3.1.19 | Shall be used for EN-DC case to convey E-RAB Level QoS Parameters |  |  |
| >>>Tunnels to Be Setup Item IEs |  | 1 .. <maxnoofULTunnels> |  |  |  |  |
| >>>>UL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - | - |
| SRB TO Be Released List |  | 0..1 |  |  |  |  |
| >SRB To Be Released Item IEs |  | <1..maxnoofSRBs> |  |  |  |  |
| >>SRB ID | M |  | 9.3.1.7 |  |  |  |
| DRB to Be Released List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Released Item IEs |  | <1 .. maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 64. |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofDLTunnels | Maximum no. of tunnels allowed towards one DRB, the maximum value is 2. |

-----------------------------------------------End of the change --------------------------------------------