3GPP TSG-RAN WG3 Meeting #99	R3-181239

# Athens, Grace, 26th Feburary- 2nd March  2018

Agenda Item:	10.5.2.1

Source:	Ericsson

Title:	Handling concurrent running of security procedures

Document for:	pCR

# 1	Discussion

At the last RAN3 meeting, LS on handling concurrent running of security procedures was received [1] and we have replied in [2] that:

RAN3 did not identify any issues with introducing signalling on Xn and NG to support synchronisation of AS and NAS security and will further work on the requested function in upcoming meetings.

RAN3 understood, that synchronisation of AS and NAS security context during HO may cause the NSCI (New Security Context Indicator) to be included NGAP HANDOVER REQUEST message (at NG HO) or the  NGAP PATH SWITCH REQUEST ACKNOWLEDGE message (at Xn HO).

This text proposal implements the change on NGAP to include NSCI as optional IE.

There is a new security specification TS 33.501, however in NGAP, we refer all the time to TS 33.401. A new reference is added to TS 33.501 in this text proposal. We need to check if the reference to TS 33.401 in the NGAP if is entirely correct and maybe should be refresh to TS 33.501, the 5G security spec.

Proposal 1: RAN3 to add the text proposal for handling the concurrent running of security procedures.

Proposal 2: RAN3 to check if reference to TS 33.401 is entirely correct, as there is a new security specification for 5G. (can be fixed by TS Rapporteur)

# 2	Reference

- [1]	R3-180006	LS on handling concurrent running of security procedures, SA3

- [2]	R3-180632	Reply LS on handling concurrent running of security procedures, RAN3

# Text Proposal for NGAP TS 38.413 v0.6.0

<<<<<<<<<<<<<<<<<<<< Begin Text Proposal >>>>>>>>>>>>>>>>>>>>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.401: “NG-RAN; Architecture description”.

[3]	3GPP TS 38.410: “NG-RAN; NG general aspects and principles”.

[4]	ITU-T Recommendation X.691 (07/2002): "Information technology – ASN.1 encoding rules: Specification of Packed Encoding Rules (PER)".

[5]	ITU-T Recommendation X.680 (07/2002): "Information technology – Abstract Syntax Notation One (ASN.1): Specification of basic notation".

[6]	ITU-T Recommendation X.681 (07/2002): "Information technology – Abstract Syntax Notation One (ASN.1): Information object specification".

[7]	3GPP TR 25.921 (version.7.0.0): "Guidelines and principles for protocol description and error handling".

[8]	3GPP TS 38.300: “NR; NR and NG-RAN Overall Description; Stage 2”.

[9]	3GPP TS 23.501: “System Architecture for the 5G System; Stage 2”.

[10]	3GPP TS 23.502: “Procedures for the 5G System; Stage 2”.

[11]	3GPP TS 32.422: “Trace control and configuration management”.

[12]	3GPP TS 37.320: “Radio measurement collection for Minimization of Drive Tests (MDT); Overall description; Stage 2”.

[13]	3GPP TS 33.401: “Security architecture”.

[14]	3GPP TS 38.414: “NG-RAN; NG data transport”.

[15]	3GPP TS 29.281: “General Packet Radio System (GPRS); Tunnelling Protocol User Plane (GTPv1-U)”.

[16]	3GPP TS 36.413: “Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol (S1AP)”.

[17]	3GPP TS 36.300: “Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2”.

[18]	3GPP TS 38.331: “NG-RAN; Radio Resource Control (RRC) Protocol Specification”.

[xx]	3GPP TS 33.501: “Security architecture and procedures for 5G System”.

------------------------------------------

Skip to next change

------------------------------------------

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core Network

AMF	Access and Mobility Management Function

EP	Elementary Procedure

GBR	Guaranteed Bit Rate

GTP	GPRS Tunnelling Protocol

IE	Information Element

NAS	Non-Access Stratum

NCGI	NR Cell Global Identifier

NGAP	NG Application Protocol

NSCI  	New Security Context Indicator

PLMN	Public Land Mobile Network

QoS	Quality of Service

SCTP	Stream Control Transmission Protocol

SMF	Session Management Function

S-NG-RAN node	Secondary NG-RAN node

S-NSSAI	Single Network Slice Selection Assistance Information

TEID	Tunnel Endpoint Identifier

UE	User Equipment

------------------------------------------

Skip to next change

------------------------------------------

### 8.4.2	Handover Resource Allocation

#### 8.4.2.1	General

The purpose of the Handover Resource Allocation procedure is to reserve resources at the target NG-RAN node for the handover of a UE.

Editor’s Note:	Further details are FFS.

#### 8.4.2.2	Successful Operation

Figure 8.4.2.2-1: Handover resource allocation: successful operation

The AMF initiates the procedure by sending the HANDOVER REQUEST message to the target NG-RAN node.

If the Masked IMEISV IE is contained in the HANDOVER REQUEST message the target NG-RAN node shall, if supported, use it to determine the characteristics of the UE for subsequent handling.

Upon receipt of the HANDOVER REQUEST message the NG-RAN node shall

-	attempt to execute the requested PDU session configuration;

-	store the UE Aggregate Maximum Bit Rate in the UE context, and use the received UE Aggregate Maximum Bit Rate for non-GBR Bearers for the concerned UE; [FFS]

-	store the received Handover Restriction List in the UE context;

-	store the received UE Security Capabilities in the UE context;

-	store the received Security Key in the UE context, take it into use and associate it with the initial value of NCC as defined in TS xx.xxx [xx]; [FFS pending SA3]

For the Initial Context Setup an initial value for the Next Hop Chaining Count is stored in the UE context. [FFS pending SA3]

Upon receiving the PDU Session Resource To Be Setup List IE contained in the HANDOVER REQUEST message, the NG-RAN node shall behave the same as defined in the PDU Session Resource Setup procedure. The NG-RAN node shall then report in the HANDOVER REQUEST ACKNOWLEDGE message the successful establishment of the result for all the requested PDU sessions. When the NG-RAN node reports the unsuccessful establishment of a PDU session, the cause value should be precise enough to enable the AMF to know the reason for the unsuccessful establishment.

The NG-RAN node shall use the information in the Handover Restriction List IE if present in the HANDOVER REQUEST message to

-	determine a target for subsequent mobility action for which the NG-RAN node provides information about the target of the mobility action towards the UE;

-	select a proper SCG during dual connectivity operation.

If the Handover Restriction List IE is not contained in the HANOVER REQUEST message, the NG-RAN node shall consider that no roaming and no access restriction apply to the UE. The NG-RAN node shall also consider that no roaming and no access restriction apply to the UE when:

-	one of the setup PDU sessions includes a particular ARP value (TS 23.501 [9]). [FFS, need to align with PDU Session Management]

If the Trace Activation IE is included in the HANDOVER REQUEST message the NG-RAN node shall, if supported, initiate the requested trace function as described in TS 32.422 [11]. In particular, the NG-RAN node shall, if supported: [FFS pending RAN2 and SA5]

-	if the Trace Activation IE does not include the MDT Configuration IE, initiate the requested trace session as described in TS 32.422 [11];

-	if the Trace Activation IE includes the MDT Activation IE, within the MDT Configuration IE, set to “Immediate MDT and Trace”, initiate the requested trace session and MDT session as described in TS 32.422 [11];

-	if the Trace Activation IE includes the MDT Activation IE, within the MDT Configuration IE, set to “Immediate MDT Only”, or “Logged MDT only”, initiate the requested MDT session as described in TS 32.422 [11] and the NG-RAN node shall ignore the Interfaces To Trace IE and Trace Depth IE;

-	if the Trace Activation IE includes the MDT Location Information IE, within the MDT Configuration IE, store this information and take it into account in the requested MDT session;

-	if the Trace Activation IE includes the Signalling based MDT PLMN List IE, within the MDT Configuration IE, the NG-RAN node may use it to propagate the MDT Configuration as described in TS 37.320 [12].

If the UE Security Capabilities IE is included in the HANDOVER REQUEST message, the NG-RAN node shall handle it accordingly. [FFS pending SA3]

If the Management Based MDT Allowed IE is contained in the HANDOVER REQUEST message, the NG-RAN node shall use it, if supported, together with information in the Management Based MDT PLMN List IE, if available in the UE context, to allow subsequent selection of the UE for management based MDT defined in TS 32.422 [11]. [FFS pending RAN2 and SA5].

If the RRC Inactive Assistance Information IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store this information in the UE context and use it for the RRC-INACTIVE state decision and configuration for the UE and RAN paging if any for a UE in RRC-INACTIVE state, as specified in TS 38.300 [8].

If the New Security Context Indicator IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use this NSCI information as specified in TS 33.501 [xx].

After all necessary resources for the admitted PDU Session Resources have been allocated, the target NG-RAN node shall generate the HANDOVER REQUEST ACKNOWLEDGE message.

Editor’s Note:	Further details are FFS.

#### 8.4.2.3	Unsuccessful Operation

Figure 8.4.2.3-1: Handover resource allocation: unsuccessful operation

If the target NG-RAN node does not admit at least one PDU Session Resource, or a failure occurs during the Handover Preparation, it shall send the HANDOVER FAILURE message to the AMF with an appropriate cause value.

Editor’s Note:	Further details are FFS.

#### 8.4.2.4	Abnormal Conditions

Editor’s Note:	Further details are FFS.

------------------------------------------

Skip to next change

------------------------------------------

### 8.4.4	Path Switch Request

#### 8.4.4.1	General

The purpose of the Path Switch Request procedure is to request the switch of a downlink GTP tunnel towards a new GTP tunnel endpoint.

Editor’s Note:	Further details are FFS.

#### 8.4.4.2	Successful Operation

Figure 8.4.4.2-1: Path switch request: successful operation

The NG-RAN node initiates the procedure by sending the PATH SWITCH REQUEST message to the AMF.

After all necessary updates including the UP path switch have been successfully completed in the 5GC for at least one of the PDU Session Resources included in the PATH SWITCH REQUEST, the AMF shall send the PATH SWITCH REQUEST ACKNOWLEDGE message to the NG-RAN node and the procedure ends.

The list of PDU sessions which failed to be setup, if any, shall be included in the PATH SWITCH REQUEST message. The AMF shall handle this information as specified in TS 23.502 [10].

If the RRC Inactive Assistance Information IE is included in the PATH SWITCH REQUEST ACKNOWLEDGE message, the NG-RAN node shall, if supported, store this information in the UE context and use it for the RRC-INACTIVE state decision and configuration for the UE and RAN paging if any for a UE in RRC-INACTIVE state, as specified in TS 38.300 [8].

If the New Security Context Indicator IE is included in the PATH SWITCH REQUEST ACKNOWLEDGE message, the target NG-RAN node shall, if supported, use this NSCI information as specified in TS 33.501 [xx].

Editor’s Note:	Further details are FFS.

#### 8.4.4.3	Unsuccessful Operation

Figure 8.4.4.3-1: Path switch request: unsuccessful operation

If the 5GC fails to switch the downlink GTP tunnel endpoint towards a new GTP tunnel endpoint for all PDU Session Resources, the AMF shall send the PATH SWITCH REQUEST FAILURE message to the NG-RAN node with an appropriate cause value.

Editor’s Note:	Further details are FFS.

#### 8.4.4.4	Abnormal Conditions

Editor’s Note:	Further details are FFS.

------------------------------------------

Skip to next change

------------------------------------------

#### 9.2.3.4	HANDOVER REQUEST

Editor’s Note:	Message structure and IEs need further checking and completion. Further details FFS.

This message is sent by the AMF to the target NG-RAN node to request the preparation of resources.

Direction: AMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| Handover Type | M |  | 9.3.1.33 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| UE Aggregate Maximum Bit Rate | M |  | <ref> |  | YES | reject |
| RRC Inactive Assistance Information | M |  | 9.3.1.26 |  | YES | ignore |
| UE Security Capabilities | O [FFS] |  | <ref> |  | YES | reject |
| Security Key | O [FFS] |  | <ref> |  | YES | reject |
| New Security Context Indicator | O |  | <ref> |  | YES | reject |
| PDU Session Resource To Be Setup List |  | 1 |  |  | YES | reject |
| >PDU Session Resource To Be Setup Item IEs |  | 1..<maxnoofPDUSessions> |  |  | EACH | reject |
| >>PDU Session ID | M |  | <ref> |  | - |  |
| >>S-NSSAI | O |  | 9.3.1.35 |  | - |  |
| >>PDU Session Setup Request Transfer | M |  | 9.3.1.11 |  | YES | ignore |
| Trace Activation | O |  | <ref> | [FFS pending RAN2 and SA5] | YES | ignore |
| Management Based MDT Allowed | O |  | <ref> | [FFS pending RAN2 and SA5] | YES | ignore |
| Management Based MDT PLMN List | O |  | <ref> | [FFS pending RAN2 and SA5] | YES | ignore |
| Masked IMEISV | O |  | <ref> |  | YES | ignore |
| Source to Target Transparent Container | M |  | 9.3.1.31 |  | YES | reject |
| Handover Restriction List | O |  | <ref> |  | YES | ignore |

------------------------------------------

Skip to next change

------------------------------------------

#### 9.2.3.9	PATH SWITCH REQUEST ACKNOWLEDGE

Editor’s Note:	Message structure and IEs need further checking and completion. Further details FFS.

This message is sent by the AMF to inform the NG-RAN node that the path switch has been successfully completed in the 5GS.

Direction: AMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| Security Context | M |  | <ref> |  | YES | ignore |
| New Security Context Indicator | O |  | <ref> |  | YES | reject |
| PDU Session To Be Switched in Uplink List |  | 0..1 |  |  | YES | reject |
| >PDU Session To Be Switched in Uplink Item IEs |  | 1..<maxnoofPDUSessionResources> |  |  | EACH | reject |
| >>PDU Session ID | M |  | <ref> |  | - |  |
| >>S-NSSAI (FFS) | O |  | 9.3.1.35 |  | - |  |
| >>PDU Session Path Switch Request Ack Transfer | M |  | 9.3.1.22 |  | YES | ignore |
| PDU Session Released List | O |  | PDU Session List 9.3.1.13 |  | YES | reject |
| RRC Inactive Assistance Information | M |  | 9.3.1.26 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< End Text Proposal  >>>>>>>>>>>>>>>>>>>>