3GPP TSG-RAN WG3 #99	Tdoc R3-181301

Athens, Greece, 26 February – 2 March 2018

Agenda Item:	23.3

Source:	Ericsson

Title:	E1 Reset procedure

Document for:	pCR TS 38.463

# Introduction

As illustrated in TR 38.806 [1], during the study item it was agreed to define the following E1 interface management procedures:

- E1 setup: this procedure allows to setup the E1 interface. It includes the exchange of the parameters needed for interface operation.

- E1 reset: this procedure allows to reset the E1 interface. It is used to initialize the peer entity after node setup and after a failure event occurred. The E1 interface reset is initiated by either the CU-CP or the CU-UP.

- E1 error indication: this procedure allows to report detected errors in one incoming message. The E1 interface reset is initiated by either the CU-CP or the CU-UP.

In this contribution, we discuss the content of the E1 Reset procedure.

# Discussion

An E1 Rest procedure is needed to address failure events at the CU-CP and/or CU-UP. The question can then be raised on whether the E1 Reset procedure should also provide support for “partial reset”. The partial reset enables for resetting only some UE-associated logical E1-connections, while keeping the others unchanged. We believe that such function may be useful in some cases.

For example, it has been agreed that the CU-CP can be virtualized and composed by different “CU-CP entities”. Each “CU-CP entity” may handle a subset of the UEs connected to a gNB. In case that a “CU-CP entity” experiences a failure, only the UEs that are handled by this “CU-CP entity” are affected. Therefore, it seems logical that only the corresponding UE-associated logical E1-connections are reset, while the others UE-associated logical E1-connections remain operational.

Proposal 1: 	The E1 Reset procedure provides support for “partial reset”.

Assuming that a partial reset function is supported, the question can be raised on how to identify a UE-associated logical E1-connection. On the F1 interface, a UE-associated logical connection is recognized by a pair of identifiers: GNB-CU UE F1AP ID and GNB-DU UE F1AP ID. The use of two identifiers facilities the retrieval of the UE context at both side of the F1 interface.

We believe that over the E1 interface a similar pair of identifiers, namely CU-CP UE E1AP ID and CU-UP UE E1AP ID is needed to allow the CU-CP and the CU-UP to easily identify a bundle of resources (e.g., PDU Sessions, Data Radio Bearers, and QoS-flows) that are associated to a given UE.

Proposal 2: 	Introduce a CU-CP UE E1AP ID allocated uniquely by the CU-CP and a CU-UP UE E1AP ID allocated uniquely by the CU-UP. A pair of CU-CP UE E1AP ID and CU-UP UE E1AP ID identify a UE-associated logical E1-connection.

For the partial reset, the initiating node (CU-CP and/or CU-UP) may include a list of UE-associated logical E1-connections, identified by the corresponding CU-CP UE E1AP ID and/or CU-UP UE E1AP ID. Only these UE-associated logical E1-connections are affected by the reset procedure, while the others remain operational.

# Conclusion

In this contribution, we discussed the allocations the E1 Reset procedure.

Proposal 1: 	The E1 Reset procedure provides support for “partial reset”.

Proposal 2: 	Introduce a CU-CP UE E1AP ID allocated uniquely by the CU-CP and a CU-UP UE E1AP ID allocated uniquely by the CU-UP. A pair of CU-CP UE E1AP ID and CU-UP UE E1AP ID identify a UE-associated logical E1-connection.

# References

TR 38.806, Study of separation of NR Control Plane (CP) and User Plane (UP) for split option 2.

# Annex I: TP for 38.463

Start of Text Proposal 1 for TS 38.463

3. 	Definitions and abbreviations

CU-CP UE E1AP ID: A CU-CP UE E1AP ID shall be allocated so as to uniquely identify the UE over the E1 interface within a CU-CP. When a CU-UP receives a CU-CP UE E1AP ID it shall store it for the duration of the UE-associated logical E1-connection for this UE. The CU-CP UE E1AP ID shall be unique within the CU-CP logical node.

CU-UP UE E1AP ID: A CU-UP UE E1AP ID shall be allocated so as to uniquely identify the UE over the E1 interface within a CU-UP. When a CU-CP receives a CU-UP UE E1AP ID it shall store it for the duration of the UE-associated logical E1-connection for this UE. The CU-UP UE E1AP ID shall be unique within the CU-UP logical node.

Editor’s note: whether to move the above definition to TS 38.401 is FFS.

UE-associated signalling: When E1AP messages associated to one UE use the UE-associated logical F1-connection for association of the message to the UE in gNB-DU and gNB-CU.

UE-associated logical E1-connection: The UE-associated logical E1-connection uses the identities CU-CP UE E1AP ID and CU-UP UE E1AP ID. For a received UE associated E1AP message the CU-CP identifies the associated UE based on the CU-CP UE E1AP ID IE and the CU-UP identifies the associated UE based on the CU-CP UE E1AP ID IE.

Start of Text Proposal 2 for TS 38.463

Table 1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| Reset | RESET | RESET ACKNOWLEDGE |  |

Start of Text Proposal 3 for TS 38.463

### 8.2.X	Reset

#### 8.2.X.1	General

The purpose of the Reset procedure is to initialise or re-initialise the E1AP UE-related contexts, in the event of a failure in the CU-CP or CU-UP. This procedure does not affect the application level configuration data exchanged during, e.g., the E1 Setup procedure. The procedure uses non-UE associated signalling.

#### 8.2.X.2	Successful Operation

##### 8.2.X.2.1	Reset Procedure Initiated from the CU-CP

Figure 8.2.X.2.1-1: Reset procedure initiated from the CU-CP. Successful operation.

In the event of a failure at the CU-CP, which has resulted in the loss of some or all transaction reference information, a RESET message shall be sent to the CU-UP.

At reception of the RESET message the CU-UP shall release all allocated resources on E1 and bearer resources related to the UE association(s) indicated explicitly or implicitly in the RESET message and remove the indicated UE information including E1AP ID.

After the CU-UP has released all assigned E1 resources and the UE E1AP IDs for all indicated UE associations which can be used for new UE-associated logical E1-connections over the E1 interface, the CU-UP shall respond with the RESET ACKNOWLEDGE message. The CU-UP does not need to wait for the release of bearer resources to be completed before returning the RESET ACKNOWLEDGE message.

If the RESET message contains the UE-associated logical E1-connection list IE, then:

-	The CU-UP shall use the CU-CP UE F1AP ID IE and/or the CU-UP UE F1AP ID IE to explicitly identify the UE association(s) to be reset.

-	The CU-UP shall include in the RESET ACKNOWLEDGE message, for each UE association to be reset, the UE-associated logical E1-connection Item IE in the UE-associated logical E1-connection list IE. The UE-associated logical E1-connection Item IEs shall be in the same order as received in the RESET message and shall include also unknown UE-associated logical E1-connections. Empty UE-associated logical E1-connection Item IEs, received in the RESET message, may be omitted in the RESET ACKNOWLEDGE message.

-	If the CU-CP E1AP ID IE is included in the UE-associated logical E1-connection Item IE for a UE association, the CU-UP shall include the CU-CP UE E1AP ID IE in the corresponding UE-associated logical E1-connection Item IE in the RESET ACKNOWLEDGE message.

-	If the CU-UP UE F1AP ID IE is included in the UE-associated logical E1-connection Item IE for a UE association, the CU-UP shall include the CU-UP UE E1AP ID IE in the corresponding UE-associated logical E1-connection Item IE in the RESET ACKNOWLEDGE message.

Interactions with other procedures:

If the RESET message is received, any other ongoing procedure (except for another Reset procedure) on the same E1 interface related to a UE association, indicated explicitly or implicitly in the RESET message, shall be aborted.

##### 8.2.X.2.2	Reset Procedure Initiated from the CU-UP

Figure 8.2.X.2.2-1: Reset procedure initiated from the CU-UP. Successful operation.

In the event of a failure at the CU-UP, which has resulted in the loss of some or all transaction reference information, a RESET message shall be sent to the CU-CP.

At reception of the RESET message the CU-CP shall release all allocated resources on E1 related to the UE association(s) indicated explicitly or implicitly in the RESET message and remove the E1AP ID for the indicated UE associations.

After the CU-CP has released all assigned E1 resources and the UE E1AP IDs for all indicated UE associations which can be used for new UE-associated logical E1-connections over the E1 interface, the CU-CP shall respond with the RESET ACKNOWLEDGE message. The CU-CP does not need to wait for the release of radio resources to be completed before returning the RESET ACKNOWLEDGE message.

If the RESET message contains the UE-associated logical E1-connection list IE, then:

-	The CU-CP shall use the CU-CP UE F1AP ID IE and/or the CU-UP UE F1AP ID IE to explicitly identify the UE association(s) to be reset.

-	The CU-CP shall include in the RESET ACKNOWLEDGE message, for each UE association to be reset, the UE-associated logical E1-connection Item IE in the UE-associated logical E1-connection list IE. The UE-associated logical E1-connection Item IEs shall be in the same order as received in the RESET message and shall include also unknown UE-associated logical E1-connections. Empty UE-associated logical E1-connection Item IEs, received in the RESET message, may be omitted in the RESET ACKNOWLEDGE message.

-	If the CU-CP E1AP ID IE is included in the UE-associated logical E1-connection Item IE for a UE association, the CU-CP shall include the CU-CP UE E1AP ID IE in the corresponding UE-associated logical E1-connection Item IE in the RESET ACKNOWLEDGE message.

-	If the CU-UP UE F1AP ID IE is included in the UE-associated logical E1-connection Item IE for a UE association, the CU-CP shall include the CU-UP UE E1AP ID IE in the corresponding UE-associated logical E1-connection Item IE in the RESET ACKNOWLEDGE message.

Interactions with other procedures:

If the RESET message is received, any other ongoing procedure (except for another Reset procedure) on the same E1 interface related to a UE association, indicated explicitly or implicitly in the RESET message, shall be aborted.

#### 8.2.X.3	Abnormal Conditions

Not applicable.

Start of Text Proposal 4 for TS 38.463

#### 9.2.1.X	RESET

This message is sent by both the CU-CP  and the CU-UP and is used to request that the E1 interface, or parts of the E1 interface, to be reset.

Direction: CU-CP  CU-UP and CU-UP  CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.x |  | YES | reject |
| Cause | M |  | 9.3.1.y |  | YES | ignore |
| CHOICE Reset Type | M |  |  |  | YES | reject |
| >E1 interface |  |  |  |  |  |  |
| >>Reset All | M |  | ENUMERATED (Reset all,…) |  | - |  |
| >Part of E1 interface |  |  |  |  |  |  |
| >>UE-associated logical E1-connection list |  | 1 |  |  | - |  |
| >>>UE-associated logical E1-connection Item |  | 1 .. <maxnoofIndividualE1ConnectionsToReset> |  |  | EACH | reject |
| >>>>CU-CP UE E1AP ID | O |  | 9.3.1.z |  | - |  |
| >>>>CU-UP UE E1AP ID | O |  | 9.3.1.xx |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofIndividualE1ConnectionsToReset | Maximum no. of UE-associated logical E1-connections allowed to reset in one message. Value is FFS. |

#### 9.2.1.Y	RESET ACKNOWLEDGE

This message is sent by both the CU-CP and the CU-UP as a response to a RESET message.

Direction: CU-CP  CU-UP and CU-UP  CU-CP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.x |  | YES | reject |
| UE-associated logical E1-connection list |  | 0..1 |  |  | YES | ignore |
| >UE-associated logical E1-connection Item |  | 1 .. <maxnoofIndividualE1ConnectionsToReset> |  |  | EACH | ignore |
| >>CU-CP UE E1AP ID | O |  | 9.3.1.z |  | - |  |
| >>CU-UP UE E1AP ID | O |  | 9.3.1.xx |  | - |  |
| Criticality Diagnostics | O |  | 9.3.1.w |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofIndividualE1ConnectionsToReset | Maximum no. of UE-associated logical E1-connections allowed to reset in one message. Value is FFS. |

Start of Text Proposal 5 for TS 38.463

#### 9.3.1.y	Cause

The purpose of the Cause IE is to indicate the reason for a particular event for the E1AP protocol.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Cause Group | M |  |  |  |
| >Radio Network Layer |  |  |  |  |
| >>Radio Network Layer Cause | M |  | ENUMERATED (Unspecified, …) |  |
| >Transport Layer |  |  |  |  |
| >>Transport Layer Cause | M |  | ENUMERATED (Unspecified, …) |  |
| >Protocol |  |  |  |  |
| >>Protocol Cause | M |  | ENUMERATED (Transfer Syntax Error, Abstract Syntax Error (Reject), Abstract Syntax Error (Ignore and Notify), Message not Compatible with Receiver State, Semantic Error, Abstract Syntax Error (Falsely Constructed Message), Unspecified, …) |  |
| >Misc |  |  |  |  |
| >>Miscellaneous Cause | M |  | ENUMERATED (Control Processing Overload, Not enough User Plane Processing Resources, Hardware Failure, O&M Intervention, Unspecified, …) |  |

The meaning of the different cause values is described in the following table. In general, "not supported" cause values indicate that the related capability is missing. On the other hand, "not available" cause values indicate that the related capability is present, but insufficient resources were available to perform the requested action.

| Radio Network Layer cause | Meaning |
| --- | --- |
| Unspecified | Sent for radio network layer cause when none of the specified cause values applies. |

| Transport Layer cause | Meaning |
| --- | --- |
| Unspecified | Sent when none of the above cause values applies but still the cause is Transport Network Layer related. |

| Protocol cause | Meaning |
| --- | --- |
| Transfer Syntax Error | The received message included a transfer syntax error. |
| Abstract Syntax Error (Reject) | The received message included an abstract syntax error and the concerning criticality indicated "reject". |
| Abstract Syntax Error (Ignore And Notify) | The received message included an abstract syntax error and the concerning criticality indicated "ignore and notify". |
| Message Not Compatible With Receiver State | The received message was not compatible with the receiver state. |
| Semantic Error | The received message included a semantic error. |
| Abstract Syntax Error (Falsely Constructed Message) | The received message contained IEs or IE groups in wrong order or with too many occurrences. |
| Unspecified | Sent when none of the above cause values applies but still the cause is Protocol related. |

| Miscellaneous cause | Meaning |
| --- | --- |
| Control Processing Overload | Control processing overload. |
| Not Enough User Plane Processing Resources Available | No enough resources are available related to user plane processing. |
| Hardware Failure | Action related to hardware failure. |
| O&M Intervention | The action is due to O&M intervention. |
| Unspecified Failure | Sent when none of the above cause values applies and the cause is not related to any of the categories Radio Network Layer, Transport Network Layer, or Protocol. |

#### 9.3.1.w	Criticality Diagnostics

The Criticality Diagnostics IE is sent by the CU-CP or the CU-UP when parts of a received message have not been comprehended or were missing, or if the message contained logical errors. When applicable, it contains information about which IEs were not comprehended or were missing.

For further details on how to use the Criticality Diagnostics IE, (see clause 10).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Procedure Code | O |  | INTEGER (0..255) | Procedure Code is to be used if Criticality Diagnostics is part of Error Indication procedure, and not within the response message of the same procedure that caused the error. |
| Triggering Message | O |  | ENUMERATED(initiating message, successful outcome, unsuccessful outcome) | The Triggering Message is used only if the Criticality Diagnostics is part of Error Indication procedure. |
| Procedure Criticality | O |  | ENUMERATED(reject, ignore, notify) | This Procedure Criticality is used for reporting the Criticality of the Triggering message (Procedure). |
| Information Element Criticality Diagnostics |  | 0 .. <maxnoof Errors> |  |  |
| >IE Criticality | M |  | ENUMERATED(reject, ignore, notify) | The IE Criticality is used for reporting the criticality of the triggering IE. The value 'ignore' shall not be used. |
| >IE ID | M |  | INTEGER (0..65535) | The IE ID of the not understood or missing IE. |
| >Type of Error | M |  | ENUMERATED(not understood, missing, …) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofErrors | Maximum no. of IE errors allowed to be reported with a single message. The value for maxnoofErrors is 256. |

#### 9.3.1.z	CU-CP UE E1AP ID

The CU-CP UE E1AP ID uniquely identifies the UE association over the E1 interface within the CU-CP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CU-CP UE E1AP ID | M |  | INTEGER (0 .. 232 -1) |  |

#### 9.3.1.xx	CU-UP UE E1AP ID

The CU-UP UE E1AP ID uniquely identifies the UE association over the E1 interface within the CU-UP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CU-UP UE E1AP ID | M |  | INTEGER (0 .. 232 -1) |  |