3GPP TSG-RAN WG3 #99	Tdoc R3-181302

Athens, Greece, 26 February – 2 March 2018

Agenda Item:	23.3

Source:	Ericsson

Title:	E1 Release procedure

Document for:	pCR TS 38.463

# Introduction

As illustrated in TR 38.806 [1], during the study item it was agreed to define the following E1 interface management procedures:

- E1 setup: this procedure allows to setup the E1 interface. It includes the exchange of the parameters needed for interface operation.

- E1 reset: this procedure allows to reset the E1 interface. It is used to initialize the peer entity after node setup and after a failure event occurred. The E1 interface reset is initiated by either the CU-CP or the CU-UP.

- E1 error indication: this procedure allows to report detected errors in one incoming message. The E1 interface reset is initiated by either the CU-CP or the CU-UP.

In this contribution, we propose to introduce an E1 Release procedure.

# Discussion

During the study item, an E1 Release procedure was not discussed.

We believe that the E1 interface may be established and released more dynamically with respect to other interfaces (such as F1 and NG). For example, in the future there may be CU-UPs that support only some special slices. The CU-CP may want to establish an E1 connectivity toward these CU-UPs only when there are UEs requesting for these slices. Afterwards, the CU-CP may want to release the E1 interface toward these CU-UPs. This leads to a more efficient resource utilization at both the CU-CP and CU-UP side. Therefore, we believe that an E1 Release procedure that is initiated by the CU-CP is beneficial.

Meanwhile, in case that there is some planned maintenance in a data center or simply for energy saving purposes a CU-UP may be shut down. In this case, the E1 Release may be initiated by the CU-UP (after all the bearers have been released).

Proposal 1: 	Introduce a Class-1 E1 Release procedure. The procedure can be initiated by both the CU-CP and the CU-UP.

Proposal 2: 	RAN3 is kindly asked to agree with the TP in Annex I.

# Conclusion

In this contribution, we discussed the E1 Release procedure.

Proposal 1: 	Introduce a Class-1 E1 Release procedure. The procedure can be initiated by both the CU-CP and the CU-UP. FFS whether to consider unsuccessful outcome.

Proposal 2: 	RAN3 is kindly asked to agree with the TP in Annex I.

# References

TR 38.806, Study of separation of NR Control Plane (CP) and User Plane (UP) for split option 2.

# Annex I: TP for 38.463

Start of Text Proposal 1 for TS 38.463

Table 1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| E1 Release | E1 RELEASE REQUEST | E1 RELEASE RESPONSE |  |

Start of Text Proposal 2 for TS 38.463

### 8.2.Z	E1 Release

#### 8.2.Z.1	General

#### The purpose of the E1 Release procedure is to release all existing signalling connections and related application level data. The procedure uses non-UE associated signalling.

#### 8.2.Z.2	Successful Operation

##### 8.2.Z.2.1	Release Procedure Initiated from the CU-CP

Figure 8.2.Z.2.1-1: Release procedure initiated from the CU-CP. Successful operation.

The CU-CP initiates the procedure by sending the E1 RELEASE REQUEST message to the CU-UP.

Upon reception of the E1 RELEASE REQUEST message, the CU-UP shall release any existing resources related to the E1 interface. The CU-UP shall respond with a E1 RELEASE RESPONSE message to confirm that it has initiated the release of the resources, if existing, and that the signalling connection for the E1AP application protocol is released.

##### 8.2.Z.2.2	Release Procedure Initiated from the CU-UP

Figure 8.2.Z.2.2-1: Reset procedure initiated from the CU-UP. Successful operation.

The CU-UP initiates the procedure by sending the E1 RELEASE REQUEST message to the CU-CP.

Upon reception of the E1 RELEASE REQUEST message, the CU-CP shall release any existing resources related to the E1 interface. The CU-CP shall respond with a E1 RELEASE RESPONSE message to confirm that it has initiated the release of the resources, if existing, and that the signalling connection for the E1AP application protocol is released.

#### 8.2.Z.3	Abnormal Conditions

Not applicable.

Start of Text Proposal 3 for TS 38.463

#### 9.2.1.Y	E1 RELEASE REQUEST

This message is sent by both the CU-CP  and the CU-UP and is used to request the release of the E1 interface.

Direction: CU-CP  CU-UP and CU-UP  CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.x |  | YES | reject |
| Cause | M |  | 9.3.1.z |  | YES | ignore |

#### 9.2.1.Z	E1 RELEASE RESPONSE

This message is sent by both the CU-CP and the CU-UP as a response to a E1 RELEASE REQUEST message.

Direction: CU-CP  CU-UP and CU-UP  CU-CP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.x |  | YES | reject |

End of Text Proposal for TS 38.463