3GPP TSG-RAN WG3 #99	Tdoc R3-181344

Athens, Greece, 26 February – 2 March 2018

Agenda Item:	31.3.1

Source:	Ericsson

Title:	(TP for NSA BL CR) CU-CP Name over F1-C

Document for:	pCR TS 38.473 (TP for NSA BL CR)

# Introduction

This contribution proposes to complement the already agreed signalling of DU name over F1-C with the corresponding CU-CP name.

# Discussion

RAN3 already agreed to introduce support to signal DU name, a printable string format useful for O&M purposes in the F1 Setup Request.

We think however that signalling back a corresponding CU(-CP) name to the DU can also be very useful for similar purposes, and hence think the solution is not complete unless the pair of logical node names is exchanged during F1 Setup.

Proposal 1: 	Agree to add the CU(-CP) name to F1 Setup Response.

We also noted that currently the gNB-DU Name is misleadingly defined in the F1AP ASN.1, as it is defined as a gNB name and not as a gNB-DU name. Even though the tabular format indicates indeed that the identifier is for a gNB-DU, we consider it bound to create misunderstanding to have an identifier in the ASN.1 that is called ‘gNB Name’, as it does not identify a full gNB (especially when it comes to the gNB-DU). Hence, we propose to avoid any possible misunderstanding by defining a gNB-DU name as a gNB-DU name.

Proposal 2: 	Change the ASN.1 definition of the gNB-DU name from gNB name to the more appropriate gNB-DU name, in line with the tabular format.

# Conclusion

In this contribution, we discussed the fact that adding the CU-CP name signalling support to F1 Setup is beneficial. A similar solution is also proposed for the E1 interface in [1].

Based on the above, we propose:

Proposal 1: 	Agree to add the CU(-CP) name to F1 Setup Response.

Proposal 2: 	Change the ASN.1 definition of the gNB-DU name from gNB name to the more appropriate gNB-DU name, in line with the tabular format.

# References

R3-181300, ‘E1 Setup Procedure’, Ericsson, February 2018.

# Annex I: TP for 38.473 (on EN-DC BL CR)

Start of Text Proposal for TS 38.473

#### 9.2.1.5	F1 SETUP RESPONSE

This message is sent by the gNB-CU to transfer information for a TNL association.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU Name | O |  | PrintableString(SIZE(1..150,…)) | Human readable name of the gNB-CU. If the gNB-CU corresponds to a gNB-CU-CP, this identifier is the same as the CU-CP name | YES | ignore |
| Cells to be Activated List |  | 0.. 1 |  |  |  |  |
| >Cells to be Activated List Item |  | 1.. <maxCellingNBDU> |  | List of cells to be activated | YES | reject |
| >>NCGI | M |  | 9.3.1.12 |  | - | - |
| >>PCI | O |  | INTEGER (0..1007) | Physical Cell ID | - | - |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |

Start of Text Proposal for TS 38.473

### 9.4.4	PDU Definitions

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

F1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Cause,

Cells-Failed-to-be-Activated-List-Item,

Cells-to-be-Activated-List-Item,

Cells-to-be-Deactivated-List-Item,

CriticalityDiagnostics,

C-RNTI,

CUtoDURRCInformation,

DRBID,

DRBs-FailedToBeModifiedConf-Item,

DRBs-FailedToBeModified-Item,

DRBs-FailedToBeSetup-Item,

DRBs-FailedToBeSetupMod-Item,

DRBs-ModifiedConf-Item,

DRBs-Modified-Item,

DRBs-Required-ToBeModified-Item,

DRBs-Required-ToBeReleased-Item,

DRBs-Setup-Item,

DRBs-SetupMod-Item,

DRBs-ToBeModified-Item,

DRBs-ToBeReleased-Item,

DRBs-ToBeSetup-Item,

DRBs-ToBeSetupMod-Item,

DRXCycle,

DUtoCURRCInformation,

EUTRANQoS,

GNB-CU-F1AP-ID,

GNB-DU-F1AP-ID,

GNB-DU-ID,

GNB-DU-Served-Cells-Item,

GNB-DU-System-Information,

GNB-DU-Name,

GNB-CU-Name,

GTPTunnelEndpoint,

NCGI,

PCI,

ResourceCoordinationTransferContainer,

RRCContainer,

SCellIndex,

SCell-ToBeRemoved-Item,

SCell-ToBeSetup-Item,

SCell-ToBeSetupMod-Item,

Served-Cell-Information,

Served-Cells-To-Add-Item,

Served-Cells-To-Delete-Item,

Served-Cells-To-Modify-Item,

SRBID,

SRBs-FailedToBeSetup-Item,

SRBs-FailedToBeSetupMod-Item,

SRBs-Required-ToBeReleased-Item,

SRBs-Setup-Item,

SRBs-ToBeReleased-Item,

SRBs-ToBeSetup-Item,

SRBs-ToBeSetupMod-Item,

TimeToWait,

TransactionID,

TransmissionStopIndicator,

UE-associatedLogicalF1-ConnectionItem,

ULTunnels-ToBeSetup-Item

FROM F1AP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerPair{},

ProtocolIE-SingleContainer{},

F1AP-PRIVATE-IES,

F1AP-PROTOCOL-EXTENSION,

F1AP-PROTOCOL-IES,

F1AP-PROTOCOL-IES-PAIR

FROM F1AP-Containers

id-Cause,

id-Cells-Failed-to-be-Activated-List,

id-Cells-Failed-to-be-Activated-List-Item,

id-Cells-to-be-Activated-List,

id-Cells-to-be-Activated-List-Item,

id-Cells-to-be-Deactivated-List,

id-Cells-to-be-Deactivated-List-Item,

id-CriticalityDiagnostics,

id-C-RNTI,

id-CUtoDURRCInformation,

id-DRBs-FailedToBeModifiedConf-Item,

id-DRBs-FailedToBeModifiedConf-List,

id-DRBs-FailedToBeModified-Item,

id-DRBs-FailedToBeModified-List,

id-DRBs-FailedToBeSetup-Item,

id-DRBs-FailedToBeSetup-List,

id-DRBs-FailedToBeSetupMod-Item,

id-DRBs-FailedToBeSetupMod-List,

id-DRBs-ModifiedConf-Item,

id-DRBs-ModifiedConf-List,

id-DRBs-Modified-Item,

id-DRBs-Modified-List,

id-DRBs-Required-ToBeModified-Item,

id-DRBs-Required-ToBeModified-List,

id-DRBs-Required-ToBeReleased-Item,

id-DRBs-Required-ToBeReleased-List,

id-DRBs-Setup-Item,

id-DRBs-Setup-List,

id-DRBs-SetupMod-Item,

id-DRBs-SetupMod-List,

id-DRBs-ToBeModified-Item,

id-DRBs-ToBeModified-List,

id-DRBs-ToBeReleased-Item,

id-DRBs-ToBeReleased-List,

id-DRBs-ToBeSetup-Item,

id-DRBs-ToBeSetup-List,

id-DRBs-ToBeSetupMod-Item,

id-DRBs-ToBeSetupMod-List,

id-DRXCycle,

id-DUtoCURRCInformation,

id-gNB-CU-F1AP-ID,

id-gNB-DU-F1AP-ID,

id-gNB-DU-ID,

id-GNB-DU-Served-Cells-Item,

id-gNB-DU-Served-Cells-List,

id-gNB-DU-Name,

id-gNB-CU-Name,

id-oldgNB-DU-F1AP-ID,

id-ResetType,

id-ResourceCoordinationTransferContainer,

id-RRCContainer,

id-SCell-ToBeRemoved-Item,

id-SCell-ToBeRemoved-List,

id-SCell-ToBeSetup-Item,

id-SCell-ToBeSetup-List,

id-SCell-ToBeSetupMod-Item,

id-SCell-ToBeSetupMod-List,

id-Served-Cells-To-Add-Item,

id-Served-Cells-To-Add-List,

id-Served-Cells-To-Delete-Item,

id-Served-Cells-To-Delete-List,

id-Served-Cells-To-Modify-Item,

id-Served-Cells-To-Modify-List,

id-SpCell-ID,

id-SRBID,

id-SRBs-FailedToBeSetup-Item,

id-SRBs-FailedToBeSetup-List,

id-SRBs-FailedToBeSetupMod-Item,

id-SRBs-FailedToBeSetupMod-List,

id-SRBs-Required-ToBeReleased-Item,

id-SRBs-Required-ToBeReleased-List,

id-SRBs-Setup-Item,

id-SRBs-Setup-List,

id-SRBs-ToBeReleased-Item,

id-SRBs-ToBeReleased-List,

id-SRBs-ToBeSetup-Item,

id-SRBs-ToBeSetup-List,

id-SRBs-ToBeSetupMod-Item,

id-SRBs-ToBeSetupMod-List,

id-TimeToWait,

id-TransactionID,

id-TransmissionStopIndicator,

id-UE-associatedLogicalF1-ConnectionItem,

id-UE-associatedLogicalF1-ConnectionListResAck,

maxCellingNBDU,

maxnoofDRBs,

maxnoofErrors,

maxnoofIndividualF1ConnectionsToReset,

maxnoofSCells,

maxnoofSRBs

FROM F1AP-Constants;

Start of Text Proposal for TS 38.473

-- **************************************************************

--

#### -- F1 SETUP ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- F1 Setup Request

--

-- **************************************************************

F1SetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {F1SetupRequestIEs} },

...

}

F1SetupRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-gNB-DU-ID					CRITICALITY reject	TYPE GNB-DU-ID					PRESENCE mandatory	}|

{ ID id-gNB-DU-Name						CRITICALITY ignore	TYPE	 GNB-DU-Name					PRESENCE optional	}|

{ ID id-gNB-DU-Served-Cells-List		CRITICALITY reject	TYPE	 GNB-DU-Served-Cells-List	PRESENCE mandatory	},

...

}

GNB-DU-Served-Cells-List 	::= SEQUENCE (SIZE(1.. maxCellingNBDU)) OF ProtocolIE-SingleContainer { { GNB-DU-Served-Cells-ItemIEs } }

GNB-DU-Served-Cells-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-GNB-DU-Served-Cells-Item		CRITICALITY reject	TYPE		GNB-DU-Served-Cells-Item	PRESENCE mandatory	},

...

}

-- **************************************************************

--

-- F1 Setup Response

--

-- **************************************************************

F1SetupResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {F1SetupResponseIEs} },

...

}

F1SetupResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID					PRESENCE mandatory	}|

{ ID id-gNB-CU-Name						CRITICALITY ignore	TYPE	 GNB-CU-Name					PRESENCE optional	}|

{ ID id-Cells-to-be-Activated-List	CRITICALITY reject	TYPE	 Cells-to-be-Activated-List		PRESENCE optional	},

...

}

Cells-to-be-Activated-List	::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Cells-to-be-Activated-List-ItemIEs } }

Cells-to-be-Activated-List-ItemIEs	F1AP-PROTOCOL-IES::= {

{ ID id-Cells-to-be-Activated-List-Item				CRITICALITY reject	TYPE	 Cells-to-be-Activated-List-Item						PRESENCE mandatory}	,

...

}

Start of Text Proposal for TS 38.473

#### -- G

GBR-QosInformation ::= SEQUENCE {

e-RAB-MaximumBitrateDL			BitRate,

e-RAB-MaximumBitrateUL			BitRate,

e-RAB-GuaranteedBitrateDL		BitRate,

e-RAB-GuaranteedBitrateUL		BitRate,

iE-Extensions					ProtocolExtensionContainer { { GBR-QosInformation-ExtIEs} } OPTIONAL,

...

}

GBR-QosInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-CU-F1AP-ID		::= INTEGER (0..4294967295)

GNB-DU-F1AP-ID		::= INTEGER (0..4294967295)

GNB-DU-ID			::= INTEGER (0..68719476735)

GNB-DU-Served-Cells-Item ::= SEQUENCE {

served-Cell-Information		Served-Cell-Information,

gNB-DU-System-Information	GNB-DU-System-Information,

iE-Extensions				ProtocolExtensionContainer { { GNB-DU-Served-Cells-ItemExtIEs} }	OPTIONAL,

...

}

GNB-DU-Served-Cells-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-DU-System-Information ::= SEQUENCE {

mIB-message		MIB-message,

sIB1-message		SIB1-message,

iE-Extensions					ProtocolExtensionContainer { { GNB-DU-System-Information-ExtIEs } } OPTIONAL,

...

}

GNB-DU-System-Information-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-DU-Name ::= PrintableString(SIZE(1..150,...))

GNB-CU-Name ::= PrintableString(SIZE(1..150,...))

GTP-TEID				::= OCTET STRING (SIZE (4))

GTPTunnelEndpoint ::= SEQUENCE {

transportLayerAddress				TransportLayerAddress,

gTP-TEID							GTP-TEID,

iE-Extensions						ProtocolExtensionContainer { { GTPTunnelEndpoint-ExtIEs} }	OPTIONAL,

...

}

GTPTunnelEndpoint-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

End of Text Proposal for TS 38.473

Start of Text Proposal for TS 38.473

-- **************************************************************

--

-- IEs

--

-- **************************************************************

id-Cause											ProtocolIE-ID ::= 0

id-CriticalityDiagnostics							ProtocolIE-ID ::= 1

id-gNB-DU-F1AP-ID									ProtocolIE-ID ::= 2

id-gNB-CU-F1AP-ID									ProtocolIE-ID ::= 3

id-ResetType											ProtocolIE-ID ::= 4

id-TimeToWait										ProtocolIE-ID ::= 5

id-UE-associatedLogicalF1-ConnectionItem 			ProtocolIE-ID ::= 6

id-UE-associatedLogicalF1-ConnectionListResAck		ProtocolIE-ID ::= 7

id-RRCContainer										ProtocolIE-ID ::= 8

id-SRBID												ProtocolIE-ID ::= 9

id-gNB-DU-ID											ProtocolIE-ID ::= 10

id-gNB-DU-Name											ProtocolIE-ID ::= 11

id-NRCellID											ProtocolIE-ID ::= 12

id-gNB-DU-Served-Cells-List							ProtocolIE-ID ::= 14

id-Cells-to-be-Activated-List						ProtocolIE-ID ::= 15

id-Served-Cells-To-Add-List							ProtocolIE-ID ::= 16

id-Served-Cells-To-Modify-List						ProtocolIE-ID ::= 17

id-Served-Cells-To-Delete-List						ProtocolIE-ID ::= 18

id-Cells-to-be-Deactivated-List						ProtocolIE-ID ::= 19

id-Cells-Failed-to-be-Activated-List					ProtocolIE-ID ::= 20

id-TransactionID										ProtocolIE-ID ::= 21

id-SpCell-ID											ProtocolIE-ID ::= 28

id-SRBs-ToBeSetup-List								ProtocolIE-ID ::= 30

id-DRBs-ToBeSetup-List								ProtocolIE-ID ::= 31

id-CUtoDURRCInformation								ProtocolIE-ID ::= 36

id-DUtoCURRCInformation								ProtocolIE-ID ::= 37

id-SCell-ToBeSetup-List								ProtocolIE-ID ::= 40

id-ResourceCoordinationTransferContainer				ProtocolIE-ID ::= 41

id-DRBs-ToBeModified-List							ProtocolIE-ID ::= 42

id-DRBs-ToBeReleased-List							ProtocolIE-ID ::= 43

id-DRBs-Modified-List								ProtocolIE-ID ::= 44

id-DRBs-FailedToBeModified-List						ProtocolIE-ID ::= 46

id-DRXCycle											ProtocolIE-ID ::= 48

id-DRBs-Setup-List									ProtocolIE-ID ::= 49

id-SRBs-Setup-List									ProtocolIE-ID ::= 50

id-DRBs-FailedToBeSetup-List							ProtocolIE-ID ::= 51

id-SRBs-FailedToBeSetup-List							ProtocolIE-ID ::= 52

id-TransmissionStopIndicator							ProtocolIE-ID ::= 55

id-DRBs-Required-ToBeModified-List					ProtocolIE-ID ::= 56

id-DRBs-Required-ToBeReleased-List					ProtocolIE-ID ::= 57

id-SRBs-Required-ToBeReleased-List					ProtocolIE-ID ::= 58

id-oldgNB-DU-F1AP-ID									ProtocolIE-ID ::= 59

id-SRBs-ToBeReleased-List							ProtocolIE-ID ::= 60

id-DRBs-ModifiedConf-List							ProtocolIE-ID ::= 61

id-SCell-ToBeSetupMod-List							ProtocolIE-ID ::= 63

id-SRBs-ToBeSetupMod-List							ProtocolIE-ID ::= 64

id-DRBs-ToBeSetupMod-List							ProtocolIE-ID ::= 65

id-DRBs-FailedToBeModifiedConf-List					ProtocolIE-ID ::= 69

id-GNB-DU-Served-Cells-Item							ProtocolIE-ID ::= 71

id-Cells-to-be-Activated-List-Item					ProtocolIE-ID ::= 72

id-Served-Cells-To-Modify-Item						ProtocolIE-ID ::= 73

id-Served-Cells-To-Delete-Item						ProtocolIE-ID ::= 74

id-Cells-to-be-Deactivated-List-Item					ProtocolIE-ID ::= 75

id-Cells-Failed-to-be-Activated-List-Item			ProtocolIE-ID ::= 76

id-SCell-ToBeSetup-Item								ProtocolIE-ID ::= 77

id-SRBs-ToBeSetup-Item								ProtocolIE-ID ::= 78

id-DRBs-Setup-Item									ProtocolIE-ID ::= 80

id-SRBs-FailedToBeSetup-Item							ProtocolIE-ID ::= 82

id-DRBs-FailedToBeSetup-Item							ProtocolIE-ID ::= 83

id-SCell-ToBeSetupMod-Item							ProtocolIE-ID ::= 84

id-SRBs-ToBeSetupMod-Item							ProtocolIE-ID ::= 85

id-DRBs-ToBeSetupMod-Item							ProtocolIE-ID ::= 86

id-DRBs-ToBeModified-Item							ProtocolIE-ID ::= 87

id-SRBs-ToBeReleased-Item							ProtocolIE-ID ::= 88

id-DRBs-ToBeReleased-Item							ProtocolIE-ID ::= 89

id-DRBs-SetupMod-Item								ProtocolIE-ID ::= 90

id-DRBs-Modified-Item								ProtocolIE-ID ::= 91

id-SRBs-FailedToBeSetupMod-Item						ProtocolIE-ID ::= 92

id-DRBs-FailedToBeSetupMod-Item						ProtocolIE-ID ::= 93

id-DRBs-FailedToBeModified-Item						ProtocolIE-ID ::= 94

id-DRBs-Required-ToBeModified-Item					ProtocolIE-ID ::= 95

id-DRBs-Required-ToBeReleased-Item					ProtocolIE-ID ::= 96

id-SRBs-Required-ToBeReleased-Item					ProtocolIE-ID ::= 97

id-DRBs-ModifiedConf-Item							ProtocolIE-ID ::= 98

id-DRBs-FailedToBeModifiedConf-Item					ProtocolIE-ID ::= 99

id-Served-Cells-To-Add-Item							ProtocolIE-ID ::= 100

id-DRBs-ToBeSetup-Item								ProtocolIE-ID ::= 101

id-SRBs-Setup-Item									ProtocolIE-ID ::= 102

id-DRBs-SetupMod-List								ProtocolIE-ID ::= 103

id-SRBs-FailedToBeSetupMod-List						ProtocolIE-ID ::= 104

id-DRBs-FailedToBeSetupMod-List						ProtocolIE-ID ::= 105

id-C-RNTI											ProtocolIE-ID ::= 106

id-SCell-ToBeRemoved-List							ProtocolIE-ID ::= 107

id-SCell-ToBeRemoved-Item								ProtocolIE-ID ::= 108

id-gNB-CU-Name										ProtocolIE-ID ::= xxx

END

End of Text Proposal for TS 38.473