3GPP TSG-RAN WG3 Meeting #99	R3-181559

# Athens, Greece, 26th Feburary- 2nd March  2018

Agenda Item:	10.5

Source:	Ericsson

Title:	Corrections to PDU Session Resource information

Document for:	pCR TS 38.413

# Introduction

In RAN3-99 a discussion was taken on the presence of the PDU Session Resource Setup List IE In the NG Initial Context Setup procedure.

It was identified that a the UE context Setup procedure is not consistently used between the f1 and NG interface. Over the F1 interface UE Context Setup procedure can be carried out only if DRBs need to be setup. However, over the NG interface the presence of the PDU Session Resource Setup List IE is optional.

No use cases have been identified to setup a UE context at the NG RAN if PDU Sessions do not need to be established at AS level. For this reason it is proposed to change the presence of the PDU Session Resource Setup List IE to mandatory.

# TP to TS38.413 v0.6.0

Start of Text Proposal 1 for TS 38.413

## 8.3	UE Context Management Procedures

### 8.3.1	Initial Context Setup

#### 8.3.1.1	General

The purpose of the Initial Context Setup procedure is to establish the necessary overall initial UE Context including PDU session context, the Security Key, Handover Restriction List, UE Radio Capability and UE Security Capabilities etc. The procedure uses UE-associated signalling.

Editor’s Note:	Further details are FFS.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: Initial context setup: successful operation

In case of the establishment of a PDU session the 5GC must be prepared to receive user data before the INITIAL CONTEXT SETUP RESPONSE message has been received by the AMF. If no UE-associated logical NG-connection exists, the UE-associated logical NG-connection shall be established at reception of the INITIAL CONTEXT SETUP REQUEST message.

The INITIAL CONTEXT SETUP REQUEST message shall contain the Subscriber Profile ID for RAT/Frequency Priority IE, if available in the AMF. [FFS pending RAN2]

If the Masked IMEISV IE is contained in the INITIAL CONTEXT SETUP REQUEST message the target NG-RAN node shall, if supported, use it to determine the characteristics of the UE for subsequent handling.

Upon receipt of the INITIAL CONTEXT SETUP REQUEST message the NG-RAN node shall

-	attempt to execute the requested PDU session configuration;

-	store the UE Aggregate Maximum Bit Rate in the UE context, and use the received UE Aggregate Maximum Bit Rate for non-GBR Bearers for the concerned UE; [FFS]

-	store the received Handover Restriction List in the UE context;

-	store the received UE Radio Capability in the UE context;

-	store the received Subscriber Profile ID for RAT/Frequency Priority in the UE context and use it as defined in TS 38.300 [8]; [FFS pending RAN2]

-	store the received UE Security Capabilities in the UE context;

-	store the received Security Key in the UE context, take it into use and associate it with the initial value of NCC as defined in TS xx.xxx [xx]. [FFS pending SA3]

For the Initial Context Setup an initial value for the Next Hop Chaining Count is stored in the UE context. [FFS pending SA3]

Regarding the PDU Session Resource Setup List IE contained in the INITIAL CONTEXT SETUP REQUEST message, the NG-RAN node shall behave the same as the one defined in the PDU Session Resource Setup procedure. The NG-RAN node shall report to the AMF, in the INITIAL CONTEXT SETUP RESPONSE message, the successful establishment of the result for all the requested PDU sessions. When the NG-RAN node reports the unsuccessful establishment of a PDU Session, the cause value should be precise enough to enable the AMF to know the reason for the unsuccessful establishment.

The NG-RAN node shall use the information in the Handover Restriction List IE if present in the INITIAL CONTEXT SETUP REQUEST message to

-	determine a target for subsequent mobility action for which the NG-RAN node provides information about the target of the mobility action towards the UE;

-	select a proper SCG during dual connectivity operation.

If the Handover Restriction List IE is not contained in the INITIAL CONTEXT SETUP REQUEST message, the NG-RAN node shall consider that no roaming and no access restriction apply to the UE. The NG-RAN node shall also consider that no roaming and no access restriction apply to the UE when:

-	one of the setup PDU sessions includes a particular ARP value (TS 23.501 [9]). [FFS, need to align with PDU Session Management]

If the Trace Activation IE is included in the INITIAL CONTEXT SETUP REQUEST message the NG-RAN node shall, if supported, initiate the requested trace function as described in TS 32.422 [11]. In particular, the NG-RAN node shall, if supported: [FFS pending RAN2 and SA5]

-	if the Trace Activation IE does not include the MDT Configuration IE, initiate the requested trace session as described in TS 32.422 [11];

-	if the Trace Activation IE includes the MDT Activation IE, within the MDT Configuration IE, set to “Immediate MDT and Trace”, initiate the requested trace session and MDT session as described in TS 32.422 [11];

-	if the Trace Activation IE includes the MDT Activation IE, within the MDT Configuration IE, set to “Immediate MDT Only”, or “Logged MDT only”, initiate the requested MDT session as described in TS 32.422 [11] and the NG-RAN node shall ignore the Interfaces To Trace IE and Trace Depth IE;

-	if the Trace Activation IE includes the MDT Location Information IE, within the MDT Configuration IE, store this information and take it into account in the requested MDT session;

-	if the Trace Activation IE includes the Signalling based MDT PLMN List IE, within the MDT Configuration IE, the NG-RAN node may use it to propagate the MDT Configuration as described in TS 37.320 [12].

If the UE Security Capabilities IE included in the INITIAL CONTEXT SETUP REQUEST message only contains the EIA0 algorithm as defined in TS 33.401 [13] and if this EIA0 algorithm is defined in the configured list of allowed integrity protection algorithms in the NG-RAN node (TS 33.401 [13]), the NG-RAN node shall take it into use and ignore the keys received in the Security Key IE. [FFS pending SA3]

If the Management Based MDT Allowed IE is contained in the INITIAL CONTEXT SETUP REQUEST message, the NG-RAN node shall use it, if supported, together with information in the Management Based MDT PLMN List IE, if available in the UE context, to allow subsequent selection of the UE for management based MDT defined in TS 32.422 [11]. [FFS pending RAN2 and SA5]

If the RRC Inactive Assistance Information IE is included in the INITIAL CONTEXT SETUP REQUEST message, the NG-RAN node shall, if supported, store this information in the UE context and use it for the RRC-INACTIVE state decision and configuration for the UE and RAN paging if any for a UE in RRC-INACTIVE state, as specified in TS 38.300 [8].

After sending the INITIAL CONTEXT SETUP RESPONSE message, the procedure is terminated in the NG-RAN node.

Editor’s Note:	Further details are FFS.

#### 8.3.1.3	Unsuccessful Operation

Figure 8.3.1.3-1: Initial context setup: unsuccessful operation

If the NG-RAN node is not able to establish an NG UE context, it shall consider the procedure as failed and reply with the INITIAL CONTEXT SETUP FAILURE message.

Editor’s Note:	Further details are FFS.

#### 8.3.1.4	Abnormal Conditions

Editor’s Note:	Further details are FFS.

End of Text Proposal 1  for TS 38.413

Start of Text Proposal 2 for TS 38.413

### 9.2.2	UE Context Management Messages

#### 9.2.2.1	INITIAL CONTEXT SETUP REQUEST

Editor’s Note:	Message structure and IEs need further checking and completion. Further details FFS.

This message is sent by the AMF to request the setup of a UE context.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| UE Aggregate Maximum Bit Rate | M |  | <ref> |  | YES | reject |
| RRC Inactive Assistance Information | M |  | 9.3.1.26 |  | YES | ignore |
| PDU Session Resource Setup List | M |  | <ref> | [FFS align with PDU Session management] | YES | reject |
| UE Security Capabilities | O [FFS] |  | <ref> |  | YES | reject |
| Security Key | O [FFS] |  | <ref> |  | YES | reject |
| Trace Activation | O |  | <ref> | [FFS pending RAN2 and SA5] | YES | ignore |
| Handover Restriction List | O |  | <ref> |  | YES | ignore |
| UE Radio Capability | O |  | <ref> |  | YES | ignore |
| Subscriber Profile ID for RAT/Frequency Priority | O |  | <ref> | [FFS] | YES | ignore |
| Management Based MDT Allowed | O |  | <ref> | [FFS pending RAN2 and SA5] | YES | ignore |
| Management Based MDT PLMN List | O |  | <ref> | [FFS pending RAN2 and SA5] | YES | ignore |
| Masked IMEISV | O |  | <ref> |  | YES | ignore |
| NAS-PDU | O |  | 9.3.3.4 | [FFS] | YES | ignore |

End of Text Proposal 2  for TS 38.413