3GPP TSG-RAN WG3 Meeting #99bis	R3-182149

# Sanya, P.R. China, 16th- 20th April  2018

Agenda Item:	10.1.1

Source:	Ericsson

Title:	Notification Control

Document for:	pCR

# Information

This pCR provides a new Notify procedure from S-NG-RAN node to M-NG-RAN node in XnAP.

# Text Proposal for NGAP TS 38.423 v0.7.0

<<<<<<<<<<<<<<<<<<<< Begin Text Proposal >>>>>>>>>>>>>>>>>>>>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ARFCN			Absolute Radio Frequency Channel Number

GFBR			Guaranteed Flow Bit Rate

M-NG-RAN node		Master NG-RAN node

PDB			Packet Delay Budget

PER			Packet Error Rate

S-NG-RAN node			Secondary NG-RAN node

SUL			Supplementary Uplink

------------------------------------------

Skip to next change

------------------------------------------

Table 7-1: Mapping between XnAP functions and XnAP EPs

| Function | Elementary Procedure(s) |
| --- | --- |
| Dual Connectivity | a) S-NG-RAN node Addition Preparation b) S-NG-RAN node Reconfiguration Completion c) M-NG-RAN node initiated S-NG-RAN node Modification Preparation d) S-NG-RAN node initiated S-NG-RAN node Modification e) M-NG-RAN node initiated S-NG-RAN node Release f) S-NG-RAN node initiated S-NG-RAN node Release g) S-NG-RAN node Counter Check h) RRC transfer i) Notification Control |
| Energy Saving | a) NG-RAN node Configuration Update b) Cell Activation |

------------------------------------------

Skip to next change

------------------------------------------

# Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| Handover Cancel | HANDOVER CANCEL |
| SN Status Transfer | SN STATUS TRANSFER |
| RAN Paging | RAN PAGING |
| Data Forwarding Address Indication | DATA FORWARDING ADDRESS INDICATION |
| S-NG-RAN node Reconfiguration Completion | S-NODE RECONFIGURATION COMPLETE |
| M-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUEST |
| S-NG-RAN node Counter Check | S-NODE COUNTER CHECK REQUEST |
| UE Context Release | UE CONTEXT RELEASE |
| RRC Transfer | RRC TRANSFER |
| Notification Control | NOTIFICATION CONTROL |

------------------------------------------

Skip to next change

------------------------------------------

### 8.3.X	Notification Control

#### 8.3.X.1	General

The purpose of the Notify procedure is to allow S-NG-RAN node to send notification to M-NG-RAN node for the already established QoS flow(s) or PDU session(s) for a given UE. The procedure uses UE-associated signalling.

#### 8.3.X.2	Successful Operation

Figure 8.3.X.2-1: Notification Control

The S-NG-RAN node initiates the procedure by sending a Notification Control message.

The Notification Control message shall contain notification for the already established PDU sessions or QoS flows which the GFBR, PDB or PER can no longer (or again) be fulfilled.

-	For each PDU session of which some QoS flows are not fulfilled any more or are fulfilled again by the S-NG-RAN node, the PDU Session Resource Notify Transfer IE shall be included to report for each PDU session of which the list of GBR QoS flows which the GFBR, PDB or PER can no longer (or again) be fulfilled by the S-NG-RAN node, if any, shall be included in the QoS Flows Notify List IE.

-	For each PDU session which is not fulfilled any more or be fulfilled again by the S-NG-RAN node, the PDU Session Resource Notify Transfer IE shall be included to report the cause in the Notification Cause IE.

Upon reception of the NOTIFICATION CONTROL message, the receiving M-NG-RAN node shall, for each PDU session indicated in the PDU Session ID IE, decide if to inform the 5GC that the PDU sessions or GBR QoS flows have fulfilled the condition for sending the notification for the QoS Notification Control purpose.

#### 8.3.X.3	Abnormal Conditions

Editor’s Note:	Further details are FFS.

------------------------------------------

Skip to next change

------------------------------------------

#### 9.1.X	Notification Control

This message is sent by a S-NG-RAN node to send notification to the M-NG-RAN node for the already established QoS flow(s) or PDU session(s) for a given UE

Direction: S-NG-RAN node  M-NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID <reference> | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID <reference> | Allocated at the S-NG-RAN node | YES | reject |
| PDU Session Resource Notify Item List |  | 1..<maxnoofPDUSessions> |  |  | EACH | reject |
| >PDU Session ID | M |  | <ref> |  | - |  |
| >PDU Session Resource Notify Transfer | M |  | 9.2.X |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions allowed towards one UE. Value is 256. |

------------------------------------------

Skip to next change

------------------------------------------

#### 9.2.X	PDU Session Resource Notify Transfer

This IE is used to indicate the Notification cause for the QoS flows or PDU sessions.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE PDU Session Resource Notify Transfer | M |  |  |  |
| >QoS Flows Notify |  |  |  |  |
| >>QoS Flows Notify List |  | 1..<maxnoofQoSFlows> |  |  |
| >>>QoS Flow Indicator | M |  | <ref> |  |
| >>>Notification Cause | M |  | 9.2.x.y |  |
| >PDU Session Resource Notify |  |  |  |  |
| >>Notification Cause | M |  | 9.2.x.y |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |

#### 9.2.x.y	Notification Cause

This IE contains the cause for sending the notification from NG-RAN node, refer to TS 23.501 [7].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Notification Cause | M |  | ENUMERATED (GFBR fulfilled, GFBR not fulfilled, PDB fulfilled, PDB not fulfilled, PER fulfilled, PER not fulfilled, …) |  |

<<<<<<<<<<<<<<<<<<<< End Text Proposal  >>>>>>>>>>>>>>>>>>>>