3GPP TSG-RAN WG3 Meeting #99bis	R3-182164

# Sanya, P.R. China, 16th- 20th April  2018

Agenda Item:	10.5.1

Source:	Ericsson

Title:	Correction of the concurrent running of security procedures

Document for:	pCR

# 1	Discussion

At the last SA3 meeting, SA3 has agreed in TS 33.501 to use K_AFM_CIK (KAMF Change Indicator) instead of NSCI.

This paper aligns with the SA3, Further the KAMF  Change Indicator IE is defined.

# Text Proposal for NGAP TS 38.413 v0.7.0

<<<<<<<<<<<<<<<<<<<< Begin Text Proposal >>>>>>>>>>>>>>>>>>>>

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core Network

AMF	Access and Mobility Management Function

EP	Elementary Procedure

GBR	Guaranteed Bit Rate

GTP	GPRS Tunnelling Protocol

IE	Information Element

K_AFM_CIK	KAMF  Change Indicator

NAS	Non-Access Stratum

NCGI	NR Cell Global Identifier

NGAP	NG Application Protocol

NRPPa	NR Positioning Protocol Annex

PLMN	Public Land Mobile Network

QoS	Quality of Service

SCTP	Stream Control Transmission Protocol

SMF	Session Management Function

S-NG-RAN node	Secondary NG-RAN node

S-NSSAI	Single Network Slice Selection Assistance Information

TEID	Tunnel Endpoint Identifier

UE	User Equipment

------------------------------------------

Skip to next change

------------------------------------------

### 8.4.2	Handover Resource Allocation

#### 8.4.2.1	General

The purpose of the Handover Resource Allocation procedure is to reserve resources at the target NG-RAN node for the handover of a UE.

Editor’s Note:	Further details are FFS.

#### 8.4.2.2	Successful Operation

Figure 8.4.2.2-1: Handover resource allocation: successful operation

The AMF initiates the procedure by sending the HANDOVER REQUEST message to the target NG-RAN node.

If the Masked IMEISV IE is contained in the HANDOVER REQUEST message the target NG-RAN node shall, if supported, use it to determine the characteristics of the UE for subsequent handling.

Upon receipt of the HANDOVER REQUEST message the target NG-RAN node shall

-	attempt to execute the requested PDU session configuration;

-	store the UE Aggregate Maximum Bit Rate in the UE context, and use the received UE Aggregate Maximum Bit Rate for non-GBR Bearers for the concerned UE; [FFS]

-	store the received Handover Restriction List in the UE context;

-	store the received UE Security Capabilities in the UE context;

-	store the received Security Key in the UE context, take it into use and associate it with the initial value of NCC as defined in TS xx.xxx [xx]; [FFS pending SA3]

For the Initial Context Setup an initial value for the Next Hop Chaining Count is stored in the UE context. [FFS pending SA3]

Upon receiving the PDU Session Resource To Be Setup List IE contained in the HANDOVER REQUEST message, the target NG-RAN node shall behave the same as defined in the PDU Session Resource Setup procedure.

The target NG-RAN node shall then report in the HANDOVER REQUEST ACKNOWLEDGE message the successful establishment of the result for all the requested PDU sessions. In particular, for each PDU session resource successfully setup, it shall include the PDU Session Resource Setup Response Transfer IE containing the following information:

-	The list of QoS flows which have been successfully established in the QoS Flows Setup List IE.

-	The list of QoS flows which have failed to be established, if any, in the QoS Flows Failed to Setup List IE.

-	The transport layer address and tunnel endpoint to be used for the PDU session.

The list of PDU session resources which failed to be setup, if any, shall be reported in the HANDOVER REQUEST ACKNOWLEDGE message with an appropriate cause value within the PDU Session Resource Setup Failed List IE.

Upon reception of the HANDOVER REQUEST ACKNOWLEDGE message the AMF shall, for each PDU session indicated in the PDU Session ID IE, transfer transparently the PDU Session Resource Setup Response Transfer IE to each SMF associated with the concerned PDU session.

When the target NG-RAN node reports unsuccessful establishment of a QoS flow, the cause value should be precise enough to know the reason for an unsuccessful establishment [FFS].

In case of inter-system handover from E-UTRAN, the target NG-RAN node includes the Data Forwarding Accepted IE for each QoS flow that the DL forwarding IE is set to “DL forwarding proposed” for the corresponding E-RAB in the Source NG-RAN Node to Target NG-RAN Node Transparent Container and that the target NG-RAN node has admitted the proposed forwarding of downlink data for the QoS flow. If the target NG-RAN node accepts the downlink data forwarding for at least one QoS flow of an admitted PDU session it shall include the DL Forwarding Transport Layer Information IE in the PDU Session Resource Setup Response Transfer IE for that PDU session within the PDU Sessions Admitted List IE of the HANDOVER REQUEST ACKNOWLEDGE message.

The target NG-RAN node shall use the information in the Handover Restriction List IE if present in the HANDOVER REQUEST message to

-	determine a target for subsequent mobility action for which the target NG-RAN node provides information about the target of the mobility action towards the UE;

-	select a proper SCG during dual connectivity operation.

If the Handover Restriction List IE is not contained in the HANDOVER REQUEST message, the target NG-RAN node shall consider that no roaming and no access restriction apply to the UE. The target NG-RAN node shall also consider that no roaming and no access restriction apply to the UE when:

-	one of the setup PDU sessions includes a particular ARP value (TS 23.501 [9]). [FFS, need to align with PDU Session Management]

If the Trace Activation IE is included in the HANDOVER REQUEST message the target NG-RAN node shall, if supported, initiate the requested trace function as described in TS 32.422 [11]. In particular, the target NG-RAN node shall, if supported: [FFS pending RAN2 and SA5]

-	if the Trace Activation IE does not include the MDT Configuration IE, initiate the requested trace session as described in TS 32.422 [11];

-	if the Trace Activation IE includes the MDT Activation IE, within the MDT Configuration IE, set to “Immediate MDT and Trace”, initiate the requested trace session and MDT session as described in TS 32.422 [11];

-	if the Trace Activation IE includes the MDT Activation IE, within the MDT Configuration IE, set to “Immediate MDT Only”, or “Logged MDT only”, initiate the requested MDT session as described in TS 32.422 [11] and the target NG-RAN node shall ignore the Interfaces To Trace IE and Trace Depth IE;

-	if the Trace Activation IE includes the MDT Location Information IE, within the MDT Configuration IE, store this information and take it into account in the requested MDT session;

-	if the Trace Activation IE includes the Signalling based MDT PLMN List IE, within the MDT Configuration IE, the target NG-RAN node may use it to propagate the MDT Configuration as described in TS 37.320 [12].

If the UE Security Capabilities IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall handle it accordingly. [FFS pending SA3]

If the Management Based MDT Allowed IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall use it, if supported, together with information in the Management Based MDT PLMN List IE, if available in the UE context, to allow subsequent selection of the UE for management based MDT defined in TS 32.422 [11]. [FFS pending RAN2 and SA5].

If the RRC Inactive Assistance Information IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store this information in the UE context and use it for the RRC-INACTIVE state decision and configuration for the UE and RAN paging if any for a UE in RRC-INACTIVE state, as specified in TS 38.300 [8].

If the KAMF Change IndicatorIE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use this K_AFM_CI  information as specified in TS 33.501 [13].

After all necessary resources for the admitted PDU Session Resources have been allocated, the target NG-RAN node shall generate the HANDOVER REQUEST ACKNOWLEDGE message.

Editor’s Note:	Further details are FFS.

#### 8.4.2.3	Unsuccessful Operation

Figure 8.4.2.3-1: Handover resource allocation: unsuccessful operation

If the target NG-RAN node does not admit at least one PDU Session Resource, or a failure occurs during the Handover Preparation, it shall send the HANDOVER FAILURE message to the AMF with an appropriate cause value.

Editor’s Note:	Further details are FFS.

#### 8.4.2.4	Abnormal Conditions

Editor’s Note:	Further details are FFS.

------------------------------------------

Skip to next change

------------------------------------------

### 8.4.4	Path Switch Request

#### 8.4.4.1	General

The purpose of the Path Switch Request procedure is to request the switch of a downlink GTP tunnel towards a new GTP tunnel endpoint.

Editor’s Note:	Further details are FFS.

#### 8.4.4.2	Successful Operation

Figure 8.4.4.2-1: Path switch request: successful operation

The NG-RAN node initiates the procedure by sending the PATH SWITCH REQUEST message to the AMF.

After all necessary updates including the UP path switch have been successfully completed in the 5GC for at least one of the PDU Session Resources included in the PATH SWITCH REQUEST, the AMF shall send the PATH SWITCH REQUEST ACKNOWLEDGE message to the NG-RAN node and the procedure ends.

The list of PDU sessions which failed to be setup, if any, shall be included in the PATH SWITCH REQUEST message. The AMF shall handle this information as specified in TS 23.502 [10].

If the RRC Inactive Assistance Information IE is included in the PATH SWITCH REQUEST ACKNOWLEDGE message, the NG-RAN node shall, if supported, store this information in the UE context and use it for the RRC-INACTIVE state decision and configuration for the UE and RAN paging if any for a UE in RRC-INACTIVE state, as specified in TS 38.300 [8].

If the KAMF Change Indicator IE is included in the PATH SWITCH REQUEST ACKNOWLEDGE message, the NG-RAN node shall, if supported, use this K_AFM_CI information as specified in TS 33.501 [13].

Editor’s Note:	Further details are FFS.

#### 8.4.4.3	Unsuccessful Operation

Figure 8.4.4.3-1: Path switch request: unsuccessful operation

If the 5GC fails to switch the downlink GTP tunnel endpoint towards a new GTP tunnel endpoint for all PDU Session Resources, the AMF shall send the PATH SWITCH REQUEST FAILURE message to the NG-RAN node with an appropriate cause value.

Editor’s Note:	Further details are FFS.

#### 8.4.4.4	Abnormal Conditions

Editor’s Note:	Further details are FFS.

------------------------------------------

Skip to next change

------------------------------------------

#### 9.2.3.4	HANDOVER REQUEST

Editor’s Note:	Message structure and IEs need further checking and completion. Further details FFS.

This message is sent by the AMF to the target NG-RAN node to request the preparation of resources.

Direction: AMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| Handover Type | M |  | 9.3.1.22 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| UE Aggregate Maximum Bit Rate | M |  | <ref> |  | YES | reject |
| RRC Inactive Assistance Information | M |  | 9.3.1.15 |  | YES | ignore |
| UE Security Capabilities | O [FFS] |  | <ref> |  | YES | reject |
| Security Key | O [FFS] |  | <ref> |  | YES | reject |
| KAMF Change Indicator | O |  | 9.3.x.y |  | YES | reject |
| PDU Session Resource To Be Setup List |  | 1 |  |  | YES | reject |
| >PDU Session Resource To Be Setup Item IEs |  | 1..<maxnoofPDUSessions> |  |  | EACH | reject |
| >>PDU Session ID | M |  | <ref> |  | - |  |
| >>S-NSSAI | O |  | 9.3.1.24 |  | - |  |
| >>Handover Request Transfer | M |  | PDU Session Resource Setup Request Transfer 9.3.4.1 |  | YES | ignore |
| Trace Activation | O |  | <ref> | [FFS pending RAN2 and SA5] | YES | ignore |
| Management Based MDT Allowed | O |  | <ref> | [FFS pending RAN2 and SA5] | YES | ignore |
| Management Based MDT PLMN List | O |  | <ref> | [FFS pending RAN2 and SA5] | YES | ignore |
| Masked IMEISV | O |  | <ref> |  | YES | ignore |
| Source to Target Transparent Container | M |  | 9.3.1.20 |  | YES | reject |
| Handover Restriction List | O |  | <ref> |  | YES | ignore |

------------------------------------------

Skip to next change

------------------------------------------

#### 9.2.3.9	PATH SWITCH REQUEST ACKNOWLEDGE

Editor’s Note:	Message structure and IEs need further checking and completion. Further details FFS.

This message is sent by the AMF to inform the NG-RAN node that the path switch has been successfully completed in the 5GS.

Direction: AMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| Security Context | M |  | <ref> |  | YES | ignore |
| KAMF Change Indicator | O |  | 9.3.x.y |  | YES | reject |
| PDU Session To Be Switched in Uplink List |  | 0..1 |  |  | YES | reject |
| >PDU Session To Be Switched in Uplink Item IEs |  | 1..<maxnoofPDUSessionResources> |  |  | EACH | reject |
| >>PDU Session ID | M |  | <ref> |  | - |  |
| >>Path Switch Request Acknowledge Transfer | M |  | 9.3.4.11 |  | YES | ignore |
| PDU Session Resource Released List | O |  | PDU Session List 9.3.1.11 |  | YES | reject |
| RRC Inactive Assistance Information | M |  | 9.3.1.15 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

------------------------------------------

Skip to next change

------------------------------------------

#### 9.3.x.y	KAMF Change Indicator

This IE indicates to if the K_AFM_CI information if presented or not.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| KAMF Change Indicator | M |  | ENUMERATED (true) |  |

<<<<<<<<<<<<<<<<<<<< End Text Proposal  >>>>>>>>>>>>>>>>>>>>