3GPP TSG-RAN WG3 Meeting #99bis	R3-182202

# Sanya, P.R China, 16th- 20th April  2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.472 | CR | 003 | rev |  | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of port number differentiation – CR 472 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-04-16 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The current text in TS 38.472 that describes the use of the different SCTP associations over F1 does not describe the possibility of using port numbers to differentiate associations, leading to the need of using expensive IP addresses to identify SCTP associations between a gNB-CU and a gNB-DU when multiple associations are configured. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrections to section 7: clarified the usage of the multiple SCTP associations with the possibility of using dynamically allocated port numbers. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description of the usage of multiple SCTP associations over F1 lacks the fundamental possibility of identifying associations by means of SCTP port numbers, leaving allocating multiple IP addresses at the gNB-CU side as the only possibility. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | CR xxxx |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# 7	Transport layer

SCTP (IETF RFC 4960 [5]) shall be supported as the transport layer of F1-C signalling bearer. The Payload Protocol Identifier assigned by IANA to be used by SCTP for the application layer protocol F1AP is 62.

SCTP refers to the Stream Control Transmission Protocol developed by the Sigtran working group of the IETF for the purpose of transporting various signalling protocols over IP network.

There shall be one or multiple SCTP associations established between one gNB-CU and gNB-DU pair.

The gNB-DU shall establish the SCTP association(s). The SCTP Destination Port number value for the first established SCTP association assigned by IANA to be used for F1AP is 38472. Additional SCTP associations are established using the port numbers received from the gNB-CU. The port numbers allocated by the gNB-CU shall be a choice between either 38472 or a dynamic port number in the range of from 49152 to 65535.