3GPP TSG-RAN WG3 #99bis	Tdoc R3-182233

Sanya, P. R. China, 16th- 20th April  2018

Agenda Item:	23.1

Source:	Ericsson

Title:	Bearer Context definition

Document for:	TP for NR Split-CP-UP BL CR for TS 38.401 (CP-UP)

# Introduction

At the previous RAN3 meeting it was agreed to introduce the definition of “Bearer Context” to identify a block UE-related of information that shall be used over the E1 interface between the gNB-CU-CP and the gNB-CU-UP. It was also agreed to introduce such definition in TS 38.401. In this contribution, we provide a definition of Bearer Context.

Proposal 1: 	RAN3 is kindly asked to agree with the definition of Bearer Context in Annex I.

# Annex: TP for BL CR TS 38.401 (CP-UP)

Start of Text Proposal for TS 38.463

6.4	UE associations in NG-RAN Node

There are several types of UE associations needed in the NG-RAN node: the "NG-RAN node UE context" used to store all information needed for a UE and the associations between the UE and the logical NG and Xn connections used for NG/XnAP UE associated messages. An "NG-RAN node UE context" exists for a UE in CM_CONNECTED.

Definitions:

NG-RAN node UE context:

An NG-RAN node UE context is a block of information in an NG-RAN node associated to one UE. The block of information contains the necessary information required to maintain the NG-RAN services towards the active UE. An NG-RAN node UE context is established when the transition to RRC CONNECTED for a UE is completed or in the target NG-RAN node after completion of handover resource allocation during handover preparation, in which case at least UE state information, security information, UE capability information and the identities of the UE-associated logical NG-connection shall be included in the NG-RAN node UE context.

For Dual Connectivity an NG-RAN node UE context is also established in the S-NG-RAN node after completion of S-NG-RAN node Addition Preparation procedure.

Bearer context:

A bearer context is a block of information in a gNB-CU-UP node associated to one UE that is used for the sake of communication over the E1 interface. It includes for example the information about data radio bearers, PDU sessions and QoS-flows associated to the UE. The block of information contains the necessary information required to maintain user-plane services toward the UE. The bearer context can be established by the gNB-CU-CP using the Bearer Context Setup procedure. The bearer context can be released by the gNB-CU-CP using the Bearer Context Release procedure, either autonomously or upon request from the gNB-CU-UP.

UE-associated logical NG/Xn/F1/E1-connection:

NGAP, XnAP, F1AP and E1AP provide means to exchange control plane messages associated with the UE over the respectively NG-C, Xn-C, F1-C or E1 interface.

A UE-associated logical connection is established during the first NGAP/XnAP/F1AP message exchange between the NG/Xn/F1 peer nodes. 
The connection is maintained as long as UE associated NG/XnAP/F1AP messages need to be exchanged over the NG/Xn/F1 interface.

The UE-associated logical NG-connection uses the identities AMF UE NGAP ID and RAN UE NGAP ID.

The UE-associated logical Xn-connection uses the identities Old NG-RAN node UE XnAP ID and New NG-RAN node UE XnAP ID, or M-NG-RAN node UE XnAP ID and S-NG-RAN node UE XnAP ID.

The UE-associated logical F1-connection uses the identities gNB-CU UE F1AP ID and gNB-DU UE F1AP ID.

The UE-associated logical E1-connection uses the identities gNB-CU-CP UE E1AP ID and gNB-CU-UP UE E1AP ID.

When a node (AMF or gNB) receives a UE associated NGAP/XnAP/F1AP/E1AP message the node retrieves the associated UE based on the NGAP/XnAP/F1AP/E1AP ID.

End of Text Proposal for TS 38.463