3GPP TSG RAN WG3 Meeting #99bis NR 	Tdoc R3-182441

Sanya, China, 16th – 20th April 2018

Agenda Item:	23.1

Source:	Ericsson

Title:	(TP for NR Split-CP-UP BL CR for TS 38.401) E1 procedures

Document for:	TP for NR BL CR for TS 38.401 (CP_UP split)

# Introduction

This text proposal is related to the following CB:

CB: # 49_St2BearerMgmt

-  remove statement: “to stop DL transmission…”

- Sec. 8.7.2.2.1, remove statement not needed

- CU-UP should not release bearers by itself

- procedure should be optional (adjust dotted lines as needed)

# Annex I

Start of Text Proposal 1 for TS 38.401

### 8.7.3	Inter-gNB handover involving gNB-CU-UP change

Figure 8.7.3-1 shows the procedure used for inter-gNB handover involving gNB-CU-UP change. Overall inter-gNB handover procedure is specified in TS 37.340 [12].

Figure 8.7.3-1: Inter-gNB handover involving gNB-CU-UP change

1.	The source gNB-CU-CP sends XN HANDOVER REQUEST message to the target gNB-CU-CP.

2-4.	Bearer context setup procedure is performed as described in Section 8.7.1.

5.	The target gNB-CU-CP responds the source gNB-CU-CP with an XN HANDOVER REQUEST ACKNOWLEDGE message.

6. The F1 UE Context Modification procedure is performed to stop UL data transmission at the gNB-DU and to send the handover command to the UE.

7-8.	Bearer context modification procedure (gNB-CU-CP initiated) is performed to enable the gNB-CU-CP to retrieve the PDCP UL/DL status and to exchange data forwarding information for the bearer.

9.	The source gNB-CU-CP sends an SN STATUS TRANSFER message to the target gNB-CU-CP.

10-11.	Bearer context modification procedure is performed as described in Section 8.7.1.

12.	Data Forwarding may be performed from the source gNB-CU-UP to the target gNB-CU-UP.

13-15.	Path Switch procedure is performed to update the DL TNL address information for the NG-U towards the core network.

16.	The target gNB-CU-CP sends an UE CONTEXT RELEASE message to the source gNB-CU-CP.

17 and 19. Bearer context release procedure is performed.

18	F1 UE context release procedure is performed to release the UE context in the source gNB-DU.

Start of Text Proposal2 for TS 38.401

#### 8.7.2.1	gNB-CU-CP initiated bearer context release

Figure 8.7.2.1-1 shows the procedure used to release the bearer context in the gNB-CU-UP initiated by the gNB-CU-CP.

Figure 8.7.2.1-1: Bearer context release over F1-U – gNB-CU-CP initiated

0.	Bearer context release (e.g., following an SgNB Release Request from the MeNB) is triggered in gNB-CU-CP.

1.	The gNB-CU-CP sends a BEARER CONTEXT MODIFICATION REQUEST  message to the gNB-CU-UP.

2.	The gNB-CU-UP responds with a BEARER CONTEXT MODIFICATION RESPONSE carrying the PDCP UL/DL status.

3.	F1 UE context modification procedure is performed to stop the data transmission for the UE. It is up to gNB-DU implementation when to stop the UE scheduling.

NOTE: step 1-3 are performed only if the PDCP status of the bearer(s) needs to be preserved e.g., for bearer type change.

4.	The gNB-CU-CP may receive the UE CONTEXT RELEASE message from the MeNB in EN-DC operation as described in Section 8.4.2.1.

5 and 7.	 Bearer context release procedure is performed.

6. F1 UE context release procedure is performed to release the UE context in the gNB-DU.

Start of Text Proposal3 for TS 38.401

#### 8.7.2.2	gNB-CU-UP initiated bearer context release

Figure 8.7.2.2-1 shows the procedure used to release the bearer context in the gNB-CU-UP initiated by the gNB-CU-UP.

Figure 8.7.2.2-1: Bearer context release over F1-U – gNB-CU-UP initiated

0.	Bearer context release is triggered in gNB-CU-UP e.g., due to local failure.

1.	The gNB-CU-UP sends a BEARER CONTEXT RELEASE REQUEST message to request the release of the bearer context in the gNB-CU-UP. This message may contain the PDCP status.

2.- 5. If the PDCP status needs to be preserved, the E1 Bearer Context Modification and the F1 UE Context Modification procedures are performed. The E1 Bearer Context Modification procedure is used to convey data forwarding information to the gNB-CU-UP. The gNB-CU-CP may receive the UE Context Release from the MeNB.

6. The gNB-CU-CP sends a BEARER CONTEXT RELEASE COMMAND message to release the bearer context in the gNB-CU-UP.

7.	The gNB-CU-UP responds with a BEARER CONTEXT RELEASE COMPLETE to confirm the release of the bearer context including also data forwarding information.

8.	F1 UE context release procedure may be performed to release the UE context in the gNB-DU.

End of Text Proposal for TS 38.401