3GPP TSG-RAN WG3 Meeting #99bis	R3-182404

# Sanya, P.R China, 16th- 20th April  2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.413 | CR | 1587 | rev | 1 | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of the Aerial Usage Indication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, NTT Docomo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_Aerial |  |  |  |  |  |  | Date: |  | 2018-04-06 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The Aerial UE subscription information from the subscription information is needed to be sent from CN to RAN during connection setup and mobility procedures for supporting the Work Item LTE_Aerial. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the “Aerial UE subscription information” in the below procedures and messages: INITIAL CONTEXT SETUP HANDOVER REQUEST UE CONTEXT MODIFICATION REQUEST PATH SWITCH REQUEST ACKNOWLEDGE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Aerial UE subscription information from the subscirption information is not conveyed from CN to RAN in Rel 15. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.1, 8.3.1.2, 8.4.2, 8.4.2.2, 9.1.4.1, 9.1.5.4, 9.2.1.xxx, 9.3.3, 9.3.4, 9.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 36.423 CR 1132 |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 8.3	Context Management procedures

### 8.3.1	Initial Context Setup

#### 8.3.1.1	General

The purpose of the Initial Context Setup procedure is to establish the necessary overall initial UE Context including E-RAB context, the Security Key, Handover Restriction List, UE Radio capability and UE Security Capabilities etc. The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: Initial Context Setup procedure. Successful operation.

In case of the establishment of an E-RAB the EPC must be prepared to receive user data before the INITIAL CONTEXT SETUP RESPONSE message has been received by the MME. If no UE-associated logical S1-connection exists, the UE-associated logical S1-connection shall be established at reception of the INITIAL CONTEXT SETUP REQUEST message.

The INITIAL CONTEXT SETUP REQUEST message shall contain within the E-RAB to be Setup List IE the information required by the eNB to build the new E-RAB configuration consisting of at least one additional E-RAB.

The E-RAB to be Setup Item IE may contain:

-	the NAS-PDU IE,

-	the Correlation ID IE in case of LIPA operation,

-	the SIPTO Correlation ID IE in case of SIPTO@LN operation,

-	the Bearer Type IE.

The INITIAL CONTEXT SETUP REQUEST message may contain

-	the Trace Activation IE.

-	the Handover Restriction List IE, which may contain roaming or access restrictions.

-	the UE Radio Capability IE.

-	the Subscriber Profile ID for RAT/Frequency priority IE.

-	the CS Fallback Indicator IE.

-	the SRVCC Operation Possible IE.

-	the CSG Membership Status IE.

-	the Registered LAI IE.

-	the GUMMEI IE, which indicates the MME serving the UE, and shall only be present according to subclauses 4.6.2 and 4.7.6.6 of TS 36.300 [14].

-	the MME UE S1AP ID 2 IE, which indicates the MME UE S1AP ID assigned by the MME, and shall only be present according to subclause 4.6.2 of TS 36.300 [14].

-	the Management Based MDT Allowed IE.

-	the Management Based MDT PLMN List IE.

-	the Additional CS Fallback Indicator IE.

-	the Masked IMEISV IE.

-	the Expected UE Behaviour IE.

-	the ProSe Authorized IE.

-	the UE User Plane CIoT Support Indicator IE.

-	the V2X Services Authorized IE.

-	the UE Sidelink Aggregate Maximum Bit Rate IE.

-	the NR UE Security Capabilities IE.

-	the Aerial UE subscription information IE.

The INITIAL CONTEXT SETUP REQUEST message shall contain the Subscriber Profile ID for RAT/Frequency priority IE, if available in the MME.

If the Correlation ID IE is included in the INITIAL CONTEXT SETUP REQUEST message towards the eNB with L-GW function for LIPA operation, then the eNB shall use this information for LIPA operation for the concerned E-RAB.

If the SIPTO Correlation ID IE is included in the INITIAL CONTEXT SETUP REQUEST message towards the eNB with L-GW function for SIPTO@LN operation, then the eNB shall use this information for SIPTO@LN operation for the concerned E-RAB.

If the Bearer Type IE is included in the INITIAL CONTEXT SETUP REQUEST message and is set to “non IP”, then the eNB shall not perform header compression for the concerned E-RAB.

If the Masked IMEISV IE is contained in the INITIAL CONTEXT SETUP REQUEST the target eNB shall, if supported, use it to determine the characteristics of the UE for subsequent handling.

If the Expected UE Behaviour IE is included in the INITIAL CONTEXT SETUP REQUEST message, the eNB shall, if supported, store this information and may use it to determine the RRC connection time.

Upon receipt of the INITIAL CONTEXT SETUP REQUEST message the eNB shall

-	attempt to execute the requested E-RAB configuration.

-	store the UE Aggregate Maximum Bit Rate in the UE context, and use the received UE Aggregate Maximum Bit Rate for non-GBR Bearers for the concerned UE.

-	pass the value contained in the E-RAB ID IE and the NAS-PDU IE received for the E-RAB for each established Data radio bearer to the radio interface protocol. The eNB shall not send the NAS PDUs associated to the failed Data radio bearers to the UE.

-	store the received Handover Restriction List in the UE context.

-	store the received UE Radio Capability in the UE context.

-	store the received Subscriber Profile ID for RAT/Frequency priority in the UE context and use it as defined in TS 36.300 [14].

-	store the received SRVCC Operation Possible in the UE context and use it as defined in TS 23.216 [9].

-	store the received UE Security Capabilities in the UE context.

-	store the received Security Key in the UE context, take it into use and associate it with the initial value of NCC as defined in TS 33.401 [15].

-	store the received CSG Membership Status, if supported, in the UE context.

-	store the received Management Based MDT Allowed information, if supported, in the UE context.

-	store the received Management Based MDT PLMN List information, if supported, in the UE context.

-	store the received ProSe Authorization information, if supported, in the UE context.

-	store the received V2X Services Authorization information, if supported, in the UE context.

-	store the received UE Sidelink Aggregate Maximum Bit Rate, if supported, in the UE context, and use it for the concerned UE’s sidelink communication in network scheduled mode for V2X services.

For the Initial Context Setup an initial value for the Next Hop Chaining Count is stored in the UE context.

The allocation of resources according to the values of the Allocation and Retention Priority IE shall follow the principles described for the E-RAB Setup procedure.

The eNB shall use the information in the Handover Restriction List IE if present in the INITIAL CONTEXT SETUP REQUEST message to

-	determine a target for subsequent mobility action for which the eNB provides information about the target of the mobility action towards the UE, except if the CS Fallback Indicator IE is set to “CS Fallback High Priority” and the Additional CS Fallback Indicator IE is not present in which case the eNB may use the information in the Handover Restriction List IE;

-	select a proper SCG during dual connectivity operation.

If the Handover Restriction List IE is not contained in the INITIAL CONTEXT SETUP REQUEST message, the eNB shall consider that no roaming and no access restriction apply to the UE. The eNB shall also consider that no roaming and no access restriction apply to the UE when:

-	one of the setup E-RABs has a particular ARP value (TS 23.401 [11]);

-	the CS Fallback Indicator IE is set to “CS Fallback High Priority” and the Additional CS Fallback Indicator IE is not present and, in case the Handover Restriction List IE is applied, no suitable target is found, in which case it shall process according to TS 23.272 [17];

-	the CS Fallback Indicator IE is set to “CS Fallback High Priority” and the Additional CS Fallback Indicator IE is set to “no restriction”, in which case it shall process according to TS 23.272 [17].

If the Trace Activation IE is included in the INITIAL CONTEXT SETUP REQUEST message then eNB shall, if supported, initiate the requested trace function as described in TS 32.422 [10]. In particular, the eNB shall, if supported:

-	if the Trace Activation IE does not include the MDT Configuration IE, initiate the requested trace session as described in TS 32.422 [10];

-	if the Trace Activation IE includes the MDT Activation IE, within the MDT Configuration IE, set to “Immediate MDT and Trace”, initiate the requested trace session and MDT session as described in TS 32.422 [10];

-	if the Trace Activation IE includes the MDT Activation IE ,within the MDT Configuration IE, set to “Immediate MDT Only”, “Logged MDT only” or “Logged MBSFN MDT”, initiate the requested MDT session as described in TS 32.422 [10] and the eNB shall ignore Interfaces To Trace IE, and Trace Depth IE.

-	if the Trace Activation IE includes the MDT Location Information IE, within the MDT Configuration IE, store this information and take it into account in the requested MDT session.

-	if the Trace Activation IE includes the Signalling based MDT PLMN List IE, within the MDT Configuration IE, the eNB may use it to propagate the MDT Configuration as described in TS 37.320 [31].

-	if the Trace Activation IE includes the MBSFN-ResultToLog IE, within the MDT Configuration IE, take it into account for MDT Configuration as described in TS 37.320 [31].

-	if the Trace Activation IE includes the MBSFN-AreaId IE in the MBSFN-ResultToLog IE, within the MDT Configuration IE, take it into account for MDT Configuration as described in TS 37.320 [31].

-	if the Trace Activation IE includes the UE Application layer measurement configuration IE, initiate the requested trace session and QoE Measurement Collection function as described in TS 36.300 [14].

If the CS Fallback Indicator IE is included in the INITIAL CONTEXT SETUP REQUEST message, it indicates that the UE Context to be set-up is subject to CS Fallback. The eNB shall reply with the INITIAL CONTEXT SETUP RESPONSE message and then act as defined in TS 23.272 [17].

If the Registered LAI IE is included in the INITIAL CONTEXT SETUP REQUEST message, it indicates that the eNB may take the Registered LAI IE into account when selecting the target cell or frequency and then act as defined in TS 23.272 [17].

If the UE Security Capabilities IE included in the INITIAL CONTEXT SETUP REQUEST message only contains the EIA0 algorithm as defined in TS 33.401 [15] and if this EIA0 algorithm is defined in the configured list of allowed integrity protection algorithms in the eNB (TS 33.401 [15]), the eNB shall take it into use and ignore the keys received in the Security Key IE.

If the GUMMEI IE is contained in the INITIAL CONTEXT SETUP REQUEST message, the eNB shall, if supported, store this information in the UE context and use it for subsequent X2 handovers.

If the MME UE S1AP ID 2 IE is contained in the INITIAL CONTEXT SETUP REQUEST message, the eNB shall, if supported, store this information in the UE context and use it for subsequent X2 handovers.

If the Management Based MDT Allowed IE is contained in the INITIAL CONTEXT SETUP REQUEST message, the eNB shall use it, if supported, together with information in the Management Based MDT PLMN List IE, if available in the UE context, to allow subsequent selection of the UE for management based MDT defined in TS 32.422 [10].

If the UE User Plane CIoT Support Indicator IE is included in the INITIAL CONTEXT SETUP REQUEST message and is set to "supported", the eNB shall, if supported, consider that User Plane CIoT EPS Optimisation as specified in TS 23.401[11] is supported for the UE.

If the Enhanced Coverage Restricted IE is included in the INITIAL CONTEXT SETUP REQUEST message, the eNB shall store this information in the UE context and use it as defined in TS 23.401 [11].

If the CE-Mode-B Restricted IE is included in the INITIAL CONTEXT SETUP REQUEST message and the Enhanced Coverage Restricted IE is not set to restricted and the Enhanced Coverage Restricted information stored in the UE context is not set to restricted, the eNB shall store this information in the UE context and use it as defined in TS 23.401 [11].

If the NR UE Security Capabilities IE is included in the INITIAL CONTEXT SETUP REQUEST message, the eNB shall, if supported, store this information in the UE context and use it as defined in TS 33.401 [15].

If the Aerial UE subscription information IE is included in the INITIAL CONTEXT SETUP REQUEST message, the eNB shall, if supported, store this information in the UE context and use it as defined in TS 36.300 [14].

The eNB shall report to the MME, in the INITIAL CONTEXT SETUP RESPONSE message, the successful establishment of the security procedures with the UE, and, the result for all the requested E-RABs in the following way:

-	A list of E-RABs which are successfully established shall be included in the E-RAB Setup List IE

-	A list of E-RABs which failed to be established shall be included in the E-RAB Failed to Setup List IE.

When the eNB reports the unsuccessful establishment of an E-RAB, the cause value should be precise enough to enable the MME to know the reason for the unsuccessful establishment, e.g., “Radio resources not available”, “Failure in the Radio Interface Procedure”.

After sending the INITIAL CONTEXT SETUP RESPONSE message, the procedure is terminated in the eNB.

#### 8.3.1.3	Unsuccessful Operation

Figure 8.3.1.3-1: Initial Context Setup procedure. Unsuccessful operation.

If the eNB is not able to establish an S1 UE context, or cannot even establish one non GBR bearer it shall consider the procedure as failed and reply with the INITIAL CONTEXT SETUP FAILURE message.

#### 8.3.1.4	Abnormal Conditions

If the eNB receives an INITIAL CONTEXT SETUP REQUEST message containing a E-RAB Level QoS Parameters IE which contains a QCI IE indicating a GBR bearer (as defined in TS 23.203 [13]), and which does not contain the GBR QoS Information IE, the eNB shall consider the establishment of the corresponding E-RAB as failed.

If the eNB receives an INITIAL CONTEXT SETUP REQUEST message containing several E-RAB ID IEs (in the E-RAB to Be Setup List IE) set to the same value, the eNB shall consider the establishment of the corresponding E-RABs as failed.

If the supported algorithms for encryption defined in the Encryption Algorithms IE in the UE Security Capabilities IE, plus the mandated support of EEA0 in all UEs (TS 33.401 [15]), do not match any allowed algorithms defined in the configured list of allowed encryption algorithms in the eNB (TS 33.401 [15]), the eNB shall reject the procedure using the INITIAL CONTEXT SETUP FAILURE message.

If the supported algorithms for integrity defined in the Integrity Protection Algorithms IE in the UE Security Capabilities IE, plus the mandated support of the EIA0 algorithm in all UEs (TS 33.401 [15]), do not match any allowed algorithms defined in the configured list of allowed integrity protection algorithms in the eNB (TS 33.401 [15]), the eNB shall reject the procedure using the INITIAL CONTEXT SETUP FAILURE message.

If the CSG Membership Status IE is not included in the INITIAL CONTEXT SETUP REQUEST message and the cell accessed by the UE is a hybrid cell, the eNB shall reject the procedure using the INITIAL CONTEXT SETUP FAILURE message.

If the eNB receives an INITIAL CONTEXT SETUP REQUEST message containing both the Correlation ID and the SIPTO Correlation ID IEs for the same E-RAB, the eNB shall consider the establishment of the corresponding E-RAB as failed.

------------------------------------------

Skip to next change

------------------------------------------

### 8.3.4	UE Context Modification

#### 8.3.4.1	General

The purpose of the UE Context Modification procedure is to partly modify the established UE Context, e.g., with the Security Key or the Subscriber Profile ID for RAT/Frequency priority. The procedure uses UE-associated signalling.

#### 8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE Context Modification procedure. Successful operation.

The UE CONTEXT MODIFICATION REQUEST message may contain.

-	the Security Key IE.

-	the Subscriber Profile ID for RAT/Frequency priority IE.

-	the UE Aggregate Maximum Bit Rate IE.

-	the CS Fallback Indicator IE.

-	the UE Security Capabilities IE.

-	the CSG Membership Status IE.

-	the Registered LAI IE.

-	the Additional CS Fallback Indicator IE.

-	the ProSe Authorized IE.

-	the SRVCC Operation Possible IE.

-	the SRVCC Operation Not Possible IE.

-	the V2X Services Authorized IE.

-	the UE Sidelink Aggregate Maximum Bit Rate IE.

-	the NR UE Security Capabilities IE.

-	the Aerial UE subscription information IE.

Upon receipt of the UE CONTEXT MODIFICATION REQUEST message the eNB shall

-	store the received Security Key IE, take it into use and associate it with the initial value of NCC as defined in TS 33.401 [15]

-	store the UE Security Capabilities IE and take them into use together with the received keys according to TS 33.401 [15].

-	if supported, store the NR UE Security Capabilities IE and use it as defined in TS 33.401 [15]

-	store the Subscriber Profile ID for RAT/Frequency priority IE and use it as defined in TS 36.300 [14].

If the UE Aggregate Maximum Bit Rate IE is included in the UE CONTEXT MODIFICATION REQUEST message the eNB shall:

-	replace the previously provided UE Aggregate Maximum Bit Rate by the received UE Aggregate Maximum Bit Rate in the UE context;

-	use the received UE Aggregate Maximum Bit Rate for non-GBR Bearers for the concerned UE.

If the CSG Membership Status IE is received in the UE CONTEXT MODIFICATION REQUEST message, the eNB shall take the following action:

-	If the cell that serves the UE is a hybrid cell, the eNB shall store the value contained in the CSG Membership Status IE and replace any previously stored membership status value by this new one. It shall then use it as defined in TS 36.300 [14].

-	If the cell that serves the UE is a CSG cell, and the CSG Membership Status IE is set to “not-member”, the eNB should initiate actions to ensure that the UE is no longer served by the CSG cell as defined in TS 36.300 [14].

-	If the UE is in dual connectivity operation and the cell configured as SCG is a hybrid cell, the eNB shall inform the eNB serving the SCG of the updated CSG membership status.

If the UE Aggregate Maximum Bit Rate IE is not contained in the UE CONTEXT MODIFICATION REQUEST message, the eNB shall use the previously provided UE Aggregate Maximum Bit Rate which is stored in the UE context.

If the CS Fallback Indicator IE is included in the UE CONTEXT MODIFICATION REQUEST message, it indicates that the concerned UE Context is subject to CS Fallback. The eNB shall reply with the UE CONTEXT MODIFICATION RESPONSE message and then act as defined in TS 23.272 [17]. If the CS Fallback Indicator IE is set to “CS Fallback High Priority” and the Additional CS Fallback Indicator IE is not present and, in case the Handover Restriction List information that may exist in the UE context is applied, no suitable target is found, or if the CS Fallback Indicator IE is set to “CS Fallback High Priority” and the Additional CS Fallback Indicator IE is set to “no restriction”, the eNB shall consider that no roaming and no access restriction apply to the UE and process according to TS 23.272 [17].

If the Registered LAI IE is included in the UE CONTEXT MODIFICATION REQUEST message, it indicates that the eNB may take the Registered LAI IE into account when selecting the target cell or frequency and then act as defined in TS 23.272 [17].

If the ProSe Authorized IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the eNB shall, if supported, update its ProSe authorization information for the UE accordingly. If the ProSe Authorized IE includes one or more IEs set to “not authorized”, the eNB shall, if supported, initiate actions to ensure that the UE is no longer accessing the relevant ProSe service(s).

If the SRVCC Operation Possible IE is included in UE CONTEXT MODIFICATION REQUEST message, the eNB shall store content of the received SRVCC Operation Possible IE in the UE context and, if supported, use it as defined in TS 23.216 [9].

If the SRVCC Operation Not Possible IE is included in UE CONTEXT MODIFICATION REQUEST message, the eNB shall, if supported, remove the SRVCC Operation Possible information from the UE context.

If the V2X Services Authorized IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the eNB shall, if supported, update its V2X services authorization information for the UE accordingly. If the V2X Services Authorized IE includes one or more IEs set to “not authorized”, the eNB shall, if supported, initiate actions to ensure that the UE is no longer accessing the relevant service(s).

If the UE Sidelink Aggregate Maximum Bit Rate IE is included in the UE CONTEXT MODIFICATION REQUEST message, the eNB shall, if supported:

-	replace the previously provided UE Sidelink Aggregate Maximum Bit Rate, if available in the UE context, with the received value;

-	use the received value for the concerned UE’s sidelink communication in network scheduled mode for V2X services.

If the Aerial UE subscription information IE is included in the UE CONTEXT MODIFICATION REQUEST message, the eNB shall, if supported, store this information in the UE context and use it as defined in TS 36.300 [14].

The eNB shall report, in the UE CONTEXT MODIFICATION RESPONSE message to the MME the successful update of the UE context.

After sending the UE CONTEXT MODIFICATION RESPONSE message, the procedure is terminated in the eNB.

#### 8.3.4.3	Unsuccessful Operation

Figure 8.3.4.3-1: UE Context Modification procedure. Unsuccessful operation.

In case the UE context update cannot be performed successfully the eNB shall respond with the UE CONTEXT MODIFICATION FAILURE message to the MME with an appropriate cause value in the Cause IE.

#### 8.3.4.4	Abnormal Conditions

If the eNB receives both the CS Fallback Indicator IE and one of the security IEs (either the Security Key IE or the UE Security Capabilities IE) in the UE Context Modification Request message, the eNB shall ignore both IEs and send back the UE CONTEXT MODIFICATION FAILURE message with an appropriate cause value.

------------------------------------------

Skip to next change

------------------------------------------

### 8.4.2	Handover Resource Allocation

#### 8.4.2.1	General

The purpose of the Handover Resource Allocation procedure is to reserve resources at the target eNB for the handover of a UE.

#### 8.4.2.2	Successful Operation

Figure 8.4.2.2-1: Handover resource allocation: successful operation

The MME initiates the procedure by sending the HANDOVER REQUEST message to the target eNB. The HANDOVER REQUEST message may contain the Handover Restriction List IE, which contains roaming or access restrictions.

If the Handover Restriction List IE is contained in the HANDOVER REQUEST message, the target eNB shall store this information in the UE context. This information shall however not be considered whenever one of the handed over E-RABs has a particular ARP value (TS 23.401 [11]).

The target eNB shall use the information in Handover Restriction List IE if present in the HANDOVER REQUEST message to

-	determine a target for subsequent mobility action for which the eNB provides information about the target of the mobility action towards the UE;

-	select a proper SCG during dual connectivity operation.

If the Handover Restriction List IE is not contained in the HANDOVER REQUEST message, the target eNB shall consider that no roaming and no access restriction apply to the UE.

Upon reception of the HANDOVER REQUEST message the eNB shall store the received UE Security Capabilities IE in the UE context and use it to prepare the configuration of the AS security relation with the UE.

If the SRVCC Operation Possible IE is included in the HANDOVER REQUEST message, the target eNB shall store the content of the received SRVCC Operation Possible IE in the UE context and, if supported, use it as defined in TS 23.216 [9].

Upon reception of the HANDOVER REQUEST message the eNB shall store the received Security Context IE in the UE context and the eNB shall use it to derive the security configuration as specified in TS 33.401 [15].

If the Trace Activation IE is included in the HANDOVER REQUEST message, the target eNB shall if supported, initiate the requested trace function as described in TS 32.422 [10]. In particular, the eNB shall, if supported:

-	if the Trace Activation IE does not include the MDT Configuration IE, initiate the requested trace session as described in TS 32.422 [10];

-	if the Trace Activation IE includes the MDT Activation IE, within the MDT Configuration IE, set to “Immediate MDT and Trace”, initiate the requested trace session and MDT session as described in TS 32.422 [10];

-	if the Trace Activation IE includes the MDT Activation IE, within the MDT Configuration IE, set to “Immediate MDT Only”, “Logged MDT only” or “Logged MBSFN MDT”, initiate the requested MDT session as described in TS 32.422 [10] and the target eNB shall ignore Interfaces To Trace IE, and Trace Depth IE.

-	if the Trace Activation IE includes the MDT Location Information IE, within the MDT Configuration IE, store this information and take it into account in the requested MDT session.

-	if the Trace Activation IE includes the Signalling based MDT PLMN List IE, within the MDT Configuration IE, the eNB may use it to propagate the MDT Configuration as described in TS 37.320 [31].

-	if the Trace Activation IE includes the MBSFN-ResultToLog IE, within the MDT Configuration IE, take it into account for MDT Configuration as described in TS 37.320 [31].

-	if the Trace Activation IE includes the MBSFN-AreaId IE in the MBSFN-ResultToLog IE, within the MDT Configuration IE, take it into account for MDT Configuration as described in TS 37.320 [31].

-	if the Trace Activation IE includes the UE Application layer measurement configuration IE, initiate the requested trace session and QoE Measurement Collection function as described in TS 36.300 [14].

If the CSG Id IE is received in the HANDOVER REQUEST message, the eNB shall compare the received value with the CSG Id broadcast by the target cell.

If the CSG Membership Status IE is received in the HANDOVER REQUEST message and the CSG Membership Status is set to “member”, the eNB may provide the QoS to the UE as for member provided that the CSG Id received in the HANDOVER REQUEST messages corresponds to the CSG Id broadcast by the target cell.

If the CSG Membership Status IE and the CSG Id IE are received in the HANDOVER REQUEST message and the CSG Id does not correspond to the CSG Id broadcast by the target cell, the eNB may provide the QoS to the UE as for a non member and shall send back in the HANDOVER REQUEST ACKNOWLEDGE message the actual CSG Id broadcast by the target cell.

If the target cell is CSG cell or hybrid cell, the target eNB shall include the CSG ID IE in the HANDOVER REQUEST ACKNOWLEDGE message.

If the target eNB receives the CSG Id IE and the CSG Membership Status IE is set to “non member” in the HANDOVER REQUEST message and the target cell is a closed cell and at least one of the E-RABs has a particular ARP value (see TS 23.401 [11]), the eNB shall send back the HANDOVER REQUEST ACKNOWLEDGE message to the MME accepting those E-RABs and failing the other E-RABs.

If the Subscriber Profile ID for RAT/Frequency priority IE is contained in the Source eNB to Target eNB Transparent Container IE, the target eNB shall store the content of the received Subscriber Profile ID for RAT/Frequency priority IE in the UE context and use it as defined in TS 36.300 [14].

Upon reception of the UE History Information IE, which is included within the Source eNB to Target eNB Transparent Container IE in the HANDOVER REQUEST message, the target eNB shall collect the information defined as mandatory in the UE History Information IE and shall, if supported, collect the information defined as optional in the UE History Information IE, for as long as the UE stays in one of its cells, and store the collected information to be used for future handover preparations.

Upon reception of the UE History Information from the UE IE, which is included within the Source eNB to Target eNB Transparent Container IE in the HANDOVER REQUEST message, the target eNB shall, if supported, store the collected information, to be used for future handover preparations.

If the Mobility Information IE is included within the Source eNB to Target eNB Transparent Container IE in the HANDOVER REQUEST message, the target eNB shall, if supported, store this information and use it as defined in TS 36.300 [14].

If the Expected UE Behaviour IE is included in the HANDOVER REQUEST message, the eNB shall, if supported, store this information and may use it to determine the RRC connection time.

If the Bearer Type IE is included in the HANDOVER REQUEST message and is set to “non IP”, then the eNB shall not perform header compression for the concerned E-RAB.

After all necessary resources for the admitted E-RABs have been allocated, the target eNB shall generate the HANDOVER REQUEST ACKNOWLEDGE message. The target eNB shall include in the E-RABs Admitted List IE the E-RABs for which resources have been prepared at the target cell. The E-RABs that have not been admitted in the target cell, if any, shall be included in the E-RABs Failed to Setup List IE.

If the HANDOVER REQUEST message contains the Data Forwarding Not Possible IE associated with a given E-RAB within the E-RABs To Be Setup List IE set to “Data forwarding not possible”, then the target eNB may decide not to include the DL Transport Layer Address IE and the DL GTP-TEID IE and for intra LTE handover the UL Transport Layer Address IE and the UL GTP-TEID IE within the E-RABs Admitted List IE of the HANDOVER REQUEST ACKNOWLEDGE message for that E-RAB.

For each bearer that target eNB has decided to admit and for which DL forwarding IE is set to “DL forwarding proposed”, the target eNB may include the DL GTP-TEID IE and the DL Transport Layer Address IE within the E-RABs Admitted List IE of the HANDOVER REQUEST ACKNOWLEDGE message indicating that it accepts the proposed forwarding of downlink data for this bearer.

If the HANDOVER REQUEST ACKNOWLEDGE message contains the UL GTP-TEID IE and the UL Transport Layer Address IE for a given bearer in the E-RABs Admitted List IE, then it means the target eNB has requested the forwarding of uplink data for this given bearer.

If the Request Type IE is included in the HANDOVER REQUEST message, then the target eNB should perform the requested location reporting functionality for the UE as described in subclause 8.11.

If the UE Security Capabilities IE included in the HANDOVER REQUEST message only contains the EIA0 algorithm as defined in TS 33.401 [15] and if this EIA0 algorithm is defined in the configured list of allowed integrity protection algorithms in the eNB (TS 33.401 [15]), the eNB shall take it into use and ignore the keys received in the Security Context IE.

The GUMMEI IE shall only be contained in the HANDOVER REQUEST message according to subclauses 4.6.2 and 4.7.6.6 of TS 36.300 [14]. If the GUMMEI IE is present, the target eNB shall store this information in the UE context and use it for subsequent X2 handovers.

The MME UE S1AP ID 2 IE shall only be contained in the HANDOVER REQUEST message according to subclause 4.6.2 of TS 36.300 [14].If the MME UE S1AP ID 2 IE is present, the target eNB shall store this information in the UE context and use it for subsequent X2 handovers.

If the Management Based MDT Allowed IE only or the Management Based MDT Allowed IE and the Management Based MDT PLMN List IE is contained in the HANDOVER REQUEST message, the target eNB shall, if supported, store the received information in the UE context, and use this information to allow subsequent selections of the UE for management based MDT defined in TS 32.422 [10].

If the Masked IMEISV IE is contained in the HANDOVER REQUEST message the target eNB shall, if supported, use it to determine the characteristics of the UE for subsequent handling.

If the HANDOVER REQUEST contains a Target Cell ID IE, as part of the Source eNB to Target eNB Transparent Container IE, for a cell which is no longer active, the eNB may respond with an HANDOVER REQUEST ACKNOWLEDGE in case the PCI of the deactivated cell is in use by another active cell.

If the ProSe Authorized IE is contained in the HANDOVER REQUEST message and it contains one or more IEs set to “authorized”, the eNB shall, if supported, consider that the UE is authorized for the relevant ProSe service(s).

If the UE User Plane CIoT Support Indicator IE is included in the HANDOVER REQUEST message and is set to "supported", the eNB shall, if supported, consider that User Plane CIoT EPS Optimisation as specified in TS 23.401[11] is supported for the UE.

If the CE-mode-B Support Indicator IE is included in the HANDOVER REQUEST ACKNOWLEDGE message and set to "supported", the MME shall, if supported, take this information into account when setting NAS timer values for the UE as specified in TS 24.301[24].

If the V2X Services Authorized IE is contained in the HANDOVER REQUEST message and it contains one or more IEs set to “authorized”, the eNB shall, if supported, consider that the UE is authorized for the relevant service(s).

If the UE Sidelink Aggregate Maximum Bit Rate IE is included in the HANDOVER REQUEST message, the eNB shall, if supported, use the received value for the concerned UE’s sidelink communication in network scheduled mode for V2X services.

If the Enhanced Coverage Restricted IE is included in the HANDOVER REQUEST message, the eNB shall store this information in the UE context and use it as defined in TS 23.401 [11].

If the CE-Mode-B Restricted IE is included in the HANDOVER REQUEST message and the Enhanced Coverage Restricted IE is not set to restricted and the Enhanced Coverage Restricted information stored in the UE context is not set to restricted, the eNB shall store this information in the UE context and use it as defined in TS 23.401 [11].

If the NR UE Security Capabilities IE is included in the HANDOVER REQUEST message, the eNB shall, if supported, store this information in the UE context and use it as defined in TS 33.401 [15].

If the Aerial UE subscription information IE is included in the HANDOVER REQUEST message, the eNB shall, if supported, store this information in the UE context and use it as defined in TS 36.300 [14].

#### 8.4.2.3	Unsuccessful Operation

Figure 8.4.2.3-1: Handover resource allocation: unsuccessful operation

If the target eNB does not admit at least one non-GBR E-RAB, or a failure occurs during the Handover Preparation, it shall send the HANDOVER FAILURE message to the MME with an appropriate cause value.

If the target eNB does not receive the CSG Membership Status IE but does receive the CSG Id IE in the HANDOVER REQUEST message and the CSG Id does not correspond to the CSG Id of the target cell, the target eNB shall send the HANDOVER FAILURE message to the MME with an appropriate cause value.

If the target eNB receives a HANDOVER REQUEST message containing RRC Container IE that does not include required information as specified in TS 36.331 [16], the target eNB shall send the HANDOVER FAILURE message to the MME.

#### 8.4.2.4	Abnormal Conditions

If the target eNB receives a HANDOVER REQUEST message containing a E-RAB Level QoS Parameters IE which contains a QCI IE indicating a GBR bearer (as defined in TS 23.203 [13]), and which does not contain the GBR QoS Information IE, the target eNB shall not admit the corresponding E-RAB.

If the target eNB receives a HANDOVER REQUEST message containing several E-RAB ID IEs (in the E-RABs To Be Setup List IE) set to the same value, the target eNB shall not admit the corresponding E-RABs.

If the Subscriber Profile ID for RAT/Frequency priority IE is not contained in the Source eNB to Target eNB Transparent Container IE whereas available in the source eNB, the target eNB shall trigger a local error handling.

NOTE:	It is assumed that the information needed to verify this condition is visible within the system, see subclause 4.1.

If the supported algorithms for encryption defined in the Encryption Algorithms IE in the UE Security Capabilities IE, plus the mandated support of EEA0 in all UEs (TS 33.401 [15]), do not match any allowed algorithms defined in the configured list of allowed encryption algorithms in the eNB (TS 33.401 [15]), the target eNB shall reject the procedure using the HANDOVER FAILURE message.

If the supported algorithms for integrity defined in the Integrity Protection Algorithms IE in the UE Security Capabilities IE, plus the mandated support of the EIA0 algorithm in all UEs (TS 33.401 [15]), do not match any allowed algorithms defined in the configured list of allowed integrity protection algorithms in the eNB (TS 33.401 [15]), the target eNB shall reject the procedure using the HANDOVER FAILURE message.

If the target eNB receives a HANDOVER REQUEST message which does not contain the Handover Restriction List IE, and the serving PLMN cannot be determined otherwise by the eNB, the target eNB shall reject the procedure using the HANDOVER FAILURE message.

If the target eNB receives a HANDOVER REQUEST message containing the Handover Restriction List IE, and the serving PLMN indicated is not supported by the target cell, the target eNB shall reject the procedure using the HANDOVER FAILURE message.

------------------------------------------

Skip to next change

------------------------------------------

### 8.4.4	Path Switch Request

#### 8.4.4.1	General

The purpose of the Path Switch Request procedure is to request the switch of a downlink GTP tunnel towards a new GTP tunnel endpoint.

#### 8.4.4.2	Successful Operation

Figure 8.4.4.2-1: Path switch request: successful operation

The eNB initiates the procedure by sending the PATH SWITCH REQUEST message to the MME.

If the ‎E-RAB To Be Switched in Downlink List IE in the PATH SWITCH REQUEST message does not include all E-RABs previously included in the UE Context, the MME shall consider the non included E-RABs as implicitly released by the eNB.

When the eNB has received from the radio interface the RRC Resume Cause IE, it shall include it in the PATH SWITCH REQUEST message.

After all necessary updates including the UP path switch have been successfully completed in the EPC for at least one of the E-RABs included in the PATH SWITCH REQUEST E-RAB To Be Switched in Downlink List IE, the MME shall send the PATH SWITCH REQUEST ACKNOWLEDGE message to the eNB and the procedure ends. The UE-associated logical S1-connection shall be established at reception of the PATH SWITCH REQUEST ACKNOWLEDGE message.

In case the EPC failed to perform the UP path switch for at least one, but not all, of the E-RABs included in the PATH SWITCH REQUEST E-RAB To Be Switched in Downlink List IE, the MME shall include the E-RABs it failed to perform UP path switch in the PATH SWITCH REQUEST ACKNOWLEDGE E-RAB To Be Released List IE. In this case, the eNB shall release the corresponding data radio bearers, and the eNB shall regard the E-RABs indicated in the E-RAB To Be Released List IE as being fully released.

If the CSG Id IE and no Cell Access Mode IE are received in the PATH SWITCH REQUEST message, the MME shall use it in the core network as specified in TS 23.401 [11]. If the CSG Id IE and the Cell Access Mode IE set to “hybrid” are received in the PATH SWITCH REQUEST message, the MME shall decide the membership status of the UE and use it in the core network as specified in TS 23.401 [11]. If no CSG Id IE and no Cell Access Mode IE are received in the PATH SWITCH REQUEST message and the UE was previously either in a CSG cell or in a hybrid cell, the MME shall consider that the UE has moved into a cell that is neither a CSG cell nor a hybrid cell and use this as specified in TS 23.401 [11].

If the GUMMEI of the MME currently serving the UE is available at the eNB (see TS 36.300 [14]) the eNB shall include the Source MME GUMMEI IE within the PATH SWITCH REQUEST message.

Upon reception of the PATH SWITCH REQUEST ACKNOWLEDGE message the eNB shall store the received Security Context IE in the UE context and the eNB shall use it for the next X2 handover or Intra eNB handovers as specified in TS 33.401 [15].

The PATH SWITCH REQUEST ACKNOWLEDGE message may contain

-	the UE Aggregate Maximum Bit Rate IE.

-	the MME UE S1AP ID 2 IE, which indicates the MME UE S1AP ID assigned by the MME.

If the UE Aggregate Maximum Bit Rate IE is included in the PATH SWITCH REQUEST ACKNOWLEDGE message the eNB shall

-	replace the previously provided UE Aggregate Maximum Bit Rate by the received UE Aggregate Maximum Bit Rate in the UE context; the eNB shall use the received UE Aggregate Maximum Bit Rate for non-GBR Bearers for the concerned UE.

If the UE Aggregate Maximum Bit Rate IE is not contained in the PATH SWITCH REQUEST ACKNOWLEDGE message, the eNB shall use the previously provided UE Aggregate Maximum Bit Rate which is stored in the UE context.

In case the EPC decides to change the uplink termination point of the tunnels, it may include the E-RAB To Be Switched in Uplink List IE in the PATH SWITCH REQUEST ACKNOWLEDGE message to specify a new uplink transport layer address and uplink GTP-TEID for each respective E-RAB for which it wants to change the uplink tunnel termination point.

When the eNB receives the PATH SWITCH REQUEST ACKNOWLEDGE message and if this message includes the E-RAB To Be Switched in Uplink List IE, the eNB shall start delivering the uplink packets of the concerned E-RABs to the new uplink tunnel endpoints as indicated in the message.

When the eNB receives the PATH SWITCH REQUEST ACKNOWLEDGE message including the CSG Membership Status IE, and if the cell that serves the UE is a hybrid cell, the eNB shall use it as defined in TS 36.300 [14].

If the MME UE S1AP ID 2 IE is contained in the PATH SWITCH REQUEST ACKNOWLEDGE message, the eNB shall store this information in the UE context and use it for subsequent X2 handovers.

If the Tunnel Information for BBF IE is received in the PATH SWITCH REQUEST message, the MME shall, if supported, use it in the core network as specified in TS 23.139 [37].

If the LHN ID IE is included in the PATH SWITCH REQUEST message, the MME shall, if supported, use it as specified in TS 23.401 [11].

If the ProSe Authorized IE is contained in the PATH SWITCH REQUEST ACKNOWLEDGE message, the eNB shall, if supported, update its ProSe authorization information for the UE accordingly. If the ProSe Authorized IE includes one or more IEs set to “not authorized”, the eNB shall, if supported, initiate actions to ensure that the UE is no longer accessing the relevant ProSe service(s).

If the UE User Plane CIoT Support Indicator IE is included in the PATH SWITCH REQUEST ACKNOWLEDGE message and is set to "supported", the eNB shall, if supported, consider that User Plane CIoT EPS Optimisation as specified in TS 23.401[11] is supported for the UE.

If the V2X Services Authorized IE is contained in the PATH SWITCH REQUEST ACKNOWLEDGE message, the eNB shall, if supported, update its V2X services authorization information for the UE accordingly. If the V2X Services Authorized IE includes one or more IEs set to “not authorized”, the eNB shall, if supported, initiate actions to ensure that the UE is no longer accessing the relevant service(s).

If the UE Sidelink Aggregate Maximum Bit Rate IE is included in the PATH SWITCH REQUEST ACKNOWLEDGE message, the eNB shall, if supported:

-	replace the previously provided UE Sidelink Aggregate Maximum Bit Rate, if available in the UE context, with the received value;

-	use the received value for the concerned UE’s sidelink communication in network scheduled mode for V2X services.

If the Enhanced Coverage Restricted IE is included in the PATH SWITCH REQUEST ACKNOWLEDGE message, the eNB shall store this information in the UE context and use it as defined in TS 23.401 [11].

If the CE-Mode-B Restricted IE is included in the PATH SWITCH REQUEST ACKNOWLEDGE message and the Enhanced Coverage Restricted IE is not set to restricted and the Enhanced Coverage Restricted information stored in the UE context is not set to restricted, the eNB shall store this information in the UE context and use it as defined in TS 23.401 [11].

If information on the UE’s NR security capabilities is available at the eNB (see TS 33.401 [15]) the eNB shall include the NR UE Security Capabilities IE within the PATH SWITCH REQUEST message.

If the NR UE Security Capabilities IE is included in the PATH SWITCH REQUEST message, the MME shall, if supported, consider that the eNB has stored the respective information in the UE context, and proceed as defined in TS 33.401 [15].

If the NR UE Security Capabilities IE is included in the PATH SWITCH REQUEST ACKNOWLEDGE message, the eNB shall, if supported, store this information in the UE context and use it as defined in TS 33.401 [15].

If the Aerial UE subscription information IE is included in the PATH SWITCH REQUEST ACKNOWLEDGE message, the eNB shall, if supported, store this information in the UE context and use it as defined in TS 36.300 [14].

#### 8.4.4.3	Unsuccessful Operation

Figure 8.4.4.3-1: Path switch request: unsuccessful operation

If the EPC fails to switch the downlink GTP tunnel endpoint towards a new GTP tunnel endpoint for all E-RABs included in the E-RAB To Be Switched in Downlink List IE during the execution of the Path Switch Request procedure, the MME shall send the PATH SWITCH REQUEST FAILURE message to the eNB with an appropriate cause value. In this case, the eNB should decide its subsequent actions and the MME should behave as described in TS 23.401 [11].

#### 8.4.4.4	Abnormal Conditions

If the MME receives a PATH SWITCH REQUEST message containing several E-RAB ID IEs (in the E-RAB To Be Switched in Downlink List IE) set to the same value, the MME shall send the PATH SWITCH REQUEST FAILURE message to the eNB.

If the MME receives a PATH SWITCH REQUEST message without the CSG Membership Status IE, and the cell accessed by the UE is a hybrid cell with a different CSG from the source cell or the source cell does not have a CSG ID, the MME shall send the PATH SWITCH REQUEST FAILURE message to the eNB.

If the CSG Membership Status IE is not included in the PATH SWITCH REQUEST ACKNOWLEDGE message and the cell accessed by the UE is a hybrid cell with a different CSG from the source cell or the source cell does not have a CSG ID, the eNB shall consider the procedure as unsuccessfully terminated and initiate local error handling.

------------------------------------------

Skip to next change

------------------------------------------

### 9.1.4	Context Management Messages

#### 9.1.4.1	INITIAL CONTEXT SETUP REQUEST

This message is sent by the MME to request the setup of a UE context.

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| UE Aggregate Maximum Bit Rate | M |  | 9.2.1.20 |  | YES | reject |
| E-RAB to Be Setup List |  | 1 |  |  | YES | reject |
| >E-RAB to Be Setup Item IEs |  | 1 .. <maxnoofE-RABs> |  |  | EACH | reject |
| >>E-RAB ID | M |  | 9.2.1.2 |  | - |  |
| >>E-RAB Level QoS Parameters | M |  | 9.2.1.15 | Includes necessary QoS parameters. | - |  |
| >>Transport Layer Address | M |  | 9.2.2.1 |  | - |  |
| >>GTP-TEID | M |  | 9.2.2.2 |  | - |  |
| >>NAS-PDU | O |  | 9.2.3.5 |  | - |  |
| >>Correlation ID | O |  | 9.2.1.80 |  | YES | ignore |
| >>SIPTO Correlation ID | O |  | Correlation ID 9.2.1.80 |  | YES | ignore |
| >>Bearer Type | O |  | 9.2.1.116 |  | YES | reject |
| UE Security Capabilities | M |  | 9.2.1.40 |  | YES | reject |
| Security Key | M |  | 9.2.1.41 | The KeNB is provided after the key-generation in the MME, see TS 33.401 [15]. | YES | reject |
| Trace Activation | O |  | 9.2.1.4 |  | YES | ignore |
| Handover Restriction List | O |  | 9.2.1.22 |  | YES | ignore |
| UE Radio Capability | O |  | 9.2.1.27 |  | YES | ignore |
| Subscriber Profile ID for RAT/Frequency priority | O |  | 9.2.1.39 |  | YES | ignore |
| CS Fallback Indicator | O |  | 9.2.3.21 |  | YES | reject |
| SRVCC Operation Possible | O |  | 9.2.1.58 |  | YES | ignore |
| CSG Membership Status | O |  | 9.2.1.73 |  | YES | ignore |
| Registered LAI | O |  | 9.2.3.1 |  | YES | ignore |
| GUMMEI | O |  | 9.2.3.9 | This IE indicates the MME serving the UE. | YES | ignore |
| MME UE S1AP ID 2 | O |  | 9.2.3.3 | This IE indicates the MME UE S1AP ID assigned by the MME. | YES | ignore |
| Management Based MDT Allowed | O |  | 9.2.1.83 |  | YES | ignore |
| Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.1.89 |  | YES | ignore |
| Additional CS Fallback Indicator | C-ifCSFBhighpriority |  | 9.2.3.37 |  | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.38 |  | YES | ignore |
| Expected UE Behaviour | O |  | 9.2.1.96 |  | YES | ignore |
| ProSe Authorized | O |  | 9.2.1.99 |  | YES | ignore |
| UE User Plane CIoT Support Indicator | O |  | 9.2.1.113 |  | YES | ignore |
| V2X Services Authorized | O |  | 9.2.1.120 |  | YES | ignore |
| UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.1.122 | This IE applies only if the UE is authorized for V2X services. | YES | ignore |
| Enhanced Coverage Restricted | O |  | 9.2.1.123 |  | YES | ignore |
| NR UE Security Capabilities | O |  | 9.2.1.127 |  | YES | ignore |
| CE-mode-B Restricted | O |  | 9.2.1.129 |  | YES | ignore |
| Aerial UE subscription information | O |  | 9.2.1.xxx |  | YES | ignore |

------------------------------------------

Skip to next change

------------------------------------------

#### 9.1.4.8	UE CONTEXT MODIFICATION REQUEST

This message is sent by the MME to provide UE Context information changes to the eNB.

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| Security Key | O |  | 9.2.1.41 | A fresh KeNB is provided after performing a key-change on the fly procedure in the MME, see TS 33.401 [15]. | YES | reject |
| Subscriber Profile ID for RAT/Frequency priority | O |  | 9.2.1.39 |  | YES | ignore |
| UE Aggregate Maximum Bit Rate | O |  | 9.2.1.20 |  | YES | ignore |
| CS Fallback Indicator | O |  | 9.2.3.21 |  | YES | reject |
| UE Security Capabilities | O |  | 9.2.1.40 |  | YES | reject |
| CSG Membership Status | O |  | 9.2.1.73 |  | YES | ignore |
| Registered LAI | O |  | 9.2.3.1 |  | YES | ignore |
| Additional CS Fallback Indicator | C-ifCSFBhighpriority |  | 9.2.3.37 |  | YES | ignore |
| ProSe Authorized | O |  | 9.2.1.99 |  | YES | ignore |
| SRVCC Operation Possible | O |  | 9.2.1.58 |  | YES | ignore |
| SRVCC Operation Not Possible | O |  | 9.2.1.119 |  | YES | ignore |
| V2X Services Authorized | O |  | 9.2.1.120 |  | YES | ignore |
| UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.1.122 | This IE applies only if the UE is authorized for V2X services. | YES | ignore |
| NR UE Security Capabilities | O |  | 9.2.1.127 |  | YES | ignore |
| Aerial UE subscription information | O |  | 9.2.1.xxx |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifCSFBhighpriority | This IE shall be present if the CS Fallback Indicator IE is set to “CS Fallback High Priority”. |

------------------------------------------

Skip to next change

------------------------------------------

#### 9.1.5.4	HANDOVER REQUEST

This message is sent by the MME to the target eNB to request the preparation of resources.

Direction: MME  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| Handover Type | M |  | 9.2.1.13 |  | YES | reject |
| Cause | M |  | 9.2.1.3 |  | YES | ignore |
| UE Aggregate Maximum Bit Rate | M |  | 9.2.1.20 |  | YES | reject |
| E-RABs To Be Setup List |  | 1 |  |  | YES | reject |
| >E-RABs To Be Setup Item IEs |  | 1 .. <maxnoofE-RABs> |  |  | EACH | reject |
| >>E-RAB ID | M |  | 9.2.1.2 |  | - |  |
| >>Transport Layer Address | M |  | 9.2.2.1 |  | - |  |
| >>GTP-TEID | M |  | 9.2.2.2 | To deliver UL PDUs. | - |  |
| >>E-RAB Level QoS Parameters | M |  | 9.2.1.15 | Includes necessary QoS parameters. | - |  |
| >>Data Forwarding Not Possible | O |  | 9.2.1.76 |  | YES | ignore |
| >>Bearer Type | O |  | 9.2.1.116 |  | YES | reject |
| Source to Target Transparent Container | M |  | 9.2.1.56 |  | YES | reject |
| UE Security Capabilities | M |  | 9.2.1.40 |  | YES | reject |
| Handover Restriction List | O |  | 9.2.1.22 |  | YES | ignore |
| Trace Activation | O |  | 9.2.1.4 |  | YES | ignore |
| Request Type | O |  | 9.2.1.34 |  | YES | ignore |
| SRVCC Operation Possible | O |  | 9.2.1.58 |  | YES | ignore |
| Security Context | M |  | 9.2.1.26 |  | YES | reject |
| NAS Security Parameters to E-UTRAN | C-iffromUTRANGERAN |  | 9.2.3.31 | The eNB shall use this IE as specified in TS 33.401 [15]. | YES | reject |
| CSG Id | O |  | 9.2.1.62 |  | YES | reject |
| CSG Membership Status | O |  | 9.2.1.73 |  | YES | ignore |
| GUMMEI | O |  | 9.2.3.9 | This IE indicates the MME serving the UE. | YES | ignore |
| MME UE S1AP ID 2 | O |  | 9.2.3.3 | This IE indicates the MME UE S1AP ID assigned by the MME. | YES | ignore |
| Management Based MDT Allowed | O |  | 9.2.1.83 |  | YES | ignore |
| Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.1.89 |  | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.38 |  | YES | ignore |
| Expected UE Behaviour | O |  | 9.2.1.96 |  | YES | ignore |
| ProSe Authorized | O |  | 9.2.1.99 |  | YES | ignore |
| UE User Plane CIoT Support Indicator | O |  | 9.2.1.113 |  | YES | ignore |
| V2X Services Authorized | O |  | 9.2.1.120 |  | YES | ignore |
| UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.1.122 | This IE applies only if the UE is authorized for V2X services. | YES | ignore |
| Enhanced Coverage Restricted | O |  | 9.2.1.123 |  | YES | ignore |
| NR UE Security Capabilities | O |  | 9.2.1.127 |  | YES | ignore |
| CE-mode-B Restricted | O |  | 9.2.1.129 |  | YES | ignore |
| Aerial UE subscription information | O |  | 9.2.1.xxx |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| C-iffromUTRANGERAN | This IE shall be present if the Handover Type IE is set to the value “UTRANtoLTE” or “GERANtoLTE”. |

| Range bound | Explanation |
| --- | --- |
| maxnoofE-RABs | Maximum no. of E-RABs for one UE. Value is 256. |

------------------------------------------

Skip to next change

------------------------------------------

#### 9.1.5.9	PATH SWITCH REQUEST ACKNOWLEDGE

This message is sent by the MME to inform the eNB that the path switch has been successfully completed in the EPC.

Direction: MME  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | ignore |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | ignore |
| UE Aggregate Maximum Bit Rate | O |  | 9.2.1.20 |  | YES | ignore |
| E-RAB To Be Switched in Uplink List |  | 0..1 |  |  | YES | ignore |
| >E-RABs Switched in Uplink Item IEs |  | 1 .. <maxnoofE-RABs> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.1.2 |  | - |  |
| >>Transport Layer Address | M |  | 9.2.2.1 |  | - |  |
| >>GTP-TEID | M |  | 9.2.2.2 |  | - |  |
| E-RAB To Be Released List | O |  | E-RAB List 9.2.1.36 | A value for E-RAB ID shall only be present once in E-RAB To Be Switched in Uplink List IE and E-RAB to Be Released List IE. | YES | ignore |
| Security Context | M |  | 9.2.1.26 | One pair of {NCC, NH} is provided. | YES | reject |
| Criticality Diagnostics | O |  | 9.2.1.21 |  | YES | ignore |
| MME UE S1AP ID 2 | O |  | 9.2.3.3 | This IE indicates the MME UE S1AP ID assigned by the MME. | YES | ignore |
| CSG Membership Status | O |  | 9.2.1.73 |  | YES | ignore |
| ProSe Authorized | O |  | 9.2.1.99 |  | YES | ignore |
| UE User Plane CIoT Support Indicator | O |  | 9.2.1.113 |  | YES | ignore |
| V2X Services Authorized | O |  | 9.2.1.120 |  | YES | ignore |
| UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.1.122 | This IE applies only if the UE is authorized for V2X services. | YES | ignore |
| Enhanced Coverage Restricted | O |  | 9.2.1.123 |  | YES | ignore |
| NR UE Security Capabilities | O |  | 9.2.1.127 |  | YES | ignore |
| CE-mode-B Restricted | O |  | 9.2.1.129 |  | YES | ignore |
| Aerial UE subscription information | O |  | 9.2.1.xxx |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofE-RABs | Maximum no. of E-RABs for one UE. Value is 256. |

------------------------------------------

Skip to next change

------------------------------------------

#### 9.2.1.xxx	Aerial UE subscription information

This information element is used by the eNB to know if the UE is allowed to use aerial UE function, refer to TS 23.401[11].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Aerial UE subscription information | M |  | ENUMERATED (allowed, not allowed,…) |  |

------------------------------------------

Skip to next change

------------------------------------------

### 9.3.3	PDU Definitions

-- **************************************************************

--

-- PDU definitions for S1AP.

--

-- **************************************************************

S1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) s1ap (1) version1 (1) s1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

UEAggregateMaximumBitrate,

BearerType,

Cause,

CellAccessMode,

Cdma2000HORequiredIndication,

Cdma2000HOStatus,

Cdma2000OneXSRVCCInfo,

Cdma2000OneXRAND,

Cdma2000PDU,

Cdma2000RATType,

Cdma2000SectorID,

EUTRANRoundTripDelayEstimationInfo,

CNDomain,

ConcurrentWarningMessageIndicator,

CriticalityDiagnostics,

CSFallbackIndicator,

CSG-Id,

CSG-IdList,

CSGMembershipStatus,

Data-Forwarding-Not-Possible,

Direct-Forwarding-Path-Availability,

Global-ENB-ID,

EUTRAN-CGI,

ENBname,

ENB-StatusTransfer-TransparentContainer,

ENB-UE-S1AP-ID,

ExtendedRepetitionPeriod,

GTP-TEID,

GUMMEI,

GUMMEIType,

HandoverRestrictionList,

HandoverType,

Masked-IMEISV,

LAI,

LPPa-PDU,

ManagementBasedMDTAllowed,

MDTPLMNList,

MMEname,

MMERelaySupportIndicator,

MME-UE-S1AP-ID,

MSClassmark2,

MSClassmark3,

NAS-PDU,

NASSecurityParametersfromE-UTRAN,

NASSecurityParameterstoE-UTRAN,

OverloadResponse,

PagingDRX,

PagingPriority,

PLMNidentity,

ProSeAuthorized,

RIMTransfer,

RelativeMMECapacity,

RequestType,

E-RAB-ID,

E-RABLevelQoSParameters,

E-RABList,

RelayNode-Indicator,

Routing-ID,

SecurityKey,

SecurityContext,

ServedGUMMEIs,

SONConfigurationTransfer,

Source-ToTarget-TransparentContainer,

SourceBSS-ToTargetBSS-TransparentContainer,

SourceeNB-ToTargeteNB-TransparentContainer,

SourceRNC-ToTargetRNC-TransparentContainer,

SubscriberProfileIDforRFP,

SRVCCOperationNotPossible,

SRVCCOperationPossible,

SRVCCHOIndication,

SupportedTAs,

TAI,

Target-ToSource-TransparentContainer,

TargetBSS-ToSourceBSS-TransparentContainer,

TargeteNB-ToSourceeNB-TransparentContainer,

TargetID,

TargetRNC-ToSourceRNC-TransparentContainer,

TimeToWait,

TraceActivation,

TrafficLoadReductionIndication,

E-UTRAN-Trace-ID,

TransportLayerAddress,

UEIdentityIndexValue,

UEPagingID,

UERadioCapability,

UERadioCapabilityForPaging,

UE-RetentionInformation,

UE-S1AP-IDs,

UE-associatedLogicalS1-ConnectionItem,

UESecurityCapabilities,

S-TMSI,

MessageIdentifier,

SerialNumber,

WarningAreaList,

RepetitionPeriod,

NumberofBroadcastRequest,

WarningType,

WarningSecurityInfo,

DataCodingScheme,

WarningMessageContents,

BroadcastCompletedAreaList,

RRC-Establishment-Cause,

BroadcastCancelledAreaList,

PS-ServiceNotAvailable,

GUMMEIList,

Correlation-ID,

GWContextReleaseIndication,

PrivacyIndicator,

VoiceSupportMatchIndicator,

TunnelInformation,

KillAllWarningMessages,

TransportInformation,

LHN-ID,

UserLocationInformation,

AdditionalCSFallbackIndicator,

ECGIListForRestart,

TAIListForRestart,

EmergencyAreaIDListForRestart,

ExpectedUEBehaviour,

Paging-eDRXInformation,

Extended-UEIdentityIndexValue,

MME-Group-ID,

Additional-GUTI,

PWSfailedECGIList,

CellIdentifierAndCELevelForCECapableUEs,

AssistanceDataForPaging,

InformationOnRecommendedCellsAndENBsForPaging,

UE-Usage-Type,

UEUserPlaneCIoTSupportIndicator,

NB-IoT-DefaultPagingDRX,

NB-IoT-Paging-eDRXInformation,

CE-mode-B-SupportIndicator,

NB-IoT-UEIdentityIndexValue,

V2XServicesAuthorized,

DCN-ID,

ServedDCNs,

UESidelinkAggregateMaximumBitrate,

DLNASPDUDeliveryAckRequest,

Coverage-Level,

EnhancedCoverageRestricted,

DL-CP-SecurityInformation,

UL-CP-SecurityInformation,

SecondaryRATusageDataRequst,

SecondaryRATusageReportList,

HandoverFlag,

NRUESecurityCapabilities,

UE-Application-Layer-Measurement-Capability,

CE-ModeBRestricted,

AerialUEsubscriptionInformation

FROM S1AP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-ContainerPair{},

ProtocolIE-ContainerPairList{},

ProtocolIE-SingleContainer{},

S1AP-PRIVATE-IES,

S1AP-PROTOCOL-EXTENSION,

S1AP-PROTOCOL-IES,

S1AP-PROTOCOL-IES-PAIR

FROM S1AP-Containers

id-AssistanceDataForPaging,

id-AerialUEsubscriptionInformation,

id-uEaggregateMaximumBitrate,

id-BearerType,

id-Cause,

id-CellAccessMode,

id-CellIdentifierAndCELevelForCECapableUEs,

id-cdma2000HORequiredIndication,

id-cdma2000HOStatus,

id-cdma2000OneXSRVCCInfo,

id-cdma2000OneXRAND,

id-cdma2000PDU,

id-cdma2000RATType,

id-cdma2000SectorID,

id-EUTRANRoundTripDelayEstimationInfo,

id-CNDomain,

id-ConcurrentWarningMessageIndicator,

id-CriticalityDiagnostics,

------------------------------------------

Skip to next change

------------------------------------------

-- **************************************************************

--

#### -- HANDOVER RESOURCE ALLOCATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Handover Request

--

-- **************************************************************

HandoverRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container		{ {HandoverRequestIEs} },

...

}

HandoverRequestIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID							CRITICALITY reject	TYPE MME-UE-S1AP-ID						PRESENCE mandatory}|

{ ID id-HandoverType							CRITICALITY reject	TYPE HandoverType						PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore	TYPE Cause								PRESENCE mandatory}|

{ ID id-uEaggregateMaximumBitrate				CRITICALITY reject	TYPE UEAggregateMaximumBitrate			PRESENCE mandatory}|

{ ID id-E-RABToBeSetupListHOReq					CRITICALITY reject	TYPE E-RABToBeSetupListHOReq			PRESENCE mandatory}|

{ ID id-Source-ToTarget-TransparentContainer	CRITICALITY reject	TYPE Source-ToTarget-TransparentContainer	PRESENCE mandatory}|

{ ID id-UESecurityCapabilities					CRITICALITY reject	TYPE UESecurityCapabilities			PRESENCE mandatory}|

{ ID id-HandoverRestrictionList					CRITICALITY ignore	TYPE HandoverRestrictionList			PRESENCE optional}|

{ ID id-TraceActivation							CRITICALITY ignore	TYPE TraceActivation					PRESENCE optional}|

{ ID id-RequestType								CRITICALITY ignore	TYPE RequestType						PRESENCE optional}|

{ ID id-SRVCCOperationPossible					CRITICALITY ignore	TYPE SRVCCOperationPossible			PRESENCE optional}|

{ ID id-SecurityContext							CRITICALITY reject	TYPE SecurityContext					PRESENCE mandatory}|

{ ID id-NASSecurityParameterstoE-UTRAN			CRITICALITY reject	TYPE NASSecurityParameterstoE-UTRAN		PRESENCE conditional

-- This IE shall be present if the Handover Type IE is set to the value "UTRANtoLTE" or "GERANtoLTE" --				}|

{ ID id-CSG-Id									CRITICALITY reject	TYPE CSG-Id								PRESENCE optional}|

{ ID id-CSGMembershipStatus						CRITICALITY ignore	TYPE CSGMembershipStatus				PRESENCE optional}|

{ ID id-GUMMEI-ID								CRITICALITY ignore	TYPE GUMMEI								PRESENCE optional}|

{ ID id-MME-UE-S1AP-ID-2						CRITICALITY ignore	TYPE MME-UE-S1AP-ID						PRESENCE optional}|

{ ID id-ManagementBasedMDTAllowed				CRITICALITY ignore	TYPE ManagementBasedMDTAllowed			PRESENCE optional}|

{ ID id-ManagementBasedMDTPLMNList				CRITICALITY ignore	TYPE MDTPLMNList						PRESENCE optional}|

{ ID id-Masked-IMEISV							CRITICALITY ignore	TYPE Masked-IMEISV						PRESENCE optional}|

{ ID id-ExpectedUEBehaviour						CRITICALITY ignore	TYPE ExpectedUEBehaviour				PRESENCE optional}|

{ ID id-ProSeAuthorized							CRITICALITY ignore	TYPE ProSeAuthorized					PRESENCE optional}|

{ ID id-UEUserPlaneCIoTSupportIndicator			CRITICALITY ignore	TYPE UEUserPlaneCIoTSupportIndicator 		PRESENCE optional}|

{ ID id-V2XServicesAuthorized					CRITICALITY ignore	TYPE V2XServicesAuthorized			PRESENCE optional}|

{ ID id-UESidelinkAggregateMaximumBitrate		CRITICALITY ignore	TYPE UESidelinkAggregateMaximumBitrate		PRESENCE optional}|

{ ID id-EnhancedCoverageRestricted				CRITICALITY ignore	TYPE EnhancedCoverageRestricted			PRESENCE optional}|

{ ID id-NRUESecurityCapabilities				CRITICALITY ignore	TYPE NRUESecurityCapabilities			PRESENCE optional}|

{ ID id-CE-ModeBRestricted						CRITICALITY ignore	TYPE CE-ModeBRestricted					PRESENCE optional}|

{ ID id-AerialUEsubscriptionInformation			CRITICALITY ignore	TYPE AerialUEsubscriptionInformation				PRESENCE optional},

...

}

E-RABToBeSetupListHOReq 					::= E-RAB-IE-ContainerList { {E-RABToBeSetupItemHOReqIEs} }

E-RABToBeSetupItemHOReqIEs S1AP-PROTOCOL-IES ::= {

{ ID id-E-RABToBeSetupItemHOReq				CRITICALITY reject	TYPE E-RABToBeSetupItemHOReq			PRESENCE mandatory	},

...

}

E-RABToBeSetupItemHOReq ::= SEQUENCE {

e-RAB-ID							E-RAB-ID,

transportLayerAddress				TransportLayerAddress,

gTP-TEID							GTP-TEID,

e-RABlevelQosParameters				E-RABLevelQoSParameters,

iE-Extensions						ProtocolExtensionContainer { {E-RABToBeSetupItemHOReq-ExtIEs} }		OPTIONAL,

...

}

E-RABToBeSetupItemHOReq-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

{ ID id-Data-Forwarding-Not-Possible		CRITICALITY ignore	EXTENSION Data-Forwarding-Not-Possible	PRESENCE optional}|

{ ID id-BearerType							CRITICALITY reject	EXTENSION BearerType					PRESENCE optional},

...

}

------------------------------------------

Skip to next change

------------------------------------------

-- **************************************************************

--

##### -- Path Switch Request Acknowledge

--

-- **************************************************************

PathSwitchRequestAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { PathSwitchRequestAcknowledgeIEs} },

...

}

PathSwitchRequestAcknowledgeIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID					CRITICALITY ignore	TYPE MME-UE-S1AP-ID						PRESENCE mandatory}|

{ ID id-eNB-UE-S1AP-ID					CRITICALITY ignore	TYPE ENB-UE-S1AP-ID						PRESENCE mandatory}|

{ ID id-uEaggregateMaximumBitrate		CRITICALITY ignore	TYPE UEAggregateMaximumBitrate			PRESENCE optional}|

{ ID id-E-RABToBeSwitchedULList			CRITICALITY ignore	TYPE E-RABToBeSwitchedULList			PRESENCE optional}|

{ ID id-E-RABToBeReleasedList			CRITICALITY ignore	TYPE E-RABList							PRESENCE optional}|

{ ID id-SecurityContext					CRITICALITY reject	TYPE SecurityContext					PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics				PRESENCE optional}|

{ ID id-MME-UE-S1AP-ID-2				CRITICALITY ignore	TYPE MME-UE-S1AP-ID						PRESENCE optional}|

{ ID id-CSGMembershipStatus				CRITICALITY ignore	TYPE CSGMembershipStatus				PRESENCE optional}|

{ ID id-ProSeAuthorized					CRITICALITY ignore	TYPE ProSeAuthorized					PRESENCE optional}|

{ ID id-UEUserPlaneCIoTSupportIndicator	CRITICALITY ignore	TYPE UEUserPlaneCIoTSupportIndicator	PRESENCE optional}|

{ ID id-V2XServicesAuthorized			CRITICALITY ignore	TYPE V2XServicesAuthorized				PRESENCE optional}|

{ ID id-UESidelinkAggregateMaximumBitrate		CRITICALITY ignore	TYPE UESidelinkAggregateMaximumBitrate	PRESENCE optional}|

{ ID id-EnhancedCoverageRestricted		CRITICALITY ignore	TYPE EnhancedCoverageRestricted			PRESENCE optional}|

{ ID id-NRUESecurityCapabilities				CRITICALITY ignore	TYPE NRUESecurityCapabilities			PRESENCE optional}|

{ ID id-CE-ModeBRestricted				CRITICALITY ignore	TYPE CE-ModeBRestricted			PRESENCE optional}|

{ ID id-AerialUEsubscriptionInformation			CRITICALITY ignore	TYPE AerialUEsubscriptionInformation				PRESENCE optional},

...

}

E-RABToBeSwitchedULList ::= E-RAB-IE-ContainerList { {E-RABToBeSwitchedULItemIEs} }

E-RABToBeSwitchedULItemIEs S1AP-PROTOCOL-IES ::= {

{ ID id-E-RABToBeSwitchedULItem		CRITICALITY ignore	TYPE E-RABToBeSwitchedULItem		PRESENCE mandatory	},

...

}

E-RABToBeSwitchedULItem ::= SEQUENCE {

e-RAB-ID							E-RAB-ID,

transportLayerAddress				TransportLayerAddress,

gTP-TEID							GTP-TEID,

iE-Extensions						ProtocolExtensionContainer { { E-RABToBeSwitchedULItem-ExtIEs} }	OPTIONAL,

...

}

E-RABToBeSwitchedULItem-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

------------------------------------------

Skip to next change

------------------------------------------

-- **************************************************************

--

#### -- INITIAL CONTEXT SETUP ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Initial Context Setup Request

--

-- **************************************************************

InitialContextSetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {InitialContextSetupRequestIEs} },

...

}

InitialContextSetupRequestIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID					CRITICALITY reject	TYPE MME-UE-S1AP-ID						PRESENCE mandatory}|

{ ID id-eNB-UE-S1AP-ID					CRITICALITY reject	TYPE ENB-UE-S1AP-ID						PRESENCE mandatory}|

{ ID id-uEaggregateMaximumBitrate		CRITICALITY reject	TYPE UEAggregateMaximumBitrate			PRESENCE mandatory}|

{ ID id-E-RABToBeSetupListCtxtSUReq		CRITICALITY reject	TYPE E-RABToBeSetupListCtxtSUReq		PRESENCE mandatory}|

{ ID id-UESecurityCapabilities			CRITICALITY reject	TYPE UESecurityCapabilities				PRESENCE mandatory}|

{ ID id-SecurityKey						CRITICALITY reject	TYPE SecurityKey						PRESENCE mandatory}|

{ ID id-TraceActivation					CRITICALITY ignore	TYPE TraceActivation					PRESENCE optional}|

{ ID id-HandoverRestrictionList			CRITICALITY ignore	TYPE HandoverRestrictionList			PRESENCE optional}|

{ ID id-UERadioCapability				CRITICALITY ignore	TYPE UERadioCapability					PRESENCE optional}|

{ ID id-SubscriberProfileIDforRFP		CRITICALITY ignore	TYPE SubscriberProfileIDforRFP			PRESENCE optional}|

{ ID id-CSFallbackIndicator				CRITICALITY reject	TYPE CSFallbackIndicator				PRESENCE optional}|

{ ID id-SRVCCOperationPossible			CRITICALITY ignore	TYPE SRVCCOperationPossible				PRESENCE optional}|

{ ID id-CSGMembershipStatus				CRITICALITY ignore	TYPE CSGMembershipStatus				PRESENCE optional}|

{ ID id-RegisteredLAI					CRITICALITY ignore	TYPE LAI								PRESENCE optional}|

{ ID id-GUMMEI-ID						CRITICALITY ignore	TYPE GUMMEI								PRESENCE optional}|

{ ID id-MME-UE-S1AP-ID-2				CRITICALITY ignore	TYPE MME-UE-S1AP-ID						PRESENCE optional}|

{ ID id-ManagementBasedMDTAllowed		CRITICALITY ignore	TYPE ManagementBasedMDTAllowed			PRESENCE optional}|

{ ID id-ManagementBasedMDTPLMNList		CRITICALITY ignore	TYPE MDTPLMNList						PRESENCE optional}|

{ ID id-AdditionalCSFallbackIndicator	CRITICALITY ignore	TYPE AdditionalCSFallbackIndicator	PRESENCE conditional}|

{ ID id-Masked-IMEISV					CRITICALITY ignore	TYPE Masked-IMEISV						PRESENCE optional}|

{ ID id-ExpectedUEBehaviour				CRITICALITY ignore	TYPE ExpectedUEBehaviour				PRESENCE optional}|

{ ID id-ProSeAuthorized					CRITICALITY ignore	TYPE ProSeAuthorized					PRESENCE optional}|

{ ID id-UEUserPlaneCIoTSupportIndicator	CRITICALITY ignore	TYPE UEUserPlaneCIoTSupportIndicator	PRESENCE optional}|

{ ID id-V2XServicesAuthorized			CRITICALITY ignore	TYPE V2XServicesAuthorized				PRESENCE optional}|

{ ID id-UESidelinkAggregateMaximumBitrate		CRITICALITY ignore	TYPE UESidelinkAggregateMaximumBitrate	PRESENCE optional}|

{ ID id-EnhancedCoverageRestricted		CRITICALITY ignore	TYPE EnhancedCoverageRestricted			PRESENCE optional}|

{ ID id-NRUESecurityCapabilities				CRITICALITY ignore	TYPE NRUESecurityCapabilities			PRESENCE optional}|

{ ID id-CE-ModeBRestricted				CRITICALITY ignore	TYPE CE-ModeBRestricted			PRESENCE optional}|

{ ID id-AerialUEsubscriptionInformation	CRITICALITY ignore	TYPE AerialUEsubscriptionInformation				PRESENCE optional},

...

}

E-RABToBeSetupListCtxtSUReq ::= SEQUENCE (SIZE(1.. maxnoofE-RABs)) OF ProtocolIE-SingleContainer { {E-RABToBeSetupItemCtxtSUReqIEs} }

E-RABToBeSetupItemCtxtSUReqIEs 	S1AP-PROTOCOL-IES ::= {

{ ID id-E-RABToBeSetupItemCtxtSUReq	CRITICALITY reject	TYPE E-RABToBeSetupItemCtxtSUReq		PRESENCE mandatory	},

...

}

E-RABToBeSetupItemCtxtSUReq ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

e-RABlevelQoSParameters			E-RABLevelQoSParameters,

transportLayerAddress			TransportLayerAddress,

gTP-TEID						GTP-TEID,

nAS-PDU							NAS-PDU		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABToBeSetupItemCtxtSUReqExtIEs} } OPTIONAL,

...

}

E-RABToBeSetupItemCtxtSUReqExtIEs S1AP-PROTOCOL-EXTENSION ::= {

{ ID id-Correlation-ID					CRITICALITY ignore	EXTENSION Correlation-ID			PRESENCE optional}|

{ ID id-SIPTO-Correlation-ID			CRITICALITY ignore	EXTENSION Correlation-ID			PRESENCE optional}|

{ ID id-BearerType						CRITICALITY reject	EXTENSION BearerType				PRESENCE optional},

...

}

------------------------------------------

Skip to next change

------------------------------------------

-- **************************************************************

--

#### -- UE CONTEXT MODIFICATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- UE Context Modification Request

--

-- **************************************************************

UEContextModificationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextModificationRequestIEs} },

...

}

UEContextModificationRequestIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID					CRITICALITY reject	TYPE MME-UE-S1AP-ID						PRESENCE mandatory}|

{ ID id-eNB-UE-S1AP-ID					CRITICALITY reject	TYPE ENB-UE-S1AP-ID						PRESENCE mandatory}|

{ ID id-SecurityKey						CRITICALITY reject	TYPE SecurityKey						PRESENCE optional}|

{ ID id-SubscriberProfileIDforRFP		CRITICALITY ignore	TYPE SubscriberProfileIDforRFP			PRESENCE optional}|

{ ID id-uEaggregateMaximumBitrate		CRITICALITY ignore	TYPE UEAggregateMaximumBitrate			PRESENCE optional}|

{ ID id-CSFallbackIndicator				CRITICALITY reject	TYPE CSFallbackIndicator				PRESENCE optional}|

{ ID id-UESecurityCapabilities			CRITICALITY reject	TYPE UESecurityCapabilities				PRESENCE optional}|

{ ID id-CSGMembershipStatus				CRITICALITY ignore	TYPE CSGMembershipStatus				PRESENCE optional}|

{ ID id-RegisteredLAI					CRITICALITY ignore	TYPE LAI								PRESENCE optional}|

{ ID id-AdditionalCSFallbackIndicator	CRITICALITY ignore	TYPE AdditionalCSFallbackIndicator	PRESENCE conditional}|

{ ID id-ProSeAuthorized					CRITICALITY ignore	TYPE ProSeAuthorized					PRESENCE optional}|

{ ID id-SRVCCOperationPossible			CRITICALITY ignore	TYPE SRVCCOperationPossible				PRESENCE optional}|

{ ID id-SRVCCOperationNotPossible		CRITICALITY ignore	TYPE SRVCCOperationNotPossible			PRESENCE optional}|

{ ID id-V2XServicesAuthorized			CRITICALITY ignore	TYPE V2XServicesAuthorized				PRESENCE optional}|

{ ID id-UESidelinkAggregateMaximumBitrate		CRITICALITY ignore	TYPE UESidelinkAggregateMaximumBitrate	PRESENCE optional}|

{ ID id-NRUESecurityCapabilities				CRITICALITY ignore	TYPE NRUESecurityCapabilities			PRESENCE optional}|

{ ID id-AerialUEsubscriptionInformation	CRITICALITY ignore	TYPE AerialUEsubscriptionInformation				PRESENCE optional},

...

}

------------------------------------------

Skip to next change

------------------------------------------

### 9.3.4	Information Element Definitions

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

S1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) s1ap (1) version1 (1) s1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-E-RABInformationListItem,

id-E-RABItem,

id-Bearers-SubjectToStatusTransfer-Item,

id-Time-Synchronisation-Info,

id-x2TNLConfigurationInfo,

id-eNBX2ExtendedTransportLayerAddresses,

id-MDTConfiguration,

id-Time-UE-StayedInCell-EnhancedGranularity,

id-HO-Cause,

id-M3Configuration,

id-M4Configuration,

id-M5Configuration,

id-MDT-Location-Info,

id-SignallingBasedMDTPLMNList,

id-MobilityInformation,

id-ULCOUNTValueExtended,

id-DLCOUNTValueExtended,

id-ReceiveStatusOfULPDCPSDUsExtended,

id-eNBIndirectX2TransportLayerAddresses,

id-Muting-Availability-Indication,

id-Muting-Pattern-Information,

------------------------------------------

Skip to next change

------------------------------------------

#### -- A

Additional-GUTI::= SEQUENCE {

gUMMEI					GUMMEI,

m-TMSI					M-TMSI,

iE-Extensions			ProtocolExtensionContainer { {Additional-GUTI-ExtIEs} } OPTIONAL,

...

}

Additional-GUTI-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

AerialUEsubscriptionInformation ::= ENUMERATED {

allowed,

not-allowed,

...

}

AreaScopeOfMDT ::= CHOICE {

cellBased					CellBasedMDT,

tABased						TABasedMDT,

pLMNWide					NULL,

...,

tAIBased					TAIBasedMDT

}

AreaScopeOfQMC ::= CHOICE {

cellBased					CellBasedQMC,

tABased						TABasedQMC,

tAIBased						TAIBasedQMC,

pLMNAreaBased				PLMNAreaBasedQMC,

...

}

AllocationAndRetentionPriority ::= SEQUENCE {

priorityLevel				PriorityLevel,

pre-emptionCapability		Pre-emptionCapability,

pre-emptionVulnerability	Pre-emptionVulnerability,

iE-Extensions				ProtocolExtensionContainer { {AllocationAndRetentionPriority-ExtIEs} } OPTIONAL,

...

}

AllocationAndRetentionPriority-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

AssistanceDataForCECapableUEs ::= SEQUENCE {

cellIdentifierAndCELevelForCECapableUEs		CellIdentifierAndCELevelForCECapableUEs,

iE-Extensions								ProtocolExtensionContainer { { InformationForCECapableUEs-ExtIEs} } OPTIONAL,

...

}

InformationForCECapableUEs-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

AssistanceDataForPaging ::= SEQUENCE {

assistanceDataForRecommendedCells	AssistanceDataForRecommendedCells		OPTIONAL,

assistanceDataForCECapableUEs		AssistanceDataForCECapableUEs			OPTIONAL,

pagingAttemptInformation			PagingAttemptInformation				OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { AssistanceDataForPaging-ExtIEs} }	OPTIONAL,

...

}

AssistanceDataForPaging-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

AssistanceDataForRecommendedCells ::= SEQUENCE {

recommendedCellsForPaging	RecommendedCellsForPaging,

iE-Extensions				ProtocolExtensionContainer { { AssistanceDataForRecommendedCells-ExtIEs} }	OPTIONAL,

...

}

AssistanceDataForRecommendedCells-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- B

------------------------------------------

Skip to next change

------------------------------------------

### 9.3.6	Constant Definitions

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

S1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) s1ap (1) version1 (1) s1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM S1AP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-HandoverPreparation						ProcedureCode ::= 0

id-HandoverResourceAllocation				ProcedureCode ::= 1

id-HandoverNotification						ProcedureCode ::= 2

id-PathSwitchRequest						ProcedureCode ::= 3

------------------------------------------

Skip to next change

------------------------------------------

id-extended-uEaggregateMaximumBitRateDL				ProtocolIE-ID ::= 259

id-extended-uEaggregateMaximumBitRateUL				ProtocolIE-ID ::= 260

id-NRrestriction									ProtocolIE-ID ::= 261

id-UEAppLayerMeasConfig								ProtocolIE-ID ::= 262

id-UE-Application-Layer-Measurement-Capability		ProtocolIE-ID ::= 263

id-SecondaryRATusageReportList						ProtocolIE-ID ::= 264

id-SecondaryRATusageReportItem						ProtocolIE-ID ::= 265

id-HandoverFlag										ProtocolIE-ID ::= 266

id-E-RABUsageReportItem								ProtocolIE-ID ::= 267

id-SecondaryRATusageDataRequst						ProtocolIE-ID ::= 268

id-NRUESecurityCapabilities							ProtocolIE-ID ::= 269

id-UnlicensedSpectrumRestriction					ProtocolIE-ID ::= 270

id-CE-ModeBRestricted								ProtocolIE-ID ::= 271

id-AerialUEsubscriptionInformation					ProtocolIE-ID ::= xxx

END