3GPP TSG-RAN WG3 Meeting #AH-1801	R3-180378

# Sophia Antipolis, France, 22nd – 26th  January 2018

Agenda Item:	10.1.1

Source:	Ericsson

Title:	QoS parameters to align with SA2

Document for:	pCR

# Text Proposal for NGAP TS 38.413 v0.5.0

<<<<<<<<<<<<<<<<<<<< Begin Text Proposal >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.23	QoS Flow Level QoS Parameters

Editor’s Note:	Further details FFS.

This IE defines the QoS to be applied to a QoS flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 5QI | O |  | INTEGER (0..255) | 5G QoS Indicator defined in TS 23.501 [9]. Logical range and coding specified in TS 23.501 [9]. eNote: Coding is FFS. |  |  |
| Allocation and Retention Priority | M |  | 9.3.1.30 |  |  |  |
| Non-standardised QoS Flow descriptor | O |  | 9.3.1.29 | eNote: Presence needs to be checked with latest stage 2 status. |  |  |
| GBR QoS Flow Information | O |  | 9.3.1.10 | This IE applies to GBR QoS flows only and shall be ignored otherwise. |  |  |
| RQA | O |  | ENUMERATED(subject to, …) | Reflective QoS Attribute: indicates that certain traffic on this QoS flow may be subject to NAS reflective QoS. This IE applies to non-GBR bearers only and shall be ignored otherwise. |  |  |

------------------------------------------

Skip to next change

------------------------------------------

#### 9.3.1.10	GBR QoS Flow Information

Editor’s Note:	Further details FFS.

This IE indicates the maximum and guaranteed bit rates of a GBR QoS flow for downlink and uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Maximum Flow Bit Rate Downlink | M |  | Bit Rate 9.3.1.4 | Maximum Bit Rate in DL. Details in TS 23.501 [9]. |  |  |
| Maximum Flow Bit Rate Uplink | M |  | Bit Rate 9.3.1.4 | Maximum Bit Rate in UL. Details in TS 23.501 [9]. |  |  |
| Guaranteed Flow Bit Rate Downlink | M |  | Bit Rate 9.3.1.4 | Guaranteed Bit Rate (provided there is data to deliver) in DL. Details in TS 23.501 [9]. |  |  |
| Guaranteed Flow Bit Rate Uplink | M |  | Bit Rate 9.3.1.4 | Guaranteed Bit Rate (provided there is data to deliver). Details in TS 23.501 [9]. |  |  |
| Notification Control | O |  | ENUMERATED (notification requested, ...) | For details refer to TS 23.501 [9]. |  |  |

------------------------------------------

Skip to next change

------------------------------------------

#### 9.3.1.29	Non-standardised QoS Flow Descriptor

This IE indicates the QoS parameters for a Non-standardised QoS flow for downlink and uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Non-standardised QoS Flow Descriptor |  |  |  |  |
| >Resource Type | M |  | ENUMERATED(GBR, Non-GBR) |  |
| >Priority Level | M |  | <ref> | For details see TS 23.501 [9]. |
| >Packet Delay Budget | M |  | <ref> | For details see TS 23.501 [9]. |
| >Packet Error Rate | M |  | <ref> | For details see TS 23.501 [9]. |
| >Averaging Window | O |  | <ref> | This IE applies to GBR bearers only. For details see TS 23.501 [9]. |

<<<<<<<<<<<<<<<<<<<< End Text Proposal  >>>>>>>>>>>>>>>>>>>>