3GPP TSG RAN WG3 NR AdHoc 1801	Tdoc R3-180420

Sophia Antipolis, France, 22 - 26 January 2018

Agenda Item:	10.10.1

Source:	Ericsson

Title:	TP on F1 TNL associations

Document for:	pCR TS 38.401

# Introduction

In this contribution, we provide the stage-2 text proposal for the solution on multiple SCTP associations over F1 that is described in R3-180419.

# Annex 1: TP for BL CR to TS 38.470

Start of Text Proposal 1 for TS 38.470

## 5.X		Multiple SCTP associations over F1-C

In the pre-operational phase, the gNB-DU is configured with the TNL address of the gNB-CU. The gNB-DU initiates the setup of the first SCTP association toward the gNB-CU. After the first SCTP association is established, the gNB-CU can send the F1 Setup Request message to the gNB-CU to start the establishment of the F1 interface. After the F1 interface is established, the first SCTP association can be used for all F1AP procedures as indicated in TS 38.472 [4].

The gNB-CU may employ the gNB-CU Configuration Update procedure to request the gNB-DU to establish additional SCTP associations. The additional SCTP associations can be used for F1AP UE-associated procedures, Initial UL RRC Message Transfer and Paging, as indicated in TS 38.472 [4]. The gNB-CU may also employ the gNB-CU Configuration Update procedure to request the gNB-DU to remove additional SCTP associations.

If the establishment of the UE-associated logical F1-conneciton is initiated by the gNB-DU, the gNB-DU can send the Initial UL RRC Message Transfer message on any SCTP association. To guide the gNB-DU decision, the gNB-CU may optionally provide weight factors. The gNB-CU may reply on a different SCTP association. The following signalling for this UE is on the SCTP association selected by the gNB-CU.

If the establishment of the UE-associated logical F1-conneciton is initiated by the gNB-CU, the gNB-CU sends the UE Context Setup Request message on any SCTP association. The following signalling for this UE is on the SCTP association selected by the gNB-CU.

End of Text Proposal for TS 38.470