3GPP TSG RAN WG3 NR AdHoc 1801	Tdoc R3-180504

Sophia Antipolis, France, 22 - 26 January 2018

Agenda Item:	10.10.8

Source:	Ericsson

Title:	Notification control over F1

Document for:	pCR TS 38.473

# Introduction

As specified in TS 23.501 [1], in the new 5G QoS framework a GBR QoS-flow may be associated with a parameter that is referred to as Notification Control. The RAN should report to the 5GC in case that the QoS for a flow that is associated with Notification Control can no longer (or again) be fulfilled. In this contribution, we analyse the impact of Notification Control on the F1 interface and we make proposals accordingly.

# Discussion

In TS 23.501 [1] the following description of the Notification Control feature is provided:

- If, for a given GBR QoS Flow, notification control is enabled and the NG-RAN determines that the GFBR cannot be fulfilled, RAN shall send a notification towards SMF. The RAN shall keep the QoS Flow, and should try to fulfil the GFBR. Upon receiving a notification from the RAN that the GFBR cannot be fulfilled, the 5GC may initiate N2 signalling to modify or remove the QoS Flow. When applicable, NG-RAN sends a new notification, informing SMF that the GFBR can be fulfilled again.

In the disaggregated architecture, only the gNB-DU can monitor the QoS over the air interface with accuracy. The reason is that the gNB-DU owns the lower-layers and therefore it has detailed knowledge of the radio conditions. The gNB-CU does not have real-time information about the quality of the radio channel and cannot estimate with precision whether the QoS of certain QoS-flows can be fulfilled. Therefore, the QoS monitoring should be performed by the gNB-DU. The gNB-DU schedules traffic over the air interface on a DRB level. Therefore, the gNB-DU cannot monitor the QoS over the air interface on a per-QoS-flow level, but it can monitor the QoS on a bearer level.

Observation 1:	Only the gNB-DU can monitor the QoS over the air interface with accuracy. The gNB-DU monitors the QoS over the air interface on a bearer level.

A consequence of the above considerations is that the gNB-CU should inform the gNB-DU on whether Notification Control is required for a given DRB.

The gNB-CU receives the QoS-flows to setup from the 5GC and for each QoS-flow it also receives indication on whether Notification Control should be supported. The gNB-CU decides how to map the QoS-flows to the DRBs. In case that the gNB-CU maps to a DRB a QoS-flow that is associated with Notification Control it should inform the gNB-DU about it. Therefore, a Notification Control IE should be added per-DRB in the F1AP UE Context Setup Request and in the F1AP UE Context Modification Request messages.

Proposal 1:	Introduce a Notification Control IE for each DRB to setup/modify in the F1AP UE Context Setup Request and in the F1AP UE Context Modification Request messages.

The gNB-DU should then monitor the QoS over the air for the bearers that are associated with Notification Control. The gNB-DU should report to the gNB-CU in case that it cannot anymore fulfil the QoS of a bearer that is subject to Notification Control. This can be realized either:

- by adding a new IE to the F1AP UE Context Modification Required message;

- or by defining a new Class 2, gNB-DU initiated, and UE-associated F1AP procedure.

Between the two options above, we believe that the latter is more efficient. The reason is that a Class 2 procedure does not require a reply message that in this case would be redundant. The same procedure can also be utilized to inform the gNB-CU in case that after some time the gNB-DU realizes that QoS for the bearer associated with Notification Control can be fulfilled again.

We suggest naming the new F1AP procedure “Notify” with the objective of aligning with the “PDU Session Resource Notify” procedure that is utilized in the NGAP for a similar purpose.

Proposal 2:	Introduce a F1AP “Notify” procedure to report that the QoS for a bearer associated with Notification Control can no longer (or again) be fulfilled. The F1AP “Notify” procedure should be: Class 2, gNB-DU initiated and UE-associated.

# Conclusion

In this contribution, we discussed the introduction of a new F1AP procedure to support Notification Control. The proposals are summarized in the following.

Observation 1:	Only the gNB-DU can monitor the QoS over the air interface with accuracy. The gNB-DU monitors the QoS over the air interface on a bearer level.

Proposal 1:	Introduce a Notification Control IE for each DRB to setup/modify in the F1AP UE Context Setup Request and in the F1AP UE Context Modification Request messages.

Proposal 2:	Introduce a F1AP “Notify” procedure to report that the QoS for a bearer associated with Notification Control can no longer (or again) be fulfilled. The F1AP “Notify” procedure should be: Class 2, gNB-DU initiated and UE-associated.

# References

TS 23.501, System Architecture for the 5G System.

# Annex 1: TP for the BL CR to TS 38.473

Start of Text Proposal 1 for TS 38.473

Table 2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Error Indication | ERROR INDICATION |
| UE Context Release Request (gNB-DU initiated) | UE CONTEXT RELEASE REQUEST |
| Initial UL RRC Message Transfer | INITIAL UL RRC MESSAGE TRANSFER |
| DL RRC Message Transfer | DL RRC MESSAGE TRANSFER |
| UL RRC Message Transfer | UL RRC MESSAGE TRANSFER |
| System Information Delivery | SYSTEM INFORMATION DELIVERY COMMAND |
| Paging | PAGING |
| Notify | NOTIFY |

Start of Text Proposal 2 for TS 38.473

### 8.3.1	UE Context Setup

#### 8.3.1.1	General

The purpose of the UE Context Setup procedure is to establish the UE Context including among others SRB, and DRB data. The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: UE Context Setup Request procedure: Successful Operation.

The gNB-CU initiates the procedure by sending UE CONTEXT SETUP REQUEST message to the gNB-DU. If the gNB-DU succeeds to establish the UE context, it replies to the gNB-CU with UE CONTEXT SETUP RESPONSE.

If the PSCell ID IE and the SCell To Be Setup List IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall act as specified in TS 38.401 [4].

If the DRX Cycle IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall take this into account.

If the SRB To Be Setup List IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall act as specified in TS 38.401 [4].

The gNB-DU shall report to the gNB-CU, in the UE CONTEXT SETUP RESPONSE message, the result for all the requested DRBs and SRBs in the following way:

-	A list of DRBs which are successfully established shall be included in the DRB Setup List IE;

-	A list of DRBs which failed to be established shall be included in the DRB Failed to Setup List IE;

-	A list of SRBs which are successfully established shall be included in the SRB Setup List IE;

-	A list of SRBs which failed to be established shall be included in the SRB Failed to Setup List IE.

When the gNB-DU reports the unsuccessful establishment of a DRB or SRB, the cause value should be precise enough to enable the gNB-CU to know the reason for the unsuccessful establishment.

For EN-DC operation, the gNB-CU shall include the E-UTRAN QoS IE. The allocation of resources according to the values of the Allocation and Retention Priority IE included in the E-UTRAN QoS IE shall follow the principles described for the E-RAB Setup procedure in TS 36.413 [3].

If the gNB-CU received the MeNB Resource Coordination Information as defined in TS 36.423 [9], it shall transparently transfer it to the gNB-DU via the Resource Coordination Transfer Container IE in the UE CONTEXT SETUP REQUEST message. The gNB-DU shall use the information received in the Resource Coordination Transfer Container IE as described in TS 36.423 [9] for reception of MeNB Resource Coordination Information at the gNB acting as secondary node.

If the Resource Coordination Transfer Container IE is included in the UE CONTEXT SETUP RESPONSE, the gNB-CU shall transparently transfer this information for the purpose of resource coordination as described in TS 36.423 [9].

If the Notification Control IE is included in the DRB to Be Setup List IE and it is set to active, the gNB-DU shall monitor the QoS of the DRB and notify the gNB-CU if the QoS cannot be fulfilled any longer or if the QoS can be fulfilled again. The Notification Control IE can only be applied to GBR bearers.

Start of Text Proposal 3 for TS 38.473

### 8.x	Notify

#### 8.x.1		General

The purpose of the Notify procedure is to enable the gNB-DU to inform the gNB-CU that the QoS of an already established GBR DRB cannot by fulfilled any longer or that it can be fulfilled again. The procedure uses UE-associated signalling.

#### 8.x.2		Successful Operation

Figure 8.x.2-1: Notify procedure. Successful operation.

The gNB-DU initiates the procedure by sending a NOTIFY message.

The NOTIFY message shall contain the list of the GBR DRBs associated with notification control for which the QoS is not fulfilled anymore or for which the QoS is fulfilled again by the gNB-DU.

Upon reception of the NOTIFY message, the gNB-CU may identify which are the affected PDU sessions and QoS flows. The gNB-CU may inform the 5GC or another NG-RAN node that the QoS for these PDU sessions or QoS flows is not fulfilled any longer or it is fulfilled again.

Editor’s Note:	Further details are FFS.

#### 8.x.3	Abnormal Conditions

Not applicable.

Start of Text Proposal 4 for TS 38.473

#### 9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| Target cell ID (FFS) | FFS |  | FFS |  | FFS | FFS |
| PSCell ID | O |  | NCGI 9.3.1.12 | PSCell Identifier in SgNB | YES | Ignore |
| CU to DU RRC Information | M |  | 9.3.1.25 |  | YES | reject |
| DRX Cycle | O |  | DRX Cycle 9.3.1.24 |  | YES | ignore |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the MeNB Resource Coordination Information IE as defined in subclause 9.1.X.1 of TS 36.423 [9]. | YES | reject |
| SCell To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Setup Item IEs |  | 0 .. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NCGI 9.3.1.12 | SCell Identifier in SgNB | - | - |
| SRB to Be Setup List |  | 0..1 |  |  |  |  |
| >SRB to Be Setup Item IEs |  | 1 .. <maxnoofSRBs>] |  |  |  |  |
| >SRB ID | M |  | 9.3.1.7 |  |  |  |
| DRB to Be Setup List |  | 1 |  |  | YES | reject |
| >DRB to Be Setup Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>Notification Control | O |  | 9.3.1.x |  | - | - |
| >>DRB QoS profile (FFS) |  |  |  |  | - |  |
| >>Flow-level QoS profile (FFS) |  |  |  |  |  |  |
| >>Flow-to-DRB mapping (FFS) |  |  |  |  |  |  |
| >>E-UTRAN QoS | O |  | 9.3.1.19 | Shall be used for EN-DC case to convey E-RAB Level QoS Parameters |  |  |
| >> Tunnels to be setup List |  | 1 |  |  |  |  |
| >>>Tunnels to Be Setup Item IEs |  | 1 .. <maxnoofULTunnels> |  |  |  |  |
| >>>>UL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - | - |

Start of Text Proposal 5 for TS 38.473

#### 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| PSCell ID | O |  | NCGI 9.3.1.12 | PSCell Identifier in SgNB | YES | Ignore |
| DRX Cycle | O |  | DRX Cycle 9.3.1.24 |  | YES | ignore |
| CU to DU RRC Information | O |  | 9.3.1.25 |  | YES | reject |
| Transmission Stop Indicator | O |  | 9.3.1.11 |  | YES | ignore |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the MeNB Resource Coordination Information IE as defined in subclause 9.1.X.1 of TS 36.423 [9]. | YES | reject |
| RRC-Container | O |  | 9.3.1.6 |  | YES | ignore |
| SCell To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Setup Item IEs |  | <0 .. maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NCGI 9.3.1.12 | SCell Identifier in SgNB | - | - |
| SRB to Be Setup List |  | 0..1 |  |  |  |  |
| >SRB to Be Setup Item IEs |  | <1..maxnoofSRBs> |  |  |  |  |
| >>SRB ID | M |  | 9.3.1.7 |  |  |  |
| DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Setup Item IEs |  | <1 .. maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>Notification Control | O |  | 9.3.1.x |  | - | - |
| >>E-UTRAN QoS | O |  | 9.3.1.19 | Shall be used for EN-DC case to convey E-RAB Level QoS Parameters |  |  |
| >> Tunnels to be setup List |  | 1 |  |  |  |  |
| >>Tunnels to Be Setup Item IEs |  | 1 .. <maxnoofULTunnels> |  |  |  |  |
| >>>UL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - | - |
| DRB to Be Modified List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Modified Item IEs |  | <1 .. maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>Notification Control | O |  | 9.3.1.x |  | - | - |
| >>E-UTRAN QoS | O |  | 9.3.1.19 | Shall be used for EN-DC case to convey E-RAB Level QoS Parameters |  |  |
| >> Tunnels to be setup List |  | 0..1 |  |  |  |  |
| >>>Tunnels to Be Setup Item IEs |  | 1 .. <maxnoofULTunnels> |  |  |  |  |
| >>>>UL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - | - |
| SRB TO Be Released List |  | 0..1 |  |  |  |  |
| >SRB To Be Released Item IEs |  | <1..maxnoofSRBs> |  |  |  |  |
| >>SRB ID | M |  | 9.3.1.7 |  |  |  |
| DRB to Be Released List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Released Item IEs |  | <1 .. maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |

Start of Text Proposal 6 for TS 38.473

#### 9.2.x		NOTIFY

This message is sent by the gNB-DU to notify the gNB-CU that the QoS for already established DRBs associated with notification control is not fulfilled any longer or it is fulfilled again.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| DRB Notify List |  | 0..1 |  |  | YES | reject |
| >DRB Notify Item IEs |  | <1 .. maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - | - |
| >>Notification Cause | M |  | ENUMERATED(Fulfilled, Not-Fulfilled, …) |  | - | - |

Editor’s Note:	Message structure and IEs need further checking and completion. Further details FFS.

Start of Text Proposal 7 for TS 38.473

#### 9.3.1.x	Notification Control

The Notification Control IE indicates whether the notification control for a given DRB is active or not-active. If the notification control is set to active, the gNB-DU shall monitor the QoS of the DRB and notify the gNB-CU.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Notification Control | M |  | ENUMERATED(Active, Not-Active, …) |  |

End of Text Proposal for TS 38.473